3GPP TS 32.172 V10.1.0 (2010-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Subscription Management (SuM)

Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)

(Release 10)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2010, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Keywords

UMTS, management

# Contents

Foreword	6

Introduction	6

1	Scope	7

2	References	7

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	8

4	Compliance rules	8

5	Modelling approach	8

6	Information Object Classes	9

6.1	Imported information entities and local labels	9

6.2	Class diagram	9

6.2.1	Attributes and relationships	9

6.2.2	Inheritance	13

6.3	Information Object Class definitions	17

6.3.1	SuMService	17

6.3.1.1	Definition	17

6.3.1.2	Attributes	17

6.3.2	CSService	17

6.3.2.1	Definition	17

6.3.2.2	Attributes	18

6.3.2.3	Attribute constraints	19

6.3.3	GPRSService	20

6.3.3.1	Definition	20

6.3.3.2	Attributes	20

6.3.3.3	Attribute constraints	21

6.3.4	IMSService	22

6.3.4.1	Definition	22

6.3.4.2	Attributes	22

6.3.4.3	Attribute constraints	22

6.3.5	SuMSubscribedService	23

6.3.5.1	Definition	23

6.3.5.2	Attributes	23

6.3.6	CSSubscribedService	23

6.3.6.1	Definition	23

6.3.6.2	Attributes	24

6.3.6.3	Attribute constraints	25

6.3.7	GPRSSubscribedService	25

6.3.7.1	Definition	25

6.3.7.2	Attributes	26

6.3.7.3	Attribute constraints	27

6.3.8	IMSSubscribedService	27

6.3.8.1	Definition	27

6.3.8.2	Attributes	27

6.3.8.3	Attribute constraints	27

6.3.9	SuMSubscriberProfile	28

6.3.9.1	Definition	28

6.3.9.2	Attributes	28

6.3.10	Void	28

6.3.11	SuMServiceProfile	28

6.3.11.1	Definition	28

6.3.11.2	Attributes	28

6.3.12	CSServiceProfile	28

6.3.12.1	Definition	28

6.3.12.2	Attributes	29

6.3.12.3	Attribute constraints	31

6.3.13	GPRSSServiceProfile	31

6.3.13.1	Definition	31

6.3.13.2	Attributes	31

6.3.13.3	Attribute constraints	32

6.3.14	IMSServiceProfile	33

6.3.14.1	Definition	33

6.3.14.2	Attributes	33

6.3.14.3	Attribute constraints	33

6.3.15	Void	34

6.3.16	SubscriptionFunction	34

6.3.16.1	Definition	34

6.3.16.2	Attributes	34

6.3.17	SuMCSSupplService	35

6.3.17.1	Definition	35

6.3.17.2	Attributes	35

6.3.17.3	Attribute constraints	35

6.3.18	SuMGPRSSupplService	36

6.3.18.1	Definition	36

6.3.18.2	Attributes	36

6.3.18.3	Attribute constraints	36

6.3.19	SuMUser	36

6.3.19.1	Definition	36

6.3.19.2	Attributes	36

6.3.20	SuMCredentials	37

6.3.20.1	Definition	37

6.3.20.2	Attributes	37

6.3.21	IMSPublicIdentification	37

6.3.21.1	Definition	37

6.3.21.2	Attributes	37

6.3.22	MobileUserData	38

6.3.22.1	Definition	38

6.3.22.2	Attributes	38

6.3.23	Void	38

6.3.24	SuMServiceParameter	38

6.3.24.1	Definition	38

6.3.24.2	Attributes	38

6.3.25	SuMIdentity	38

6.3.25.1	Definition	38

6.3.25.2	Attributes	39

6.3.26	ServerCapabilities	39

6.3.26.1	Definition	39

6.3.26.2	Attributes	39

6.3.27	EPSService	39

6.3.27.1	Definition	39

6.3.27.2	Attributes	40

6.3.27.3	Attribute constraints	41

6.3.28	EPSSubscribedService	41

6.3.28.1	Definition	41

6.3.28.2	Attributes	42

6.3.28.3	Attribute constraints	43

6.3.29	EPSServiceProfile	43

6.3.29.1	Definition	43

6.3.29.2	Attributes	43

6.3.29.3	Attribute constraints	43

6.3.30	PSCommonParameters	43

6.3.30.1	Definition	43

6.3.30.2	Attributes	44

6.3.30.3	Attribute constraints	44

6.3.31	IWANParameters	45

6.3.31.1	Definition	45

6.3.31.2	Attributes	45

6.3.31.3	Attribute constraints	45

6.3.32	OptHandover3GPP2Parameters	46

6.3.32.1	Definition	46

6.3.32.2	Attributes	46

6.3.32.3	Attribute constraints	46

6.3.33	Non3GPPAccessParameters	46

6.3.33.1	Definition	46

6.3.33.2	Attributes	46

6.3.33.3	Attribute constraints	46

6.4	Information relationship definitions	47

6.4.1	agreement	47

6.4.1.1	Definition	47

6.4.1.2	Roles	47

6.4.1.3	Constraints	47

6.4.2	description	47

6.4.2.1	Definition	47

6.4.2.2	Roles	47

6.4.2.3	Constraints	47

6.4.3	userSubscriberRelation	47

6.4.3.1	Definition	47

6.4.3.2	Roles	47

6.4.4	relatedMobileUserData	48

6.4.4.1	Definition	48

6.4.4.2	Roles	48

6.4.5	relatedServiceProfile	48

6.4.5.1	Definition	48

6.4.5.2	Roles	48

6.4.6	relatedIMSServiceProfile	48

6.4.6.1	Definition	48

6.4.6.2	Roles	48

6.4.6.3	Constraints	48

6.4.7	relatedIMSPublicId	49

6.4.7.1	Definition	49

6.4.7.2	Roles	49

6.4.7.3	Constraints	49

6.5	Information attribute definitions	50

6.5.1	Definitions and legal values	50

Annex A (informative):	Change history	56

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The interface Itf-N, defined in 3GPP TS 32.102 [2], is built up by a number of Integration Reference Points (IRPs) and a related Name Convention, which realise the functional capabilities over this interface. The basic structure of the IRPs is defined in 3GPP TS 32.101 [1] and 3GPP TS 32.102 [2].

The present document is part of a TS-family covering the 3rd Generation Partnership Project: Technical Specification Group Services and System Aspects; Telecommunication management, as identified below:

TS 32.140:	"Subscription Management (SuM) requirements".

TS 32.141:	"Subscription Management (SuM) architecture".

TS 32.171: 	"Subscription Management (SuM) Network Resource Model (NRM) Integration Reference Point (IRP): Requirements ".

TS 32.172: 	"Subscription Management (SuM) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

TS 32.176:	"Subscription Management (SuM) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Sets (SS)".

# 1	Scope

The present document defines the Network Resources Model (NRM) for the SuM IRP.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.140: "Telecommunication management; Subscription Management (SuM) Requirements".

[4]	3GPP TS 32.141: "Telecommunication management; Subscription Management (SuM) Architecture."

[5]	3GPP TS 32.171: "Telecommunication management; Subscription Management (SuM) Network Resource Model (NRM) Integration Reference Point (IRP): Requirements ".

[6]	3GPP TS 32.622: "Telecommunication management; Configuration Management (CM); Generic network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[7]	3GPP TS 23.008: "Organization of subscriber data".

[8]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx interfaces; Signalling flows and message contents".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply. For terms and definitions not found here, please refer to 3GPP TS 32.101 [1], 3GPP TS 32.102 [2], 3GPP TS 32.140 [3] and TS 32.141 [4].

Subscription Profile: see 3GPP TS 32.140 [3].

Subscription Profile Component: see 3GPP TS 32.140 [3].

Subscription Profile Component Definition: specifies a Subscription Profile Component.

Subscriber Profile: see 3GPP TS 32.140 [3].

User: see 3GPP TS 32.140 [3].

## 3.2	Abbreviations

For the purposes of the present document the following and the abbreviations in 32.101 [1], 32.102 [2], 32.140 [3] and 32.141 [4] apply.

EM	Element Manager

IOC	Information Object Class

IRP	Integration Reference Point

MOC	Managed Object Class

NE	Network Element

SuM	Subscription Management

# 4	Compliance rules

The following defines the meaning of Mandatory and Optional IOC attributes and associations between IOCs, in Solution Sets to the IRP defined by the present document.

Solution Sets to the SuM IRP:

- The IRPManager shall support all Mandatory attributes/associations. The IRPManager shall be prepared to receive information related to Mandatory as well as Optional attributes/associations without failure; however the IRPManager does not have to support handling of the Optional attributes/associations.

- The IRPAgent shall support all Mandatory attributes/associations. It may support Optional attributes/associations.

An IRPAgent that incorporates vendor-specific extensions shall support normal communication with a 3GPP SA5-compliant IRPManager with respect to all Mandatory and Optional Managed Object Classes (MOCs), attributes, associations, operations, parameters and notifications without requiring the IRPManager to have any knowledge of the extensions.

# 5	Modelling approach

The modelling approach adopted and used in this IRP is described in 3GPP TS 32.622 [6].

# 6	Information Object Classes

## 6.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| 32.622 [6], information object class, ManagedFunction | ManagedFunction |
| 32.622 [6], information object class, SubNetwork | SubNetwork |
| 32.622 [6], information object class, Top | Top |

## 6.2	Class diagram

### 6.2.1	Attributes and relationships

This clause depicts the set of IOCs that encapsulate information relevant for this service. This sub-clause provides the overview of all information object classes in UML. Subsequent clauses provide more detailed specification of various aspects of these Information Object Classes (IOCs).

Figure 1a: SuM NRM naming diagram 1

Figure 1b: SuM NRM Containment and Association diagram 1

Figure 1c: SuM NRM Containment and Association diagram 2

Figure 1d: SuM Supplementary Service NRM Aggregation diagram 1

Figure 1e: SuM Supplementary Service NRM Aggregation diagram 2

Figure 1f: SuM Supplementary Service NRM Aggregation diagram 3

Figure 1g: SuM IMS Service Containment and Association diagram 1

Figure 1.x: Service Profile NRM Association diagram

### 6.2.2	Inheritance

- Figure 2a: SuM NRM Inheritance Hierarchy 1

- Figure 2b: SuM NRM Inheritance Hierarchy 2

- Figure 2c: SuM NRM Inheritance Hierarchy 3

- Figure 2d: SuM NRM Inheritance Hierarchy 4

- Figure 2e: SuM NRM Inheritance Hierarchy 5

- Figure 2f: Void

Figure 2g: SuM NRM Inheritance Hierarchy 8

Figure 2h: SuM NRM Inheritance Hierarchy 9

### 6.3	Information Object Class definitions

### 6.3.1	SuMService

#### 6.3.1.1	Definition

This is an abstract IOC provided for sub-classing only. It provides attribute(s) that are common to Services IOCs.

#### 6.3.1.2	Attributes

- Attributes of SuMService

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| theContract | M | M | O |

### 6.3.2	CSService

#### 6.3.2.1	Definition

This class represents a CS service (see 3GPP TS 23.008 [7]) provided by a service provider

This class contains the valid parameters that define the CS service from subscription point of view.

#### 6.3.2.2	Attributes

- Attributes of CSService

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| networkAccessMode | M | M | M |
| basicMSISDNIndicator | O | M | M |
| mSISDNAlertIndicator | O | M | M |
| subscriptionRestriction | O | M | M |
| rSZIlists | O | M | M |
| lSAIdentity | O | M | M |
| lSAPriority | O | M | M |
| lSAPreferentialAccessIndicator | O | M | M |
| lSAActiveModeSupportIndicator | O | M | M |
| lSAOnlyAccessIndicator | O | M | M |
| lSAActiveModeIndicator | O | M | M |
| vPLMNIdentifier | O | M | M |
| accessRestrictionData | O | M | M |
| privacyExceptionList | O | M | M |
| gMLCNumbers | O | M | M |
| mOLRList | O | M | M |
| serviceTypes | O | M | M |
| mobileStationCategory | O | M | M |
| lMUIdentifier | O | M | M |
| subscriberStatus | O | M | M |
| gsmSCFaddresslistForODB | M | M | M |
| provisionOfBearerService | M | M | M |
| provisionOfTeleservice | M | M | M |
| bCAllocation | O | M | M |
| barringOfOutgoingCalls | O | M | M |
| barringOfIncomingCalls | O | M | M |
| barringOfRoaming | O | M | M |
| barringOfPremiumRateCalls | O | M | M |
| barringOfSupplementaryServiceManagement | O | M | M |
| barringOfRegistrationOfCallForwarding | O | M | M |
| barringOfInvocationOfCallTransfer | O | M | M |
| operatorDeterminedBarringPLMNspecificData | O | M | M |
| vGCSGroupMembershipList | O | M | M |
| vBSGroupMembershipList | O | M | M |
| broadcastCallInitiationAllowedList | O | M | M |
| originatingCAMELSubscriptionInformationOCSI | O | M | M |
| terminatingCAMELSubscriptionInformationTCSI | O | M | M |
| vMSCTerminatingCAMELSubscriptionInformationVTCSI | O | M | M |
| locationInformationSubscriberStateInformation | O | M | M |
| uSSDCAMELSubscriptionInformationUCSI | O | M | M |
| sSInvocationNotificationSSCSI | O | M | M |
| translationInformationFlagTIFCSI | O | M | M |
| dialledServiceCAMELSubscriptionInformationDCSI | O | M | M |
| uSSDGeneralCAMELServiceInformationUGCSI | O | M | M |
| shortMessageServiceCAMELSubscriptionInformationMOSMSCSI | O | M | M |
| shortMessageServiceCAMELSubscriptionInformationMTSMSCSI | O | M | M |
| mTSMSCSIVLRNegotiatedCAMELCapabilityHandling | O | M | M |
| gsmSCFaddressforCSI | O | M | M |
| iSTAlertTimer | O | M | M |
| cSAllocationRetentionpriority | O | M | M |
| theCSSupplServiceForCS | M | M | O |
| iCSIndicator | O | M | M |

#### 6.3.2.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

### 6.3.3	GPRSService

#### 6.3.3.1	Definition

This class represents a GPRS service (see 3GPP TS 23.008 [7]) provided by a service provider

This class contains the valid parameters that define the GPRS service from subscription point of view.

#### 6.3.3.2	Attributes

- Attributes of GPRSService

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| networkAccessMode | M | M | M |
| basicMSISDNIndicator | O | M | M |
| mSISDNAlertIndicator | O | M | M |
| rSZIlists | O | M | M |
| lSAIdentity | O | M | M |
| lSAPriority | O | M | M |
| lSAPreferentialAccessIndicator | O | M | M |
| lSAActiveModeSupportIndicator | O | M | M |
| lSAOnlyAccessIndicator | O | M | M |
| lSAActiveModeIndicator | O | M | M |
| vPLMNIdentifier | O | M | M |
| accessRestrictionData | O | M | M |
| privacyExceptionList | O | M | M |
| gMLCNumbers | O | M | M |
| mOLRList | O | M | M |
| serviceTypes | O | M | M |
| gGSNNumber | M | M | M |
| provisionofTeleservice | O | M | M |
| transferofSMoption | M | M | M |
| subscriberStatus | M | M | M |
| barringofPacketOrientedServices | O | M | M |
| pDPType | O | M | M |
| pDPAddress | O | M | M |
| accessPointName | O | M | M |
| vPLMNAddressAllowed | O | M | M |
| qualityofServiceSubscribed | O | M | M |
| pDPContextChargingCharacteristics | O | M | M |
| gPRSCAMELSubscriptionInformationGPRSCSI | O | M | M |
| mOShortMessageServiceCAMELSubscriptionInformationMOSMSCSI | O | M | M |
| mTShortMessageServiceCAMELSubscriptionInformationMTSMSCSI | O | M | M |
| mOSMSCSISGSNNegotiatedCAMELCapabilityHandling | O | M | M |
| mTSMSCSISGSNNegotiatedCAMELCapabilityHandling | O | M | M |
| mobilityManagementforGPRSeventnotificationMGCSI | O | M | M |
| mGCSINegotiatedCAMELCapabilityHandling | O | M | M |
| gsmSCFaddressforCSI | O | M | M |
| subscribedChargingCharacteristics | O | M | M |
| closedSubscriberGroupInformation | O | M | M |
| gSMSCFaddresslistForODB | O | M | M |
| barringOfOutgoingCalls | O | M | M |
| barringOfRoaming | O | M | M |
| operatorDeterminedBarringPLMNspecificData | O | M | M |
| sTNSR | O | M | M |
| iCSIndicator | O | M | M |
| listofauthorizedvisitednetworkidentifiers | O | M | M |
| wLANUERemoteIPaddress | O | M | M |
| staticWLANUERemoteIPAddressList | O | M | M |
| w-APNBarringTypeList | O | M | M |
| w-APNChargingDataList | O | M | M |
| accessDependenceFlag | O | M | M |
| chargingCharacteristics | O | M | M |
| primaryOCSChargingFunctionName | O | M | M |
| secondaryOCSChargingFunctionName | O | M | M |
| primaryChargingCollectionFunctionName | O | M | M |
| secondaryChargingCollectionFunctionName | O | M | M |
| thePSSupplServiceForPS | M | M | O |
| w-APNIdentifierList | O | M | M |
| maxNumberAccessesList | O | M | M |
| maxSubscribedBandwidth | O | M | M |
| subscribed3GPPWLANQoSProfile | O | M | M |

#### 6.3.3.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (See TS 23.008 [7]).

### 6.3.4	IMSService

#### 6.3.4.1	Definition

This class represents a IMS service (see 3GPP TS 23.008 [7]) provided by a service provider

This class contains the valid parameters that define the IMS service from subscription point of view.

#### 6.3.4.2	Attributes

- Attributes of IMSService

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| listofauthorizedvisitednetworkidentifiers | M | M | M |
| registrationStatus | M | M | M |
| sCSCFName | O | M | - |
| diameterClientAddressofSCSCF | O | M | - |
| theServerCapabilities | O | M | - |
| subscribedMediaProfileIdentifier | O | M | M |
| initialFilterCriteriaList | O | M | M |
| applicationServerInformation | O | M | M |
| serviceIndicationList | O | M | - |
| primaryEventChargingFunctionName | O | M | M |
| secondaryEventChargingFunctionName | O | M | M |
| primaryChargingCollectionFunctionName | M | M | M |
| secondaryChargingCollectionFunctionName | O | M | M |
| gsmSCFaddressforIMCSI | O | M | M |
| oIMCSI | O | M | M |
| vTIMCSI | O | M | M |
| dIMCS | O | M | M |
| gAAServiceType | O | M | M |
| gAAServiceIdentifier | O | M | M |
| gBAUserSecuritySettings | O | M | M |
| userSecuritySetting | O | M | M |
| userPublicIdentity | O | M | M |
| gAAAuthorizationflag | O | M | M |
| keyLifetime | O | M | M |
| uICCSecurityType | O | M | M |
| nAFGroupIdentity | O | M | M |

#### 6.3.4.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (See TS 23.008 [7]).

### 6.3.5	SuMSubscribedService

#### 6.3.5.1	Definition

This is an abstract IOC provided for sub-classing only. It provides attribute(s) that are common to Subscribed Services IOCs.

#### 6.3.5.2	Attributes

- Attributes of SuMSubscribedService

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| theService | M | M | O |
| theOffer | M | M | O |

### 6.3.6	CSSubscribedService

#### 6.3.6.1	Definition

This class represents subscription to a CS service (see 3GPP TS 23.008 [7]) for a subscriber.  The entire set of CS subscribed services owned by a subscriber constitutes the subscriber’s Subscribed Services Profile for CS services.

This class contains the agreed (between the Subscriber and the Service Provider) parameter settings for a CS service that a Subscriber subscribes to.

#### 6.3.6.2	Attributes

- Attributes of CSSubscribedService

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| networkAccessMode | M | M | M |
| basicMSISDNIndicator | O | M | M |
| mSISDNAlertIndicator | O | M | M |
| subscriptionRestriction | O | M | M |
| rSZIlists | O | M | M |
| lSAIdentity | O | M | M |
| lSAPriority | O | M | M |
| lSAPreferentialAccessIndicator | O | M | M |
| lSAActiveModeSupportIndicator | O | M | M |
| lSAOnlyAccessIndicator | O | M | M |
| lSAActiveModeIndicator | O | M | M |
| vPLMNIdentifier | O | M | M |
| accessRestrictionData | O | M | M |
| privacyExceptionList | O | M | M |
| gMLCNumbers | O | M | M |
| mOLRList | O | M | M |
| serviceTypes | O | M | M |
| mobileStationCategory | O | M | M |
| lMUIdentifier | O | M | M |
| subscriberStatus | O | M | M |
| gsmSCFaddresslistForODB | M | M | M |
| provisionOfBearerService | M | M | M |
| provisionOfTeleservice | M | M | M |
| bCAllocation | O | M | M |
| barringOfOutgoingCalls | O | M | M |
| barringOfIncomingCalls | O | M | M |
| barringOfRoaming | O | M | M |
| barringOfPremiumRateCalls | O | M | M |
| barringOfSupplementaryServiceManagement | O | M | M |
| barringOfRegistrationOfCallForwarding | O | M | M |
| barringOfInvocationOfCallTransfer | O | M | M |
| operatorDeterminedBarringPLMNspecificData | O | M | M |
| vGCSGroupMembershipList | O | M | M |
| vBSGroupMembershipList | O | M | M |
| broadcastCallInitiationAllowedList | O | M | M |
| originatingCAMELSubscriptionInformationOCSI | O | M | M |
| terminatingCAMELSubscriptionInformationTCSI | O | M | M |
| vMSCTerminatingCAMELSubscriptionInformationVTCSI | O | M | M |
| locationInformation/SubscriberStateInformation | O | M | M |
| uSSDCAMELSubscriptionInformationUCSI | O | M | M |
| sSInvocationNotificationSSCSI | O | M | M |
| translationInformationFlagTIFCSI | O | M | M |
| dialledServiceCAMELSubscriptionInformationDCSI | O | M | M |
| uSSDGeneralCAMELServiceInformationUGCSI | O | M | M |
| shortMessageServiceCAMELSubscriptionInformationMOSMSCSI | O | M | M |
| shortMessageServiceCAMELSubscriptionInformationMTSMSCSI | O | M | M |
| mTSMSCSIVLRNegotiatedCAMELCapabilityHandling | O | M | M |
| gsmSCFaddressforCSI | O | M | M |
| iSTAlertTimer | O | M | M |
| cSAllocationRetentionpriority | O | M | M |
| theCSSupplServiceForCSSS | M | M | O |
| iCSIndicator | O | M | M |

#### 6.3.6.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (See TS 23.008 [7]).

### 6.3.7	GPRSSubscribedService

#### 6.3.7.1	Definition

This class represents subscription to a GPRS service (see 3GPP TS 23.008 [7]) for a subscriber.  The entire set of GPRS subscribed services owned by a subscriber constitutes the subscriber’s Subscribed Services Profile for GPRS services.

#### 6.3.7.2	Attributes

- Attributes of GPRSSubscribedService

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| networkAccessMode | M | M | M |
| basicMSISDNIndicator | O | M | M |
| mSISDNAlertIndicator | O | M | M |
| rSZIlists | O | M | M |
| lSAIdentity | O | M | M |
| lSAPriority | O | M | M |
| lSAPreferentialAccessIndicator | O | M | M |
| lSAActiveModeSupportIndicator | O | M | M |
| lSAOnlyAccessIndicator | O | M | M |
| lSAActiveModeIndicator | O | M | M |
| vPLMNIdentifier | O | M | M |
| accessRestrictionData | O | M | M |
| privacyExceptionList | O | M | M |
| gMLCNumbers | O | M | M |
| mOLRList | O | M | M |
| serviceTypes | O | M | M |
| gGSNNumber | M | M | M |
| provisionofTeleservice | O | M | M |
| transferofSMoption | M | M | M |
| subscriberStatus | M | M | M |
| barringofPacketOrientedServices | O | M | M |
| pDPType | O | M | M |
| pDPAddress | O | M | M |
| accessPointName | O | M | M |
| vPLMNAddressAllowed | O | M | M |
| qualityofServiceSubscribed | O | M | M |
| pDPContextChargingCharacteristics | O | M | M |
| gPRSCAMELSubscriptionInformationGPRSCSI | O | M | M |
| mOShortMessageServiceCAMELSubscriptionInformationMOSMSCSI | O | M | M |
| mTShortMessageServiceCAMELSubscriptionInformationMTSMSCSI | O | M | M |
| mOSMSCSISGSNNegotiatedCAMELCapabilityHandling | O | M | M |
| mTSMSCSISGSNNegotiatedCAMELCapabilityHandling | O | M | M |
| mobilityManagementforGPRSeventnotificationMGCSI | O | M | M |
| mGCSINegotiatedCAMELCapabilityHandling | O | M | M |
| gsmSCFaddressforCSI | O | M | M |
| subscribedChargingCharacteristics | O | M | M |
| closedSubscriberGroupInformation | O | M | M |
| gSMSCFaddresslistForODB | O | M | M |
| barringOfOutgoingCalls | O | M | M |
| barringOfRoaming | O | M | M |
| operatorDeterminedBarringPLMNspecificData | O | M | M |
| sTNSR | O | M | M |
| iCSIndicator | O | M | M |
| listofauthorizedvisitednetworkidentifiers | O | M | M |
| wLANUERemoteIPaddress | O | M | M |
| staticWLANUERemoteIPAddressList | O | M | M |
| w-APNBarringTypeList | O | M | M |
| w-APNChargingDataList | O | M | M |
| accessDependenceFlag | O | M | M |
|  |  | - | - |
| chargingCharacteristics | O | M | M |
| primaryOCSChargingFunctionName | O | M | M |
| secondaryOCSChargingFunctionName | O | M | M |
| primaryChargingCollectionFunctionName | O | M | M |
| secondaryChargingCollectionFunctionName | O | M | M |
| w-APNIdentifierList | O | M | M |
| maxNumberAccessesList | O | M | M |
| maxSubscribedBandwidth | O | M | M |
| subscribed3GPPWLANQoSProfile | O | M | M |

#### 6.3.7.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

### 6.3.8	IMSSubscribedService

#### 6.3.8.1	Definition

This class represents subscription to an IMS (see 3GPP TS 23.008 [7]) service for a subscriber.  The entire set of IMS subscribed services owned by a subscriber constitutes the subscriber’s Subscribed Services profile for IMS services.

This class contains the agreed (between the Subscriber and the Service Provider) parameter settings for an IMS service that a Subscriber subscribes to.

#### 6.3.8.2	Attributes

- Attributes of IMSSubscribedService

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| listofauthorizedvisitednetworkidentifiers | M | M | M |
| registrationStatus | M | M | M |
| sCSCFName | O | M | - |
| diameterClientAddressofSCSCF | O | M | - |
| theServerCapabilities | O | M | - |
| subscribedMediaProfileIdentifier | O | M | M |
| initialFilterCriteriaList | O | M | M |
| applicationServerInformation | O | M | M |
| serviceIndicationList | O | M | - |
| primaryEventChargingFunctionName | O | M | M |
| secondaryEventChargingFunctionName | O | M | M |
| primaryChargingCollectionFunctionName | M | M | M |
| secondaryChargingCollectionFunctionName | O | M | M |
| gsmSCFaddressforIMCSI | O | M | M |
| oIMCSI | O | M | M |
| vTIMCSI | O | M | M |
| dIMCS | O | M | M |
| gAAServiceType | O | M | M |
| gAAServiceIdentifier | O | M | M |
| gBAUserSecuritySettings | O | M | M |
| userSecuritySetting | O | M | M |
| userPublicIdentity | O | M | M |
| gAAAuthorizationflag | O | M | M |
| keyLifetime | O | M | M |
| uICCSecurityType | O | M | M |
| nAFGroupIdentity | O | M | M |

#### 6.3.8.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

### 6.3.9	SuMSubscriberProfile

#### 6.3.9.1	Definition

This class represents the Subscriber Profile.

This class also contains the reference to its aggregated Subscribed Service as well as the users for a subscriber.

#### 6.3.9.2	Attributes

- Attributes of SuMSubscriberProfile

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| suMSubscriberProfileInfo | M | M | O |
| theUser | M | M | O |
| theSubscribedService | M | M | O |
| theMobileUserData | M | M | O |

### 6.3.10	Void

### 6.3.11	SuMServiceProfile

#### 6.3.11.1	Definition

This is an abstract IOC provided for sub-classing only. It provides attribute(s) that are common to Service Profiles IOCs.

#### 6.3.11.2	Attributes

- Attributes of SuMServiceProfile

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| theSpecification | M | M | O |

### 6.3.12	CSServiceProfile

#### 6.3.12.1	Definition

This class represents for a subscribed CS service (see 3GPP TS 23.008 [7]) the service preferences and settings chosen for a user.

#### 6.3.12.2	Attributes

- Attributes of CSServiceProfile

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| networkAccessMode | M | M | M |
| basicMSISDNIndicator | O | M | M |
| mSISDNAlertIndicator | O | M | M |
| vLRNumber | M | M | - |
| mSCNumber | M | M | - |
| subscriptionRestriction | O | M | M |
| rSZIlists | O | M | M |
| mSCAreaRestrictedFlag | M | M | - |
| oDBinducedBarringData | O | M | - |
| roamingRestrictionDueToUnsupportedFeature | M | M | - |
| lSAIdentity | O | M | M |
| lSAPriority | O | M | M |
| lSAPreferentialAccessIndicator | O | M | M |
| lSAActiveModeSupportIndicator | O | M | M |
| lSAOnlyAccessIndicator | O | M | M |
| lSAActiveModeIndicator | O | M | M |
| vPLMNIdentifier | O | M | M |
| accessRestrictionData | O | M | M |
| privacyExceptionList | O | M | M |
| gMLCNumbers | O | M | M |
| mOLRList | O | M | M |
| serviceTypes | O | M | M |
| ageIndicator | O | M | - |
| lMSI | M | M | - |
| mobileStationCategory | M | M | M |
| lMUIdentifier | O | M | M |
| mNRRMSC | M | M | - |
| subscriberStatus | O | M | M |
| notificationToCSEFlagForODB | O | M | - |
| gsmSCFaddresslistForODB | O | M | M |
| messagesWaitingData | O | M | - |
| mobileStationNotReachableFlag | O | M | - |
| memoryCapacityExceededFlag | M | M | - |
| traceReference | O | M | - |
| traceType | O | M | - |
| operationsSystemsIdentity | O | M | - |
| hLRTraceType | O | M | - |
| mAPErrorOnTrace | O | M | - |
| traceActivatedInVLR | O | M | - |
| provisionOfBearerService | M | M | M |
| provisionOfTeleservice | M | M | M |
| bCAllocation | O | M | M |
| checkSSIndicator | M | M | - |
| mSPurgedForNonGPRSFlag | M | M | - |
| barringOfOutgoingCalls | O | M | M |
| barringOfIncomingCalls | O | M | M |
| barringOfRoaming | O | M | M |
| barringOfPremiumRateCalls | O | M | M |
| barringOfSupplementaryServiceManagement | O | M | M |
| barringOfRegistrationOfCallForwarding | O | M | M |
| barringOfInvocationOfCallTransfer | O | M | M |
| operatorDeterminedBarringPLMNspecificData | O | M | M |
| vGCSGroupMembershipList | O | M | M |
| vBSGroupMembershipList | O | M | M |
| broadcastCallInitiationAllowedList | O | M | M |
| originatingCAMELSubscriptionInformationOCSI | O | M | M |
| terminatingCAMELSubscriptionInformationTCSI | O | M | M |
| vMSCTerminatingCAMELSubscriptionInformationVTCSI | O | M | M |
| locationInformationSubscriberStateInformation | O | M | M |
| uSSDCAMELSubscriptionInformationUCSI | O | M | M |
| sSInvocationNotificationSSCSI | O | M | M |
| translationInformationFlagTIFCSI | O | M | M |
| dialledServiceCAMELSubscriptionInformationDCSI | O | M | M |
| uSSDGeneralCAMELServiceInformationUGCSI | O | M | M |
| oCSINegotiatedCAMELCapabilityHandling | O | M | - |
| sSCSINegotiatedCAMELCapabilityHandling | O | M | - |
| vTCSINegotiatedCAMELCapabilityHandling | O | M | - |
| shortMessageServiceCAMELSubscriptionInformationMOSMSCSI | O | M | M |
| shortMessageServiceCAMELSubscriptionInformationMTSMSCSI | O | M | M |
| mOSMSCSIVLRNegotiatedCAMELCapabilityHandling | O | M | - |
| mTSMSCSIVLRNegotiatedCAMELCapabilityHandling | O | M | M |
| mCSINegotiatedCAMELCapabilityHandling | O | M | - |
| vLRSupportedCAMELPhases | O | M | - |
| gsmSCFaddressforCSI | O | M | M |
| vLROfferedCAMEL4CSIs | O | M | - |
| iSTAlertTimer | O | M | M |
| cSAllocationRetentionpriority | O | M | M |
| TraceReference2 | O | M | - |
| TraceDepth | O | M | - |
| ListofNeTypestoTrace | O | M | - |
| TriggeringEvents | O | M | - |
| ListofInterfacestoTrace | O | M | - |
| iCSIndicator | O | M | M |

#### 6.3.12.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

### 6.3.13	GPRSSServiceProfile

#### 6.3.13.1	Definition

This class represents for a subscribed GPRS service (see 3GPP TS 23.008 [7]) the service preferences and settings chosen for a user.

#### 6.3.13.2	Attributes

- Attributes of GPRSServiceProfile

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| basicMSISDNIndicator | O | M | M |
| mSISDNAlertIndicator | O | M | M |
| vLRNumber | M | M | - |
| lSAIdentity | O | M | M |
| lSAPriority | O | M | M |
| lSAPreferentialAccessIndicator | O | M | M |
| lSAActiveModeSupportIndicator | O | M | M |
| lSAOnlyAccessIndicator | O | M | M |
| lSAActiveModeIndicator | O | M | M |
| vPLMNIdentifier | O | M | M |
| ageIndicator | O | M | - |
| sGSNNumber | M | M | - |
| gGSNNumber | M | M | M |
| mNRG | M | M | - |
| provisionofTeleservice | O | M | M |
| transferofSMoption | M | M | M |
| mSpurgedforGPRSflag | M | M | - |
| pDPType | O | M | M |
| pDPAddress | O | M | M |
| gGSNlist | M | M | - |
| qualityofServiceSubscribed | O | M | M |
| pDPContextIdentifier | O | M | - |
| pDPContextChargingCharacteristics | O | M | M |
| gPRSCAMELSubscriptionInformationGPRSCSI | O | M | M |
| mOShortMessageServiceCAMELSubscriptionInformationMOSMSCSI | O | M | M |
| mTShortMessageServiceCAMELSubscriptionInformationMTSMSCSI | O | M | M |
| mOSMSCSISGSNNegotiatedCAMELCapabilityHandling | O | M | M |
| mTSMSCSISGSNNegotiatedCAMELCapabilityHandling | O | M | M |
| mobilityManagementforGPRSeventnotificationMGCSI | O | M | M |
| mGCSINegotiatedCAMELCapabilityHandling | O | M | M |
| gPRSCSINegotiatedCAMELCapabilityHandling | O | M | - |
| sGSNSupportedCAMELPhases | O | M | - |
| sGSNOfferedCAMEL4CSIs | O | M | - |
| gsmSCFaddressforCSI | O | M | M |
| gSMSCFaddresslistForODB | O | M | M |
| iPSMGWNumber | O | M | - |
| mNRR-SGSN | O | M | - |
| notificationToCSEFlagForODB | O | M | - |
| thePSCommonPar | M | M | M |

#### 6.3.13.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

### 6.3.14	IMSServiceProfile

#### 6.3.14.1	Definition

This class represents for a subscribed IMS (see 3GPP TS 23.008 [7]) service the service preferences and settings chosen for a user.

This class contains the reference to its aggregated IMS Public Identifications.

#### 6.3.14.2	Attributes

- Attributes of IMSServiceProfile

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| listofauthorizedvisitednetworkidentifiers | M | M | M |
| registrationStatus | M | M | M |
| sCSCFName | O | M | - |
| diameterClientAddressofSCSCF | O | M | - |
| theServerCapabilities | O | M | - |
| subscribedMediaProfileIdentifier | O | M | M |
| initialFilterCriteriaList | O | M | M |
| applicationServerInformation | O | M | M |
| serviceIndicationList | O | M | - |
| primaryEventChargingFunctionName | O | M | M |
| secondaryEventChargingFunctionName | O | M | M |
| primaryChargingCollectionFunctionName | M | M | M |
| secondaryChargingCollectionFunctionName | O | M | M |
| gsmSCFaddressforIMCSI | O | M | M |
| iMSSFaddressforIMCSI | O | M | - |
| oIMCSI | O | M | M |
| vTIMCSI | O | M | M |
| dIMCS | O | M | M |
| gAAServiceType | O | M | M |
| gAAServiceIdentifier | O | M | M |
| gBAUserSecuritySettings | O | M | M |
| userSecuritySetting | O | M | M |
| userPublicIdentity | O | M | M |
| gAAAuthorizationflag | O | M | M |
| keyLifetime | O | M | M |
| uICCSecurityType | O | M | M |
| nAFGroupIdentity | O | M | M |
| thePublicId | M | M | O |

#### 6.3.14.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

### 6.3.15	Void

### 6.3.16	SubscriptionFunction

#### 6.3.16.1	Definition

This class represents subscription functionality.

All managed elements related to the subscription of services have this functionality.

This class inherits from ManagedFunction (defined in 32.622 [6]).

This class contains the reference to its aggregated Users.

This class contains the reference to its aggregated Subscriber Profiles.

#### 6.3.16.2	Attributes

- Attributes of SubscriptionFunction

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| userLabel | M | M | M |
| theUser | M | M | O |
| theSubscriberProfile | M | M | O |

### 6.3.17	SuMCSSupplService

#### 6.3.17.1	Definition

This class represents a Supplementary Service for a CS service (see 3GPP TS 23.008 [7]) provided by a service provider

This class contains the valid parameters that define the CS supplementary service from subscription point of view.

#### 6.3.17.2	Attributes

- Attributes of SuMCSSupplService

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| sS-Code | M | M | M |
| provisioningState | M | M | M |
| activationState | M | M | M |
| registrationState | O | M | M |
| overrideCategory | O | M | M |
| cLIRestrictionOption | O | M | M |
| forwarded-toNumber | O | M | M |
| forwarded-toSubaddress | O | M | M |
| noReplyConditionTimer | O | M | M |
| maximumEntitledPriority | O | M | M |
| defaultPriority | O | M | M |
| nbrUser | O | M | M |
| nbrSB | O | M | M |

#### 6.3.17.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

### 6.3.18	SuMGPRSSupplService

#### 6.3.18.1	Definition

This class represents a Supplementary Service for a GPRS service (see 3GPP TS 23.008 [7]) provided by a service provider

This class contains the valid parameters that define the GPRS supplementary service from subscription point of view.

#### 6.3.18.2	Attributes

- Attributes of SuMGPRSSupplService

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| sS-Code | M | M | M |
| provisioningState | M | M | M |
| activationState | M | M | M |

#### 6.3.18.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (See TS 23.008 [7]).

### 6.3.19	SuMUser

#### 6.3.19.1	Definition

This class represents a User.

This class contains the valid parameters that define the User from subscription point of view.

#### 6.3.19.2	Attributes

- Attributes of SuMUser

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| theMobileUserData | M | M | O |
| theSubscriber | O | M | O |
| theCredentials | M | M | O |
| theServiceProfile | M | M | O |
| theIdentity | O | M | O |

### 6.3.20	SuMCredentials

#### 6.3.20.1	Definition

This class represents a User’s credentials.

This class contains the valid parameters that define the credentials from subscription point of view.

#### 6.3.20.2	Attributes

- Attributes of SuMCredentials

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| thePublicId | M | M | O |
| theServiceProfile | M | M | O |
| iMSPrivateUserIdentity | M | M | M |

### 6.3.21	IMSPublicIdentification

#### 6.3.21.1	Definition

This class represents the IMS Public Identification (see 3GPP TS 23.008 [7] and 3GPP TS 29.228 [8]).

This class contains the valid parameters that define the IMS Public Identification from subscription point of view.

#### 6.3.21.2	Attributes

Attributes of IMSPublicIdentification

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| publicUserIdentity | M | M | M |
| barringIndication | M | M | M |

### 6.3.22	MobileUserData

#### 6.3.22.1	Definition

This class represents user data of a mobile User Equipment

This class contains the valid parameters that define the user data of the mobile User Equipment from subscription point of view.

#### 6.3.22.2	Attributes

- Attributes of MobileUserData

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| theServiceProfile | M | M | O |
| iMSI | M | M | M |
| mSISDN | M | M | M |
| multinumberingMSISDNs | M | M | M |
| iMEISV | O | M | - |

### 6.3.23	Void

### 6.3.24	SuMServiceParameter

#### 6.3.24.1	Definition

This class represents a structured Service Parameter defined for a service and is an abstract class provided for sub-classing only.

This class shall be used as a base when having the requirement to provision common shared data referred to by specific SuMSubscribedService or SuMServiceProfile instances.

#### 6.3.24.2	Attributes

- Attributes of SuMServiceParameter

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |

### 6.3.25	SuMIdentity

#### 6.3.25.1	Definition

This class represents a user related identity, regardless of any type service to be provisioned and is an abstract class provided for sub-classing only.

This class allows an identity to be used for multiple services.

#### 6.3.25.2	Attributes

- Attributes of SuMIdentity

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |

### 6.3.26	ServerCapabilities

#### 6.3.26.1	Definition

This class is used to provision common shared ServerCapabilities data referred by specific SuMSubscribedService or SuMServiceProfile instances.

This class contains information to assist the I-CSCF in the selection of an S-CSCF for an IMS subscriber or a PSI user.

#### 6.3.26.2	Attributes

- Attributes of ServerCapabilities

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| serverCapabilities | M | M | M |

### 6.3.27	EPSService

#### 6.3.27.1	Definition

This class represents an EPS service (see 3GPP TS 23.008 [7]) provided by a service provider.  This class contains the valid parameters that define the EPS service from a subscription point of view.

#### 6.3.27.2	Attributes

Attributes of EPSService

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| accessPointName | O | M | M |
| access Restriction Data | O | M | M |
| aPNAMBR | M | M | M |
| barringofPacketOrientedServices | O | M | M |
| contextIdentifier | M | M | M |
| operatorDeterminedBarringPLMNspecificData | O | M | M |
| pDNAddress | O | M | M |
| rSZILists | O | M | M |
| sTNSR | O | M | M |
| subscribedChargingCharacteristics | O | M | M |
| subscriberStatus | O | M | M |
| networkAccessMode | M | M | M |
| barringOfOutgoingCalls | O | M | M |
| barringOfRoaming | O | M | M |
| privacyExceptionList | O | M | M |
| gMLCNumbers | O | M | M |
| mOLRList | O | M | M |
| serviceTypes | O | M | M |
| iCSIndicator | O | M | M |
| aPNOIReplacement | O | M | M |
| subscribedUEAMBR | M | M | M |
| aPNConfigurationProfile | M | M | M |
| rFSP | O | M | M |
| vPLMNAddressAllowed-EPS | M | M | M |
| pDNGWIdentity | O | M | M |
| ePSPDNConnectionChargingCharacteristics | O | M | M |
| pDNGWAllocationType | M | M | M |
| aPNLevelAPNOIReplacement | O | M | M |
| closedSubscriberGroupInformation | O | M | M |
| listofauthorizedvisitednetworkidentifiers | O | M | M |
| staticWLANUERemoteIPAddressList | O | M | M |
| w-APNBarringTypeList | O | M | M |
| w-APNChargingDataList | O | M | M |
| wLANUERemoteIPAddress | O | M | M |
| accessDependenceFlag | O | M | M |
| chargingCharacteristics | O | M | M |
| primaryOCSChargingFunctionName | O | M | M |
| secondaryOCSChargingFunctionName | O | M | M |
| primaryChargingCollectionFunctionName | O | M | M |
| secondaryChargingCollectionFunctionName | O | M | M |
| w-APNIdentifierList | O | M | M |
| maxNumberAccessesList | O | M | M |
| maxSubscribedBandwidth | O | M | M |
| subscribed3GPPWLANQoSProfile | O | M | M |
| permanentUserIdentity | O | M | M |
| mIPSubscriberProfile | O | M | M |
| accessRestrictionData | O | M | M |

#### 6.3.27.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (See TS 23.008 [7]).

### 6.3.28	EPSSubscribedService

#### 6.3.28.1	Definition

This class represents subscription to an EPS service (see 3GPP TS 23.008 [7]) for a subscriber.  The entire set of EPS subscribed services owned by a subscriber constitutes the subscriber’s Subscribed Services Profile for EPS services.

This class contains the agreed (between the Subscriber and the Service Provider) parameter settings for an EPS service that a Subscriber subscribes to.

#### 6.3.28.2	Attributes

Attributes of EPSSubscribedService

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| accessPointName | O | M | M |
| access Restriction Data | O | M | M |
| aPNAMBR | M | M | M |
| barringofPacketOrientedServices | O | M | M |
| contextIdentifier | M | M | M |
| operatorDeterminedBarringPLMNspecificData | O | M | M |
| pDNAddress | O | M | M |
| rSZILists | O | M | M |
| sTNSR | O | M | M |
| subscribedChargingCharacteristics | O | M | M |
| subscriberStatus | O | M | M |
| networkAccessMode | M | M | M |
| barringOfOutgoingCalls | O | M | M |
| barringOfRoaming | O | M | M |
| privacyExceptionList | O | M | M |
| gMLCNumbers | O | M | M |
| mOLRList | O | M | M |
| serviceTypes | O | M | M |
| iCSIndicator | O | M | M |
| aPNOIReplacement | O | M | M |
| subscribedUEAMBR | M | M | M |
| aPNConfigurationProfile | M | M | M |
| rFSP | O | M | M |
| vPLMNAddressAllowed-EPS | M | M | M |
| pDNGWIdentity | O | M | M |
| ePSPDNConnectionChargingCharacteristics | O | M | M |
| pDNGWAllocationType | M | M | M |
| aPNLevelAPNOIReplacement | O | M | M |
| closedSubscriberGroupInformation | O | M | M |
| Listofauthorizedvisitednetworkidentifiers | O | M | M |
| wLANUERemoteIPaddress | O | M | M |
| staticWLANUERemoteIPAddressList | O | M | M |
| w-APNBarringTypeList | O | M | M |
| w-APNChargingDataList | O | M | M |
| accessDependenceFlag | O | M | M |
| chargingCharacteristics | O | M | M |
| primaryOCSChargingFunctionName | O | M | M |
| secondaryOCSChargingFunctionName | O | M | M |
| primaryChargingCollectionFunctionName | O | M | M |
| secondaryChargingCollectionFunctionName | O | M | M |
| w-APNIdentifierList | O | M | M |
| maxNumberAccessesList | O | M | M |
| maxSubscribedBandwidth | O | M | M |
| subscribed3GPPWLANQoSProfile | O | M | M |
| permanentUserIdentity | O | M | M |
| mIPSubscriberProfile | O | M | M |
| accessRestrictionData | O | M | M |

#### 6.3.28.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (See TS 23.008 [7]).

### 6.3.29	EPSServiceProfile

#### 6.3.29.1	Definition

This class represents for a subscribed EPS service (see 3GPP TS 23.008 [7]) the service preferences and settings chosen for a user.

#### 6.3.29.2	Attributes

Attributes of EPSServiceProfile

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| aPNAMBR | M | M | M |
| contextIdentifier | M | M | M |
| pDNAddress | O | M | M |
| aPNOIReplacement | O | M | M |
| subscribedUEAMBR | M | M | M |
| aPNConfigurationProfile | M | M | M |
| rFSP | O | M | M |
| pDNGWIdentity | O | M | M |
| ePSPDNConnectionChargingCharacteristics | O | M | M |
| vPLMNAddressAllowed-EPS | M | M | M |
| pDNGWAllocationType | M | M | M |
| aPNLevelAPNOIReplacement | O | M | M |
| roamingRestrictionDueToUnsupportedFeatureMME | M | M | - |
| roamingRestrictionDueToUnsupportedFeatureSGSN | M | M | - |
| mSPurgedForEPSFlag | M | M | - |
| uRRPMME | O | M | - |
| mMEName | M | M | - |
| sGSNName | M | M | - |
| iWFNumber | O | M | - |
| sGSNAreaRestrictedFlag | M | M | - |
| theOptHandover3GPP2Par | M | M | O |
| theNon3GPPAccessPar | M | M | O |
| thePSCommonPar | M | M | M |

#### 6.3.29.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

### 6.3.30	PSCommonParameters

#### 6.3.30.1	Definition

This class represents for a subscribed GPRS or EPS service (see 3GPP TS 23.008 [7]) the service preferences and settings (attributes) chosen for a user that are common to both services.  Either a single instance of this IOC or two separate instances may be associated to a user who possesses both services.

#### 6.3.30.2	Attributes

- Attributes of PSCommonParameters

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| rSZIlists | O | M | M |
| roamingRestrictedintheSGSNduetounsupportedfeature | M | M | - |
| accessRestrictionData | O | M | M |
| subscriberStatus | O | M | M |
| barringOfOutgoingCalls | O | M | M |
| barringOfRoaming | O | M | M |
| operatorDeterminedBarringPLMNspecificData | O | M | M |
| barringofPacketOrientedServices | O | M | M |
| traceReference2 | O | M | - |
| traceDepth | O | M | - |
| listOfNETypestoTrace | O | M | - |
| triggeringEvents | O | M | - |
| listOfInterfacesToTrace | O | M | - |
| traceActivatedinSGSN | O | M | - |
| iPAddressOfTraceCollectionEntity | O | M | - |
| accessPointName | O | M | M |
| privacyExceptionList | O | M | M |
| gMLCNumbers | O | M | M |
| iCSIndicator | O | M | M |
| mOLRList | O | M | M |
| serviceTypes | O | M | M |
| subscribedChargingCharacteristics | O | M | M |
| sTNSR | O | M | M |
| networkAccessMode | M | M | M |
| closedSubscriberGroupInformation | O | M | M |
| theIWANPar | M | M | O |

#### 6.3.30.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

### 6.3.31	IWANParameters

#### 6.3.31.1	Definition

This class represents for a subscribed GPRS or EPS service (see 3GPP TS 23.008 [7]) the service preferences and settings chosen for a user that are relevant to IWAN services.

#### 6.3.31.2	Attributes

- Attributes of IWANParameters

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| w-APN | O | M | - |
| listofauthorizedvisitednetworkidentifiers | O | M | M |
| 3GPPAAAServerName | O | M | - |
| 3GPPAAAProxyName | O | M | - |
| wLANUERemoteIPaddress | O | M | M |
| staticWLANUERemoteIPAddressList | O | M | M |
| w-APNBarringTypeList | O | M | M |
| w-APNChargingDataList | O | M | M |
| accessDependenceFlag | O | M | M |
| i-WLANAccessType | O | M | - |
| wLANIdentifierList | O | M | M |
| maxNumberAccessesList | O | M | M |
| maxSubscribedBandwidth | O | M | M |
| subscribed3GPPWLANQoSProfile | O | M | M |
| chargingCharacteristics | M | M | M |
| primaryOCSChargingFunctionName | O | M | M |
| secondaryOCSChargingFunctionName | O | M | M |
| primaryChargingCollectionFunctionName | O | M | M |
| secondaryChargingCollectionFunctionName | O | M | M |
| userStatus | M | M | - |

#### 6.3.31.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

### 6.3.32	OptHandover3GPP2Parameters

#### 6.3.32.1	Definition

This class represents for a subscribed EPS service (see 3GPP TS 23.008 [7]) the service preferences and settings chosen for a user that are relevant to the optimization of handover to a 3GPP2 network.

#### 6.3.32.2	Attributes

- Attributes of OptHandover3GPP2Parameters

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| rATType | O | M | - |

#### 6.3.32.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

### 6.3.33	Non3GPPAccessParameters

#### 6.3.33.1	Definition

This class represents for a subscribed EPS service (see 3GPP TS 23.008 [7]) the service preferences and settings chosen for a user that are relevant to use of an non-3GPP access network.

#### 6.3.33.2	Attributes

- Attributes of NonGPPAccessParameters

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| rATType | O | M | - |
| visitedNetworkIdentifier | O | M | - |
| mIPSubscriberProfile | M | M | M |
| permanentUserIdentity | M | M | M |

#### 6.3.33.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

## 6.4	Information relationship definitions

### 6.4.1	agreement

#### 6.4.1.1	Definition

- This represents the agreement relationship between SuMService and SuMSubscribedService.

#### 6.4.1.2	Roles

| Name | Definition |
| --- | --- |
| theContract | The SuMSubscribedService represents an agreed contract for a SuMService. |
| theOffer | The SuMService represents an offer of an agreement to a SuMSubscribedService. |

#### 6.4.1.3	Constraints

The relationship is between a service and a subscription to the same type of service (for example between CSService and CSSubscribedService).

### 6.4.2	description

#### 6.4.2.1	Definition

- This represents the content description relationship between SuMSubscribedService and SuMServiceProfile.

#### 6.4.2.2	Roles

| Name | Definition |
| --- | --- |
| theSpecification | SuMSubscribedService acts as the specification for the description of SuMServiceProfile. |
| theService | SuMServiceProfile acts as the service specified in the description. |

#### 6.4.2.3	Constraints

The relationship is between a subscribed service and a service profile of the same type of service (for example between CSSubscribedService and CSServiceProfile).

### 6.4.3	userSubscriberRelation

#### 6.4.3.1	Definition

This represents the relationship between SuMUser and SuMSubscriberProfile. This relationship is optional.

#### 6.4.3.2	Roles

| Name | Definition |
| --- | --- |
| theUser | Represents the User to which the Subscriber links. |
| theSubscriber | Represents the Subscriber to which the User links. |

### 6.4.4	relatedMobileUserData

#### 6.4.4.1	Definition

- This represents the relationship between SuMUser and MobileUserData.

#### 6.4.4.2	Roles

| Name | Definition |
| --- | --- |
| theMobileUserData | Represents the user data of the mobile User Equipment being used. |

### 6.4.5	relatedServiceProfile

#### 6.4.5.1	Definition

- This represents the relationship between MobileUserData and SuMServiceProfile.

#### 6.4.5.2	Roles

| Name | Definition |
| --- | --- |
| theServiceProfile | Represents the Service Profile to which the user data of the mobile User Equipment links. |

### 6.4.6	relatedIMSServiceProfile

#### 6.4.6.1	Definition

- This represents the relationship between SuMCredentials and IMSServiceProfile.

#### 6.4.6.2	Roles

| Name | Definition |
| --- | --- |
| theServiceProfile | Represents the IMS Service Profile to which IMS Private User Identity links. |

#### 6.4.6.3	Constraints

- 1)	This relationship is Conditional Mandatory. The condition is that no instances of relatedIMSPublicId are present for the same object instance of SuMCredentials.

2)	All instances of this relationship from the same object instance of SuMCredentials shall be to object instances of IMSServiceProfile that are

i)	name-contained by the same object instance of SuMUser as name-contains the SuMCredentials object instance, and

ii)	associated (through the description relationship to SuMSubscribedService name-contained by SuMSubscriberProfile) to a single object instance of SuMSubscriberProfile.

### 6.4.7	relatedIMSPublicId

#### 6.4.7.1	Definition

- This represents the relationship between SuMCredentials and IMSPublicIdentification.

#### 6.4.7.2	Roles

| Name | Definition |
| --- | --- |
| thePublicId | Represents the IMS Public Identification to which IMS Private User Identity links. |

#### 6.4.7.3	Constraints

- 1)	This relationship is Conditional Mandatory. The condition is that no instances of relatedIMSServiceProfile are present for the same object instance of SuMCredentials.

2)	All instances of this relationship from the same object instance of SuMCredentials shall be to object instances of IMSPublicIdentification that are name-contained by object instances of IMSServiceProfile that are

i)	name-contained by the same object instance of SuMUser as name-contains the SuMCredentials object instance, and

ii)	associated (through the description relationship to SuMSubscribedService name-contained by SuMSubscriberProfile) to a single object instance of SuMSubscriberProfile.

## 6.5	Information attribute definitions

### 6.5.1	Definitions and legal values

The following table defines the attributes that are present in several IOCs of the present document.

- Attributes

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| id | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the SuM information object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| suMSubscriberProfileInfo | A description of (information for) the Subscriber Profile. It is with free structure and format. |  |
| userLabel | A user-friendly (and user assigned) name of the associated object. IOCs inheriting from ManagedFunction will have this attribute. Other IOCs may have it as well, if explicitly defined for them. |  |
| theContract | Models the agreement relationship to a subscribed service. This attribute contains a list of the DN(s) of the related SuMSubscribedService instance(s). |  |
| theOffer | Models the agreement relationship to a service. This attribute contains the DN of the related SuMService instance. |  |
| theService | Models the description relationship to a service profile. This attribute contains a list of the DN(s) of the related SuMServiceProfile instance(s). |  |
| theSpecification | Models the description relationship to a subscribed service. An attribute whose ‘name+value’ can be used as DN when referencing an instance of the SuMSubscribedService object class. |  |
| thePublicId | Models the relatedIMSPublicId relationship from a user’s credentials to the IMS Public Identification. This attribute contains a list of the DN(s) of the related IMSPublicIdentification instance(s). |  |
| theMobileUserData | Models the relatedMobileUserData relationship from a user to the user data of a mobile User Equipment. This attribute contains a list of the DN(s) of the related MobileUserData instance(s). |  |
| theServiceProfile | Models the relatedServiceProfile relationship from the user data of a mobile User Equipment to a service profile or from a user’s credentials to an IMS service profile. This attribute contains a list of the DN(s) of the related SuMServiceProfile derived instance(s). |  |
| theSubscriber | Models the userSubscriberRelation relationship from a user to a subscriber. This attribute contains a list of the DN(s) of the related SuMSubscriberProfile instance(s). |  |
| theUser | Models the userSubscriberRelation relationship from a subscriber to a user. This attribute contains a list of the DN(s) of the related SuMUser instance(s). |  |
| theCSSupplServiceForCS | Models the relationship from a CS service to a CS supplement service. This attribute contains a list of the DN(s) of the aggregated SuMCSSupplService instance(s). |  |
| thePSSupplServiceForPS | Models the relationship from a PS service to a PS supplement service. This attribute contains a list of the DN(s) of the aggregated SuMPSSupplService instance(s). |  |
| theCSSupplServiceForCSSS | Models the relationship from a CS subscribed service to a CS supplement service. This attribute contains a list of the DN(s) of the aggregated SuMCSSupplService instance(s). |  |
| thePSSupplServiceForPSSS | Models the relationship from a PS subscribed service to a PS supplement service. This attribute contains a list of the DN(s) of the aggregated SuMPSSupplService instance(s). |  |
| theCSSupplServiceForCSSP | Models the relationship from a CS service profile to a CS supplement service. This attribute contains a list of the DN(s) of the aggregated SuMCSSupplService instance(s). |  |
| thePSSupplServiceForPSSP | Models the relationship from a PS service profile to a PS supplement service. This attribute contains a list of the DN(s) of the aggregated SuMPSSupplService instance(s). |  |
| theSubscribedService | Models the relationship from a subscriber profile to a subscribed service. This attribute contains a list of the DN(s) of the aggregated SuMSubscribedService derived instance(s). |  |
| thePublicId | Models the relationship from an IMS service profile to a public identification. This attribute contains a list of the DN(s) of the aggregated IMSPublicIdentification instance(s). |  |
| theUser | Models the relationship from a Subscription Function to a user. This attribute contains a list of the DN(s) of the aggregated SuMUser instance(s). |  |
| theSubscriberProfile | Models the relationship from a Subscription Function to a subscriber profile. This attribute contains a list of the DN(s) of the aggregated SuMSubscriberProfile instance(s). |  |
| theCredentials | Models the relationship from a user to a credential. This attribute contains a list of the DN(s) of the aggregated SuMCredentials instance(s). |  |
| theServiceProfile | Models the relationship from a user to a service profile. This attribute contains a list of the DN(s) of the aggregated SuMServiceProfile instance(s). |  |
| theIdentity | Models the relationship from a user to an identity. This attribute contains a list of DN(s) of the related SuMIdentity instance(s). |  |
| theServerCapabilities | Models the relationship from a SuM IOC object instance (E.g., IMSService, IMSSubscribedService, IMSServiceProfile) to a Server Capabilities object instance. |  |
| thePSCommonParameters | Models the common packet access service profile relationship from either a GPRS service profile or an EPS service profile. This attribute contains one of DN of the related PSCommonParameters instance. |  |
| theIWANParameters | Models the relationship between the common packet access service parameters and the I-WAN service parameters. This attribute contains the DN of the related IWANParameters instance. |  |
| theOptHandover3GPP2Parameters | Models the relationship between the EPS service profile when designed for optimal handover to a 3GPP2 access network. This attribute contains the DN of the related OptHandover3GPP2Parameters instance. |  |
| theNon3GPPAccessParameters | Models the relationship between the EPS service profile and another set of service parameters needed to when access is via a non 3GPP network. This attribute contains the DN of the related Non3GPPAccessParameters instance. |  |

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| iMSI | 23.008 clause 2.1.1.1 |  |
| networkAccessMode | 23.008 clause 2.1.1.2 |  |
| mSISDN | 23.008 clause 2.1.2 |  |
| multinumberingMSISDNs | 23.008 clause 2.1.3 |  |
| basicMSISDNIndicator | 23.008 clause 2.1.3.1 |  |
| mSISDNAlertIndicator | 23.008 clause 2.1.3.2 |  |
| lMSI | 23.008 clause 2.1.8 |  |
| mobileStationCategory | 23.008 clause 2.2.1 |  |
| lMUIdentifier | 23.008 clause 2.2.2 |  |
| iMEISV | 23.008 clause 2.2.3 |  |
| subscriptionRestriction | 23.008 clause 2.4.10 |  |
| rSZIlists | 23.008 clause 2.4.11.1 |  |
| mSCAreaRestrictedFlag | 23.008 clause 2.4.12 |  |
| sGSNarearestrictedflag | 23.008 clause 2.4.14 |  |
| oDBinducedBarringData | 23.008 clause 2.4.15.1 |  |
| roamingRestrictionDueToUnsupportedFeature | 23.008 clause 2.4.15.2 |  |
| roamingRestrictedintheSGSNduetounsupportedfeature | 23.008 clause 2.4.15.3 |  |
| lSAIdentity | 23.008 clause 2.4.17.1 |  |
| lSAPriority | 23.008 clause 2.4.17.2 |  |
| lSAPreferentialAccessIndicator | 23.008 clause 2.4.17.2A |  |
| lSAActiveModeSupportIndicator | 23.008 clause 2.4.17.2B |  |
| lSAOnlyAccessIndicator | 23.008 clause 2.4.17.3 |  |
| lSAActiveModeIndicator | 23.008 clause 2.4.17.4 |  |
| vPLMNIdentifier | 23.008 clause 2.4.17.5 |  |
| accessRestrictionData | 23.008 clause 2.4.18 |  |
| vLRNumber | 23.008 clause 2.4.5 |  |
| mSCNumber | 23.008 clause 2.4.6 |  |
| sGSNNumber | 23.008 clause 2.4.8.1 |  |
| gGSNNumber | 23.008 clause 2.4.8.2 |  |
| provisionOfBearerService | 23.008 clause 2.5.1 |  |
| provisionOfTeleservice | 23.008 clause 2.5.2 |  |
| bCAllocation | 23.008 clause 2.5.3 |  |
| transferofSMoption | 23.008 clause 2.5.4 |  |
| mNRG | 23.008 clause 2.7.2 |  |
| checkSSIndicator | 23.008 clause 2.7.4.4 |  |
| mSPurgedForNonGPRSFlag | 23.008 clause 2.7.5 |  |
| mSpurgedforGPRSflag | 23.008 clause 2.7.6 |  |
| mNRRMSC | 23.008 clause 2.7.7 |  |
| subscriberStatus | 23.008 clause 2.8.1 |  |
| barringOfOutgoingCalls | 23.008 clause 2.8.2.1 |  |
| barringOfIncomingCalls | 23.008 clause 2.8.2.2 |  |
| barringOfRoaming | 23.008 clause 2.8.2.3 |  |
| barringOfPremiumRateCalls | 23.008 clause 2.8.2.4 |  |
| barringOfSupplementaryServiceManagement | 23.008 clause 2.8.2.5 |  |
| barringOfRegistrationOfCallForwarding | 23.008 clause 2.8.2.6 |  |
| barringOfInvocationOfCallTransfer | 23.008 clause 2.8.2.7 |  |
| barringofPacketOrientedServices | 23.008 clause 2.8.2.8 |  |
| operatorDeterminedBarringPLMNspecificData | 23.008 clause 2.8.3 |  |
| notificationToCSEFlagForODB | 23.008 clause 2.8.4 |  |
| gsmSCFaddresslistForODB | 23.008 clause 2.8.5 |  |
| messagesWaitingData | 23.008 clause 2.10.1 |  |
| mobileStationNotReachableFlag | 23.008 clause 2.10.2 |  |
| memoryCapacityExceededFlag | 23.008 clause 2.10.3 |  |
| traceReference | 23.008 clause 2.11.1 |  |
| traceType | 23.008 clause 2.11.2 |  |
| operationsSystemsIdentity | 23.008 clause 2.11.3 |  |
| hLRTraceType | 23.008 clause 2.11.4 |  |
| mAPErrorOnTrace | 23.008 clause 2.11.5 |  |
| traceActivatedInVLR | 23.008 clause 2.11.6 |  |
| traceActivatedinSGSN | 23.008 clause 2.11.7 |  |
| vGCSGroupMembershipList | 23.008 clause 2.12.1 |  |
| vBSGroupMembershipList | 23.008 clause 2.12.2 |  |
| broadcastCallInitiationAllowedList | 23.008 clause 2.12.2.1 |  |
| pDPType | 23.008 clause 2.13.1 |  |
| gGSNlist | 23.008 clause 2.13.11 |  |
| qualityofServiceSubscribed | 23.008 clause 2.13.12 |  |
| pDPAddress | 23.008 clause 2.13.2 |  |
| pDPContextIdentifier | 23.008 clause 2.13.24 |  |
| pDPContextChargingCharacteristics | 23.008 clause 2.13.25 |  |
| accessPointName | 23.008 clause 2.13.6 |  |
| vPLMNAddressAllowed | 23.008 clause 2.13.8 |  |
| vPLMNAddressAllowed-EPS | 23.008 clause 2.13.40 |  |
| originatingCAMELSubscriptionInformationOCSI | 23.008 clause 2.14.1.1/3.1 |  |
| gPRSCAMELSubscriptionInformationGPRSCSI | 23.008 clause 2.14.1.10/2.14.4.4 |  |
| dialledServiceCAMELSubscriptionInformationDCSI | 23.008 clause 2.14.1.11/3.7 |  |
| mobilityManagementforGPRSeventnotificationMGCSI | 23.008 clause 2.14.1.12/2.14.4.4 |  |
| terminatingCAMELSubscriptionInformationTCSI | 23.008 clause 2.14.1.2 |  |
| vMSCTerminatingCAMELSubscriptionInformationVTCSI | 23.008 clause 2.14.1.2/3.2 |  |
| locationInformationSubscriberStateInformation | 23.008 clause 2.14.1.3 |  |
| uSSDCAMELSubscriptionInformationUCSI | 23.008 clause 2.14.1.4 |  |
| sSInvocationNotificationSSCSI | 23.008 clause 2.14.1.5/3.2 |  |
| translationInformationFlagTIFCSI | 23.008 clause 2.14.1.6/3.6 |  |
| shortMessageServiceCAMELSubscriptionInformationMOSMSCSI | 23.008 clause 2.14.1.8/2.14.3.5 |  |
| mOShortMessageServiceCAMELSubscriptionInformationMOSMSCSI | 23.008 clause 2.14.1.8/2.14.4.1 |  |
| shortMessageServiceCAMELSubscriptionInformationMTSMSCSI | 23.008 clause 2.14.1.9/2.14.3.6 |  |
| mTShortMessageServiceCAMELSubscriptionInformationMTSMSCSI | 23.008 clause 2.14.1.9/2.14.4.2. |  |
| gPRSCSINegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| mCSINegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| mGCSINegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| mOSMSCSISGSNNegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| mOSMSCSIVLRNegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| mTSMSCSISGSNNegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| mTSMSCSIVLRNegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| oCSINegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| sSCSINegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| vTCSINegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| SGSNOfferedCAMEL4CSIs | 23.008 clause 2.14.2.2A |  |
| vLROfferedCAMEL4CSIs | 23.008 clause 2.14.2.2A |  |
| sGSNSupportedCAMELPhases | 23.008 clause 2.14.2.2 |  |
| vLRSupportedCAMELPhases | 23.008 clause 2.14.2.2 |  |
| gsmSCFaddressforCSI | 23.008 clause 2.14.2.4 |  |
| uSSDGeneralCAMELServiceInformationUGCSI | 23.008 clause 2.14.2.3 |  |
| iSTAlertTimer | 23.008 clause 2.15.1 |  |
| privacyExceptionList | 23.008 clause 2.16.1.1 |  |
| gMLCNumbers | 23.008 clause 2.16.1.2 |  |
| mOLRList | 23.008 clause 2.16.1.3 |  |
| serviceTypes | 23.008 clause 2.16.1.4 |  |
| ageIndicator | 23.008 clause 2.17.1 |  |
| cSAllocationRetentionpriority | 23.008 clause 2.18.1 |  |
| subscribedChargingCharacteristics | 23.008 clause 2.19.1 |  |
| iMSPrivateUserIdentity | 23.008 clause 3.1.1 |  |
| publicUserIdentity | 23.008 clause 3.1.2 |  |
| barringIndication | 23.008 clause 3.1.3 |  |
| listofauthorizedvisitednetworkidentifiers | 23.008 clause 3.1.4 23.008 clause 3B.1.4 |  |
| registrationStatus | 23.008 clause 3.2.1 |  |
| sCSCFName | 23.008 clause 3.2.2 |  |
| diameterClientAddressofSCSCF | 23.008 clause 3.2.3 |  |
| serverCapabilities | 23.008 clause 3.4.1 |  |
| initialFilterCriteriaList | This attribute is a list which contains multiple items of initialFilterCriteria (Ref. 23.008 clause 3.5.2) |  |
| applicationServerInformation | 23.008 clause 3.5.3 |  |
| serviceIndicationList | This attribute is a list which contains multiple items of serviceIndication (Ref. 23.008 clause 3.5.4) |  |
| subscribedMediaProfileIdentifier | 23.008 clause 3.6.1 |  |
| primaryEventChargingFunctionName | 23.008 clause 3.7.1 23.008 clause 3B.7.4 |  |
| secondaryEventChargingFunctionName | 23.008 clause 3.7.2 23.008 clause 3B.7.5 |  |
| primaryChargingCollectionFunctionName | 23.008 clause 3.7.3 |  |
| secondaryChargingCollectionFunctionName | 23.008 clause 3.7.4 |  |
| gsmSCFaddressforIMCSI | 23.008 clause 3.8.4 |  |
| iMSSFaddressforIMCSI | 23.008 clause 3.8.5 |  |
| oIMCSI | 23.008 clause 3.8.1 |  |
| vTIMCSI | 23.008 clause 3.8.2 |  |
| dIMCS | 23.008 clause 3.8.3 |  |
| gAAServiceType | 23.008 clause 3A.1 |  |
| gAAServiceIdentifier | 23.008 clause 3A.2 |  |
| gBAUserSecuritySettings | 23.008 clause 3A.3 |  |
| userSecuritySetting | 23.008 clause 3A.4 |  |
| userPublicIdentity | 23.008 clause 3A.5 |  |
| gAAAuthorizationflag | 23.008 clause 3A.6 |  |
| keyLifetime | 23.008 clause 3A.8 |  |
| uICCSecuritySetting | 23.008 clause 3A.9 |  |
| nAFGroupIdentity | 23.008 clause 3A.11 |  |
| w-APN | 23.008 clause 3B.1.3 |  |
| 3GPPAAAServerName | 23.008 clause 3B.1.6 |  |
| wLANUERemoteIPaddress | 23.008 clause 3B.1.10 |  |
| staticWLANUERemoteIPAddressList | 23.008 clause 3B.5.1.4 |  |
| w-APNBarringTypeList | 23.008 clause 3B.5.1.2 |  |
| w-APNChargingDataList | 23.008 clause 3B.5.1.3 |  |
| accessDependenceFlag | 23.008 clause 3B.5.2 |  |
| i-WLANAccessType | 23.008 clause 3B.5.3 |  |
| chargingCharacteristics | 23.008 clause 3B.7.1.1 |  |
| primaryOCSChargingFunctionName | 23.008 clause 3B.7.2 |  |
| secondaryOCSChargingFunctionName | 23.008 clause 3B.7.3 |  |
| sS-Code | 23.008 clause 2.6 29.002 clause 7.6.4.1 |  |
| provisioningState | 23.008 clause 2.6 29.002 clause 7.6.4.2 |  |
| activationState | 23.008 clause 2.6 29.002 clause 7.6.4.2 |  |
| registrationState | 23.008 clause 2.6 29.002 clause 7.6.4.2 |  |
| overrideCategory | 23.008 clause 2.6 29.002 clause 7.6.4.4 |  |
| cLIRestrictionOption | 23.008 clause 2.6 29.002 clause 7.6.4.5 |  |
| forwarded-toNumber | 23.008 clause 2.6 29.002 clause 7.6.2.22 |  |
| forwarded-toSubaddress | 23.008 clause 2.6 29.002 clause 7.6.2.23 |  |
| noReplyConditionTimer | 23.008 clause 2.6 29.002 clause 7.6.4.7 |  |
| maximumEntitledPriority | 23.008 clause 2.6 29.002 clause 7.6.4.41 |  |
| defaultPriority | 23.008 clause 2.6 29.002 clause 7.6.4.41 |  |
| nbrUser | 23.008 clause 2.6 29.002 clause 7.6.4.47 |  |
| nbrSB | 23.008 clause 2.6 29.002 clause 7.6.4.47 |  |
| TraceReference2 | 23.008 Clause 2.11.9 |  |
| TraceDepth | 23.008 Clause 2.11.10 |  |
| ListofNeTypestoTrace | 23.008 Clause 2.11.11 |  |
| TriggeringEvents | 23.008 Clause 2.11.12 |  |
| ListofInterfacestoTrace | 23.008 Clause 2.11.13 |  |
| iCSIndicator | 23.008 Clause 2.20.1 |  |
| iCSIndicator | 23.008 Clause 2.20.1 |  |
| sTNSR | 23.008 Clause 2.21.1 |  |
| 3GPPAAAProxyName | 23.008 Clause 3B.1.5 |  |
| w-APNIdentifierList | 23.008 Clause 3B.5.1.1 |  |
| maxNumberAccessesList | 23.008 Clause 3B.5.1.5 |  |
| maxSubscribedBandwidth | 23.008 Clause 3B.6.1 |  |
| subscribed3GPPWLANQoSProfile | 23.008 Clause 3B.6.4 |  |
| userStatus | 23.008 Clause 3B.2.1 |  |
| rATType | 23.008 Clause 2.13.75 |  |
| iPAddressOfTraceCollectionEntity | 23.008 Clause 2.11.14 |  |
| aPNConfigurationProfile | 23.008 Clause 2.13.31 |  |
| pDNAddress | 23.008 Clause 2.13.39 |  |
| permanentUserIdentity | 23.008 Clause 2.13.79 |  |
| visitedNetworkIdentifier | 23.008 Clause 2.13.82 |  |
| mIPSubscriberProfile | 23.008 Clause 2.13.86 |  |
| aPNAMBR | 23.008 Clause 2.13.32 |  |
| contextIdentifier | 23.008 Clause 2.13.38 |  |
| aPNOIReplacement | 23.008 Clause 2.13.29 |  |
| subscribedUEAMBR | 23.008 Clause 2.13.30 |  |
| rFSP | 23.008 Clause 2.13.33 |  |
| pDNGWIdentity | 23.008 Clause 2.13.41 |  |
| ePSPDNConnectionChargingCharacteristics | 23.008 Clause 2.13.71 |  |
| pDNGWAllocationType | 23.008 Clause 2.13.105 |  |
| aPNLevelAPNOIReplacement | 23.008 Clause 2.13.109 |  |
| closedSubscriberGroupInformation | 23.008 Clause 2.4.22 |  |
| roamingRestrictionDueToUnsupportedFeatureMME | 23.008 Clause 2.4.15.3a |  |
| mSPurgedForEPSFlag | 23.008 Clause 2.7.6A |  |
| uRRPMME | 23.008 Clause 2.7.9.1 |  |
| mMEName | 23.008 Clause 2.13.26 |  |
| sGSNName | 23.008 Clause 2.13.100 |  |
| iWFNumber | 23.008 Clause 2.4.8.3 |  |

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Dec 2003 | SA_22 | SP-030617 | -- | -- | Submitted to TSG SA#22 for Information | -- | 1.0.0 |  |
| Sep 2004 | SA_25 | SP-040546 | -- | -- | Submitted to TSG SA#25 for Approval | -- | 2.0.0 |  |
| Dec 2004 | SA_26 | SP-040830 | -- | -- | Submitted to TSG SA#26 for Approval | -- | 2.7.0 | 6.0.0 |
| Mar 2005 | SA_27 | SP-050024 | 0001 | -- | Add Generic Authentication Architecture (GAA) attributes – Align with CN4’s 23.008 (Organisation of subscriber data) | F | 6.0.0 | 6.1.0 |
| Mar 2005 | SA_27 | SP-050024 | 0002 | -- | Add Interworking WLAN (I-WLAN) parameters – Align with CN4’s 23.008 (Organisation of subscriber data) | F | 6.0.0 | 6.1.0 |
| Mar 2005 | SA_27 | SP-050024 | 0003 | -- | Add Supplementary Service attributes – Align with CN4’s 23.008 (Organisation of subscriber data) | F | 6.0.0 | 6.1.0 |
| Jun 2005 | SA_28 | SP-050269 | 0004 | -- | Add Rel-6 trace parameters to SuM - align with CT4's TS 23.008 (Organisation of subscriber data) | F | 6.1.0 | 6.2.0 |
| Mar 2006 | SA_31 | SP-060092 | 0005 | -- | Remove duplicate attributes in the Information Service (IS) | F | 6.2.0 | 6.3.0 |
| Jun 2007 | SA_36 | SP-070269 | 0006 | -- | Add definition of missing attributes | F | 6.3.0 | 6.4.0 |
| Jun 2007 | SA_36 | SP-070269 | 0007 | -- | Add missing inheritance definitions | F | 6.3.0 | 6.4.0 |
| Jun 2007 | SA_36 | -- | -- | -- | Automatic upgrade to Rel-7 (no CR) at freeze of Rel-7. | -- | 6.4.0 | 7.0.0 |
| Sep 2007 | SA_37 | SP-070607 | 0009 | -- | Add missing subscriberStatus attribute to PS object class - Align with CT4’s 23.008 (Organisation of subscriber data) | A | 7.0.0 | 7.1.0 |
| Sep 2007 | SA_37 | SP-070614 | 0011 | -- | Update information attribute definitions - Align with CT4 23.008 and 29.002 | F | 7.1.0 | 8.0.0 |
| Sep 2007 | SA_37 | SP-070614 | 0012 | -- | Changes to modelling of User | C | 7.1.0 | 8.0.0 |
| Sep 2009 | SA_45 | SP-090534 | 0013 | -- | Correction of SuMSubscriptionProfile and suMSubscriptionProfileId | F | 8.0.0 | 8.1.0 |
| Dec 2009 | - | - | - | - | Update to Rel-9 version (MCC) | - | 8.1.0 | 9.0.0 |
| Mar 2010 | SA_47 | SP-100035 | 0014 | - | Aligning SuM with the agreed methodology for attribute name in release 9 | F | 9.0.0 | 9.1.0 |
| Mar 2010 | SA_47 | SP-100035 | 0015 | - | The splitting of naming and containment | C | 9.0.0 | 9.1.0 |
| Mar 2010 | SA_47 | SP-100035 | 0016 | - | Adding missing IMS related parameters | F | 9.0.0 | 9.1.0 |
| Mar 2010 | SA_47 | SP-100035 | 0017 | - | Removing Visibility column in SuM IS | F | 9.0.0 | 9.1.0 |
| Mar 2010 | SA_47 | SP-100035 | 0018 | - | Remove misleading text | F | 9.0.0 | 9.1.0 |
| Mar 2010 | SA_47 | SP-100035 | 0019 | - | Correcting CS Service Profile parameters | F | 9.0.0 | 9.1.0 |
| Mar 2010 | SA_47 | SP-100035 | 0020 | - | Introducing new IOCs for common data and user identity parameters | C | 9.0.0 | 9.1.0 |
| Apr 2010 | -- | -- | -- | -- | Removal of erroneous hyperlinks | - | 9.1.0 | 9.1.1 |
| Jun 2010 | SA_48 | SP-100263 | 0011 | -- | Correcting and extending Packet Access service parameters and modeling | C | 9.1.1 | 9.2.0 |
| Sep 2010 | SA_49 | SP-100494 | 0022 | -- | SuM IS - removing some SuM IOCs from Itf-N | C | 9.2.0 | 10.0.0 |