8

3GPP TS 32.692 V10.1.0 (2011-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Inventory Management (IM) network resources

Integration Reference Point (IRP);

Network Resource Model (NRM)

(Release 10)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

GSM, UMTS, management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2011, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	7

4	System overview	8

4.1	Void	8

4.2	Compliance rules	8

5	Modelling approach	8

6	Information Object Classes	9

6.1	Imported information entities and local labels	9

6.2	Class diagram	9

6.2.1	Attributes and relationships	9

6.2.2	Inheritance	12

6.3	Information object class definitions	13

6.3.1	InventoryUnit	13

6.3.1.1	Definition	13

6.3.1.2	Attributes	13

6.3.1.3	Attribute constraints	13

6.3.1.4	Relationships	13

6.3.1.5	State diagram	13

6.3.1.6	Notifications	13

6.3.2	InventoryUnitNE	13

6.3.2.1	Definition	13

6.3.2.2	Attributes	14

6.3.2.3	Attribute constraints	14

6.3.2.4	Relationships	14

6.3.2.5	State diagram	14

6.3.2.6	Notifications	14

6.3.3	InventoryUnitHw	14

6.3.3.1	Definition	14

6.3.3.2	Attributes	14

6.3.3.3	Attribute constraints	15

6.3.3.4	Relationships	15

6.3.3.5	State diagram	15

6.3.3.6	Notifications	15

6.3.4	InventoryUnitSw	15

6.3.4.1	Definition	15

6.3.4.2	Attributes	15

6.3.4.3	Attribute constraints	15

6.3.4.4	Relationships	15

6.3.4.5	State diagram	15

6.3.4.6	Notifications	15

6.3.5	InventoryUnitLic	16

6.3.5.1	Definition	16

6.3.5.2	Attributes	16

6.3.5.3	Attribute constraints	16

6.3.5.4	Relationships	16

6.3.5.5	State diagram	16

6.3.5.6	Notifications	16

6.3.6	TmaInventoryUnit	16

6.3.6.1	Definition	16

6.3.6.2	Attributes	17

6.3.6.3	Attribute constraints	17

6.3.6.4	Notifications	17

6.3.7	AntennaInventoryUnit	17

6.3.7.1	Definition	17

6.3.7.2	Attributes	18

6.3.7.3	Attribute constraints	18

6.3.7.4	Notifications	18

6.4	Information relationship definitions	18

6.4.1	MFNERelation (O)	18

6.4.1.1	Definition	18

6.4.1.2	Roles	18

6.4.1.3	Constraints	18

6.4.2	NESWRelation (O)	18

6.4.2.1	Definition	18

6.4.2.2	Roles	19

6.4.2.3	Constraints	19

6.4.3	NEHWRelation (O)	19

6.4.3.1	Definition	19

6.4.3.2	Roles	19

6.4.3.3	Constraints	19

6.4.4	NELICRelation (O)	19

6.4.4.1	Definition	19

6.4.4.2	Roles	19

6.4.4.3	Constraints	19

6.4.5	SWLICRelation (O)	19

6.4.5.1	Definition	19

6.4.5.2	Roles	19

6.4.5.3	Constraints	19

6.4.6	SWHWRelation (O)	20

6.4.6.1	Definition	20

6.4.6.2	Roles	20

6.4.6.3	Constraints	20

6.4.7	HWLICRelation (O)	20

6.4.7.1	Definition	20

6.4.7.2	Roles	20

6.4.7.3	Constraints	20

6.4.8	MFSWRelation (O)	20

6.4.8.1	Definition	20

6.4.8.2	Roles	20

6.4.8.3	Constraints	20

6.4.9	MFHWRelation (O)	20

6.4.9.1	Definition	20

6.4.9.2	Roles	21

6.4.9.3	Constraints	21

6.4.10	MFLICRelation (O)	21

6.4.10.1	Definition	21

6.4.10.2	Roles	21

6.4.10.3	Constraints	21

6.4.11	Inventory-Function (M)	21

6.4.11.1	Definition	21

6.4.11.2	Roles	21

6.4.11.3	Constraints	21

6.5	Information attribute definitions	22

6.5.1	Definition and legal values	22

6.5.2	Constraints	25

6.6	Particular information configurations	25

Annex A (informative):	Change history	26

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; as identified below:

32.690:	"Inventory Management (IM): Requirements".

32.691:	"Inventory Management (IM) network resources Integration Reference Point (IRP): Requirements".

32.692:	"Inventory Management (IM) network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

32.696:	Inventory Management (IM) network resources Integration Reference Point (IRP): Solution Set (SS) definitions.

Inventory Management (IM), in general, provides the operator with the ability to assure correct and effective operation of the 3G network as it evolves. IM actions have the objective to monitor the actual configuration on the Network Elements (NEs) and Network Resources (NRs), and they may be initiated by the operator or by functions in the Operations Systems (OSs) or NEs. The final goal of IM is the establishment of an accurate and timely model of the actual inventory in the NEs or NRs.

IM actions may be requested to reflect changes initiated by Configuration Management (CM) actions or to make sure that the inventory model is in synch with the actual inventory. IM actions are initiated either as single actions on single NEs of the 3G network or as part of a complex procedure involving actions on many resources/objects in one or several NEs.

# 1	Scope

The present document defines an Integration Reference Point (IRP) through which an 'IRPAgent' (typically an Element Manager or Network Element) can communicate Network Management related information to one or several 'IRPManagers' (typically Network Managers).

The present document specifies an Inventory Management Network Resource Model, NRM (also referred to as a Management Information Model - MIM) with definitions of Information Object Classes.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.302: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): Information Service (IS)".

[4]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[5]	Void.

[6]	3GPP TS 32.642: "Telecommunication management; Configuration Management (CM): UTRAN network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[7]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[8]	3GPP TS 32.150: "Telecommunication management; Integration Reference Point (IRP) Concept and Definitions".

[9]	3GPP TS 32.151: "Telecommunication management; Integration Reference Point (IRP) Information Service (IS) template.

[10]	3GPP TS 32.622: "Telecommunication management; Configuration Management (CM); Generic network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[11]	3GPP TS 32.690: "Telecommunication management; Inventory Management (IM): Requirements".

[12]		3GPP TS 25.466: "UTRAN Iuant interface: Application Part".

[13]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[14]	3GPP TS 32.792: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TS 32.101 [1], 3GPP TS 32.102 [2] and 3GPP TS 32.600 [4] and the following apply:

association: in general it is used to model relationships between Managed Objects
Associations can be implemented in several ways, such as:

- name bindings;

- reference attributes; and

- association objects.

This IRP stipulates that containment associations shall be expressed through name bindings, but it does not stipulate the implementation for other types of associations as a general rule. These are specified as separate entities in the object models (UML diagrams). Currently however, all (non-containment) associations are modelled by means of reference attributes of the participating MOs.

Managed Element (ME): instance of the Managed Object Class Managed Element defined in [6]

Managed Object (MO): in the context of the present document, a Managed Object (MO) is a software object that encapsulates the manageable characteristics and behaviour of a particular Network Resource
The MO is instance of a class defined in a MIM/NRM. This class, called Information Object Class (IOC) has attributes that provide information used to characterize the objects that belong to the class (the term "attribute" is taken from TMN and corresponds to a "property" according to CIM). Furthermore, the IOC can have operations that represent the behaviour relevant for that class (the term "operation" is taken from TMN and corresponds to a "method" according to CIM). The IOC may support the emission of notifications that provide information about an event occurrence within a network resource.

Management Information Model (MIM): also referred to as NRM (see the NRM definition)

Network Resource Model (NRM): model representing the actual managed telecommunications network resources that a System is providing through the subject IRP
An NRM identifies and describes the IOCs, their associations, attributes and operations. The NRM is also referred to as "MIM" (see above), which originates from the ITU-T TMN.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

CIM	Common Information Model

DN	Distinguished Name (see 3GPP TS 32.300 [7])

EM	Element Manager

IM	Inventory Management

IOC	Information Managed Object

IRP	Integration Reference Point

ITU-T	International Telecommunication Union, Telecommunication Sector

ME	Managed Element

MIM	Management Information Model

MO	Managed Object

MOC	Managed Object Class

NE	Network Element

NM	Network Manager

NR	Network Resource

NRM	Network Resource Model

RDN	Relative Distinguished Name (see 3GPP TS 32.300 [7])

TMN	Telecommunications Management Network

UML	Unified Modelling Language

UMTS	Universal Mobile Telecommunications System

UTRAN	UMTS Terrestrial Radio Access Network

# 4	System overview

## 4.1	Void

## 4.2	Compliance rules

The following defines the meaning of Mandatory and Optional IOC attributes and associations between IOCs, in Solution Sets to the IRP defined by the present specification:

- The IRPManager shall support all mandatory attributes/associations. The IRPManager shall be prepared to receive information related to mandatory as well as optional attributes/associations without failure; however the IRPManager does not have to support handling of the optional attributes/associations.

- The IRPAgent shall support all mandatory attributes/associations. It may support optional attributes/associations.

An IRPAgent that incorporates vendor-specific extensions shall support normal communication with a 3GPP SA5-compliant IRPManager with respect to all Mandatory and Optional managed object classes, attributes, associations, operations, parameters and notifications without requiring the IRPManager to have any knowledge of the extensions.

Given that:

- rules for vendor-specific extensions remain to be fully specified; and

- many scenarios under which IRPManager and IRPAgent interwork may exist;

it is recognised that the IRPManager, even though it is not required to have knowledge of vendor-specific extensions, may be required to be implemented with an awareness that extensions can exist and behave accordingly.

# 5	Modelling approach

See 3GPP TS 32.150 [8].

# 6	Information Object Classes

## 6.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| 32.622 [10], information object class, Top | Top |
| 32.622 [10], information object class, ManagedElement | ManagedElement |

## 6.2	Class diagram

### 6.2.1	Attributes and relationships

This clause depicts the set of IOCs that encapsulate information relevant for this service. This clause provides the overview of all information object classes in UML. Subsequent clauses provide more detailed specification of various aspects of these information object classes.

The inventory NRM contains two alternatives for inventory data modeling. Alternative 1 is for NE structure and hardware inventory. Alternative 2 is an extended version for inventory information modeling consisting of NE structure, hardware, software and license data inventory.

Alternative 1, hardware inventory model

NOTE:	The listed cardinality numbers represent transient as well as steady-state numbers, and reflect all managed object creation and deletion scenarios.

Figure 6.2.1: Alternative 1 - Inventory Management NRM Containment/Naming and Association diagram

Each IOC instance is identified with a Distinguished Name (DN) according to 3GPP TS 32.300 [7] that expresses its containment hierarchy. As an example, the DN of a IOC representing a InventoryUnit could have a format like:

SubNetwork=Sweden,meContext=MEC-Gbg-1,ManagedElement=RNC-Gbg-1,InventoryUnit=Inv-1.

Alternative 2, extended model for hardware, software and licence inventory:

Figure 6.2.1-2: Alternative 2 - Inventory Management NRM Containment/Naming and Association diagram

NOTE:	Inventory information upload in alternative 2 is done using the the FT IRP and related FT IRP notification capabilites

### 6.2.2	Inheritance

This subclause depicts the inheritance relationships that exist between IOCs.

Figure 6.2.2 shows the inheritance hierarchy for the IM NRM.

Figure 6.2.2: Inventory Management NRM Inheritance Hierarchy

## 6.3	Information object class definitions

### 6.3.1	InventoryUnit

#### 6.3.1.1	Definition

This IOC represents inventory information for an Inventory Unit.

#### 6.3.1.2	Attributes

Attributes of InventoryUnit

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| inventoryUnitType | M | M | - |
| vendorUnitFamilyType | CM | M | - |
| vendorUnitTypeNumber | CM | M | - |
| versionNumber | O | M | - |
| vendorName | M | M | - |
| serialNumber | CM | M | - |
| dateOfManufacture | O | M | - |
| dateOfLastService | O | M | - |
| unitPosition | O | M | - |
| manufacturerData | O | M | - |
| relatedFunction | O | M | - |

#### 6.3.1.3	Attribute constraints

| Name | Definition |
| --- | --- |
| vendorUnitFamilyType CM Support Qualifier | The inventory is hardware. |
| vendorUnitTypeNumber CM Support Qualifier | The inventory is hardware. |
| serialNumber CM Support Qualifier | The inventory is hardware. |

#### 6.3.1.4	Relationships

None.

#### 6.3.1.5	State diagram

None.

#### 6.3.1.6	Notifications

None.

### 6.3.2	InventoryUnitNE

#### 6.3.2.1	Definition

This SupportIOC represents the logical and physical structure of the NE.

#### 6.3.2.2	Attributes

Attributes of InventoryUnitNE

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M |  |
| neId | M | M |  |
| customerIdentifier | O | M | - |
| productName | M | M | - |
| vendorName | M | M | - |
| productType | O | M | - |
| salesUniqueId | O | M | - |
| operatorUniqueName | O | M | - |
| siteId | O | M | - |
| additionalInformation | O | M | - |

#### 6.3.2.3	Attribute constraints

None.

#### 6.3.2.4	Relationships

None.

#### 6.3.2.5	State diagram

None.

#### 6.3.2.6	Notifications

None.

### 6.3.3	InventoryUnitHw

#### 6.3.3.1	Definition

This SupportIOC represents the hardware components.

#### 6.3.3.2	Attributes

Attributes of InventoryUnitHw

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| serialNo | M | M | - |
| hwType | M | M | - |
| hwName | O | M | - |
| hwVersion | M | M | - |
| vendorName | O | M | - |
| salesUniqueId | O | M | - |
| hwUnitLocation | M | M | - |
| model | O | M | - |
| hwCapability | O | M | - |
| modificationDate | O | M | - |
| manualDataEntry | O | M | - |
| additionalInformation | O | M | - |

#### 6.3.3.3	Attribute constraints

None.

#### 6.3.3.4	Relationships

None.

#### 6.3.3.5	State diagram

None.

#### 6.3.3.6	Notifications

None.

### 6.3.4	InventoryUnitSw

#### 6.3.4.1	Definition

This SupportIOC represents the software components.

#### 6.3.4.2	Attributes

Attributes of InventoryUnitSw

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| swId | M | M | - |
| swName | O | M | - |
| swVersion | O | M | - |
| vendorName | O | M | - |
| salesUniqueId | O | M | - |
| classification | M | M | - |
| swStatus | O | M | - |
| additionalInformation | O | M | - |

#### 6.3.4.3	Attribute constraints

None.

#### 6.3.4.4	Relationships

None.

#### 6.3.4.5	State diagram

None.

#### 6.3.4.6	Notifications

None.

### 6.3.5	InventoryUnitLic

#### 6.3.5.1	Definition

This SupportIOC represents the licence components.

#### 6.3.5.2	Attributes

Attributes of InventoryUnitLic

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| licId | M | M | - |
| licType | O | M | - |
| vendorName | O | M | - |
| validity | O | M | - |
| key | O | M | - |
| licStatus | O | M | - |
| salesUniqueId | O | M | - |
| additionalInformation | O | M | - |

#### 6.3.5.3	Attribute constraints

None.

#### 6.3.5.4	Relationships

None.

#### 6.3.5.5	State diagram

None.

#### 6.3.5.6	Notifications

None.

### 6.3.6	TmaInventoryUnit

#### 6.3.6.1	Definition

This IOC represents inventory information for a Tower Mounted Amplifier Unit.

#### 6.3.6.2	Attributes

Attributes of TmaInventoryUnit

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| tmaNumberOfNon-LinearGainValues | CM | M | - |
| tmaNon-LinearGainValue | CM | M | O |
| tmaAdditionalDataFieldNumber | CO | M | O |
| tmaAntennaModelNumber | CO | M | O |
| tmaAntennaOperatingBands | CO | M | O |
| tmaBeamwidthForEachOpBandInBandOrder | CO | M | O |
| tmaGainForEachOpBandInBandOrder | CO | M | O |
| tmaInstallationDate | CO | M | O |
| tmaInstallersId | CO | M | O |
| tmaMaxSupportedGain | CO | M | O |
| tmaMinSupportedGain | CO | M | O |

#### 6.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| tmaNumberOfNon-LinearGainValues CM Support Qualifier | It is supported over the Iuant interface. |
| tmaNon-LinearGainValue CM Support Qualifier | It is supported over the Iuant interface. |
| tmaAdditionalDataFieldNumber CO Support Qualifier | It is supported over the Iuant interface. |
| tmaAntennaModelNumber CO Support Qualifier | It is supported over the Iuant interface. |
| tmaAntennaOperatingBands CO Support Qualifier | It is supported over the Iuant interface. |
| tmaBeamwidthForEachOpBandInBandOrder CO Support Qualifier | It is supported over the Iuant interface. |
| tmaGainForEachOpBandInBandOrder CO Support Qualifier | It is supported over the Iuant interface. |
| tmaInstallationDate CO Support Qualifier | It is supported over the Iuant interface. |
| tmaInstallersId CO Support Qualifier | It is supported over the Iuant interface. |
| tmaMaxSupportedGain CO Support Qualifier | It is supported over the Iuant interface. |
| tmaMinSupportedGain CO Support Qualifier | It is supported over the Iuant interface. |

#### 6.3.6.4	Notifications

None

### 6.3.7	AntennaInventoryUnit

#### 6.3.7.1	Definition

This IOC represents inventory information for an Antenna Unit.

#### 6.3.7.2	Attributes

Attributes of AntennaInventoryUnit

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| maxTiltValue | CO | M | O |
| minTiltValue | CO | M | O |
| mechanicalOffset | CO | M | O |
| baseElevation | CO | M | O |
| latitude | CO | M | O |
| longitude | CO | M | O |
| patternLabel | CO | M | O |

#### 6.3.7.3	Attribute constraints

| Name | Definition |
| --- | --- |
| maxTiltValue CO Support Qualifier | It is supported over the Iuant interface. |
| minTiltValue CO Support Qualifier | It is supported over the Iuant interface. |
| mechanicalOffset CO Support Qualifier | It is supported over the Iuant interface. |
| baseElevation CO Support Qualifier | It is supported over the Iuant interface. |
| latitude CO Support Qualifier | It is supported over the Iuant interface. |
| longitude CO Support Qualifier | It is supported over the Iuant interface. |
| patternLabel CO Support Qualifier | It is supported over the Iuant interface. |

#### 6.3.7.4	Notifications

None.

## 6.4	Information relationship definitions

### 6.4.1	MFNERelation (O)

#### 6.4.1.1	Definition

This association represents the unidirectional relation between ManagedFunction and InventoryUnitNE.

#### 6.4.1.2	Roles

| Name | Definition |
| --- | --- |
| mFunction | This role represents the associated ManagedFunction instances of a InventoryUnitNE instance. |

#### 6.4.1.3	Constraints

None.

### 6.4.2	NESWRelation (O)

#### 6.4.2.1	Definition

This association represents the bidirectional relation between InventoryUnitNE and InventoryUnitSW.

#### 6.4.2.2	Roles

| Name | Definition |
| --- | --- |
| nEList | This role represents the associated InventoryUnitNE instances of a InventoryUnitSW instance. |
| sWList | This role represents the associated InventoryUnitSW instances of a InventoryUnitNE instance. |

#### 6.4.2.3	Constraints

None.

### 6.4.3	NEHWRelation (O)

#### 6.4.3.1	Definition

This association represents the bidirectional relation between InventoryUnitNE and InventoryUnitHW.

#### 6.4.3.2	Roles

| Name | Definition |
| --- | --- |
| nEList | This role represents the associated InventoryUnitNE instances of a InventoryUnitHW instance. |
| hWList | This role represents the associated InventoryUnitHW instances of a InventoryUnitNE instance. |

#### 6.4.3.3	Constraints

None.

### 6.4.4	NELICRelation (O)

#### 6.4.4.1	Definition

This association represents the bidirectional relation between InventoryUnitNE and InventoryUnitLIC.

#### 6.4.4.2	Roles

| Name | Definition |
| --- | --- |
| nEList | This role represents the associated InventoryUnitNE instances of a InventoryUnitLIC instance. |
| lICList | This role represents the associated InventoryUnitLIC instances of a InventoryUnitNE instance. |

#### 6.4.4.3	Constraints

None.

### 6.4.5	SWLICRelation (O)

#### 6.4.5.1	Definition

This association represents the bidirectional relation between InventoryUnitSW and InventoryUnitLIC.

#### 6.4.5.2	Roles

| Name | Definition |
| --- | --- |
| sWList | This role represents the associated InventoryUnitSW instances of a InventoryUnitLIC instance. |
| lICList | This role represents the associated InventoryUnitLIC instances of a InventoryUnitSW instance. |

#### 6.4.5.3	Constraints

None.

### 6.4.6	SWHWRelation (O)

#### 6.4.6.1	Definition

This association represents the bidirectional relation between InventoryUnitSW and InventoryUnitHW.

#### 6.4.6.2	Roles

| Name | Definition |
| --- | --- |
| sWList | This role represents the associated InventoryUnitSW instances of a InventoryUnitHW instance. |
| hWList | This role represents the associated InventoryUnitHW instances of a InventoryUnitSW instance. |

#### 6.4.6.3	Constraints

None.

### 6.4.7	HWLICRelation (O)

#### 6.4.7.1	Definition

This association represents the bidirectional relation between InventoryUnitHW and InventoryUnitLIC.

#### 6.4.7.2	Roles

| Name | Definition |
| --- | --- |
| hWList | This role represents the associated InventoryUnitHW instances of a InventoryUnitLIC instance. |
| lICList | This role represents the associated InventoryUnitLIC instances of a InventoryUnitHW instance. |

#### 6.4.7.3	Constraints

None.

### 6.4.8	MFSWRelation (O)

#### 6.4.8.1	Definition

This association represents the unidirectional relation between ManagedFunction and InventoryUnitSW.

#### 6.4.8.2	Roles

| Name | Definition |
| --- | --- |
| mFunction | This role represents the associated ManagedFunction instances of a InventoryUnitSW instance. |

#### 6.4.8.3	Constraints

None.

### 6.4.9	MFHWRelation (O)

#### 6.4.9.1	Definition

This association represents the unidirectional relation between ManagedFunction and InventoryUnitHW.

#### 6.4.9.2	Roles

| Name | Definition |
| --- | --- |
| mFunction | This role represents the associated ManagedFunction instances of a InventoryUnitHW instance. |

#### 6.4.9.3	Constraints

None.

### 6.4.10	MFLICRelation (O)

#### 6.4.10.1	Definition

This association represents the unidirectional relation between ManagedFunction and InventoryUnitLIC.

#### 6.4.10.2	Roles

| Name | Definition |
| --- | --- |
| mFunction | This role represents the associated ManagedFunction instances of a InventoryUnitLIC instance. |

#### 6.4.10.3	Constraints

None.

### 6.4.11	Inventory-Function (M)

#### 6.4.11.1	Definition

This association represents the unidirectional association from an InventoryUnit and its related ManagedFunctions.

#### 6.4.11.2	Roles

| Name | Definition |
| --- | --- |
| relatedFunction | This role represents the ManagedFunction that is associated with an InventoryUnit. |

#### 6.4.11.3	Constraints

None

## 6.5	Information attribute definitions

### 6.5.1	Definition and legal values

Table 6.5.1 defines the attributes that are present in several Information Object Classes of the present document.

Table 6.5.1: Attributes

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| baseElevation | The elevation in meters above sea level at the base of the antenna structure. This value, when subtracted from height (see TS 32.792 [14]), provides the height of the antenna above the ground. Note: The value of this attribute has no operational impact on the network, e.g. the NE behavior is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface according to Ref. 3GPP TS 25.466 [12]. | An integral value representing a number of meters in 0.1 meter increments. |
| additionalInformation | Supplementary information about inventory data (if any) |  |
| classification | Name of installed SW (e.g. SW release, SW build, SW patches), empty value possible |  |
| customerIdentifier | Unique identification of a vendors’ customer |  |
| dateOfManufacture | Date of Manufacture of inventory unit. |  |
| dateOfLastService | Date of last service or repair of inventory unit. |  |
| hwCapability | Hardware capability e.g. capacity, size (empty value is possible) |  |
| hwName | Mnemonic of hw inventory unit family type (e.g. Fan, PSU) assigned by vendor. |  |
| hwType | Type of the HW unit e.g. equipment holder, carriage |  |
| hwUnitLocation | Unique physical / logical location identifier within NE |  |
| hwVersion | Version / revision no. of current unit e.g. firmware version (empty value possible in case no versioning is available) |  |
| id | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of this object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| inventoryUnitId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of this object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| inventoryUnitType | Type of inventory unit (see TS 32.690 [11]) |  |
| key | License activation key according to the used licensing system |  |
| licId | Unique identifier of a license (e.g. name, code) |  |
| licStatus | License status – applicable only for managed licenses (e.g. scheduled, valid, expired, invalid, capacity violated) |  |
| licType | Describing type of current license (e.g. capacity, particular feature, no. of subscribers) |  |
| manualDataEntry | Indicates whether unit is passive (manual insertion of inventory data is needed) or active (inventory data can be read from the unit) |  |
| latitude | The latitude of the antenna location based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to the northern hemisphere. Note: The value of this attribute has no operational impact on the network, e.g. the NE behavior is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface according to Ref. 3GPP TS 25.466 [12]. | Valid values described in 3GPP TS 23.032 [13]. |
| longitude | The longitude of the antenna location based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to degrees east of 0 degrees longitude. Note: The value of this attribute has no operational impact on the network, e.g. the NE behavior is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface according to Ref. 3GPP TS 25.466 [12]. | Valid values described in 3GPP TS 23.032 [13] |
| manufacturerData | Manufacturer specific data of inventory unit. |  |
| model | Equipment configuration, e.g. standard hw unit or a variant that may contain additional disk capacity (empty value possible) |  |
| modificationDate | Date/time stamp of last change (e.g. repair action) |  |
| neId | vendor defined unique identifier of a logical or physical network element unit |  |
| operatorUniqueName | Unique NE identifier used by operator |  |
| productName | NE name classifying a vendor’s product family or function |  |
| productType | Identifier of the e.g. platform, in case the product can be based on different HW/SW platforms (not used for logical NEs) |  |
| salesUniqueId | Unique identifier used by vendor (used e.g. for ordering a new unit) |  |
| serialNo | Serial number given from factory |  |
| maxTiltValue | The maximum amount of tilt the RET system can support. "Maximum supported tilt" in Ref. 3GPP TS 25.466 [12]. | See "Maximum supported tilt" in Ref. 3GPP TS 25.466 [12]. |
| mechanicalOffset | This is a value representing a non-adjustable tilt value, which is imparted to the antenna due to the physical installation. The actual tilt at any point in time is the summation of mechanicalOffset and retTiltValue. Note: The value of this attribute has no operational impact on the network, e.g. the NE behavior is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface according to Ref. 3GPP TS 25.466 [12]. | A single integral value corresponding to an angle in degrees between 0 and 360 with a resolution of 0.1 degrees. |
| minTiltValue | The minimum amount of tilt the RET system can support, "Minimum supported tilt" in Ref. 3GPP TS 25.466 [12]. | See "Minimum supported tilt" in Ref. 3GPP TS 25.466 [12]. |
| patternLabel | The pattern name is a textual, alpha-numeric string to allow identification of the antenna pattern along with the antenna vendor information such as model number, etc. Note: The value of this attribute has no operational impact on the network, e.g. the NE behavior is not affected by the value setting of this attribute. Note as well that this attribute is not supported over the Iuant interface according to Ref. 3GPP TS 25.466 [12]. |  |
| relatedFunction | This attribute contains the DN of one ManagedFunction. |  |
| serialNumber | Serial number of inventory unit. |  |
| siteId | NE site in customer network |  |
| swId | Unique identifier of a SW unit |  |
| swName | SW release name used |  |
| swStatus | Status of the SW unit (e.g. installed, archived) |  |
| swVersion | Version identifier of the SW unit |  |
| tmaAdditionalDataFieldNumber | This attribute identifies a standard data field which has no operational impact. Used by the procedures SetDeviceData and GetDevicedata. Defined in Table B.3 of 3GPP TS 25.466 [12]. | Defined in TS 25.466 [12] |
| tmaAntennaModelNumber | A data field defined in Table B.3 of 3GPP TS 25.466 [12] | Defined in TS 25.466 [12] |
| tmaAntennaOperatingBands | A data field defined in Table B.3 of 3GPP TS 25.466 [12] | Defined in TS 25.466 [12] |
| tmaBeamwidthForEachOpBandInBandOrder | A data field defined in Table B.3 of 3GPP TS 25.466 [12] | Defined in TS 25.466 [12] |
| tmaGainForEachOpBandInBandOrder | A data field defined in Table B.3 of 3GPP TS 25.466 [12] | Defined in TS 25.466 [12] |
| tmaInstallationDate | A data field defined in Table B.3 of 3GPP TS 25.466 [12] | Defined in TS 25.466 [12] |
| tmaInstallersId | A data field defined in Table B.3 of 3GPP TS 25.466 [12] | Defined in TS 25.466 [12] |
| tmaMaxSupportedGain | A data field defined in Table B.3 of 3GPP TS 25.466 [12] | Defined in TS 25.466 [12] |
| tmaMinSupportedGain | A data field defined in Table B.3 of 3GPP TS 25.466 [12] | Defined in TS 25.466 [12] |
| tmaNon-LinearGainValue | Defined in 3GPP TS 25.466 [12] | Defined in TS 25.466 [12] |
| tmaNumberOfNon-LinearGainValues | Defined in 3GPP TS 25.466 [12] | Defined in TS 25.466 [12] |
| unitPosition | Position of inventory unit (e.g. Rack, shelf, slot, etc.). Depending on the implementation of the inventory unit in the managed system, the value and meaning of this attribute may vary. For example, if a system has three levels and types of inventory units representing Rack, Shelf and Slot respectively (i.e. the Managed Element contains multiple Rack inventory units, each Rack inventory unit contains multiple Shelf inventory units and each Shelf inventory unit contains multiple Slot inventory units), then for this example: - for the Inventory Unit representing a Rack, the Frame Identification code may be used as the value of this attribute; - for the Inventory Unit representing a Shelf, the Rack Shelf code may be used as the value of this attribute; - for the Inventory Unit representing a Slot, the position code may be used as the value of this attribute. |  |
| validity | License validity which may include one of the elements duration, end (expiration date) or forever |  |
| vendorName | Name of inventory unit vendor (or vendors may provide manufacturer name) |  |
| vendorName | Name of inventory unit vendor. |  |
| vendorUnitFamilyType | Mnemonic of inventory unit family type (e.g. Fan, PSU) assigned by vendor. |  |
| vendorUnitTypeNumber | A vendor/manufacturer defined and assigned number which uniquely identifies the unit type and optionally for backward compatibility reasons only, also version (used for replacing HW units, spares). |  |
| versionNumber | The version information related to vendorUnitTypeNumber. |  |

### 6.5.2	Constraints

None

## 6.6	Particular information configurations

None

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Sep 2002 | SA_17 | SP-020473 | -- | -- | Submitted to TSG SA #17 for Approval | -- | 1.0.0 | 5.0.0 |
| Dec 2004 | SA_26 | SP-040816 | 0001 | -- | Align Inventory Management Network Resource Model with the latest template from Rel-6 TS 32.150 | F | 5.0.0 | 6.0.0 |
| Jun 2005 | SA_28 | SP-050301 | 0002 | -- | Remove obsolete compliance text in 4.2 | F | 6.0.0 | 6.1.0 |
| Dec 2005 | SA_30 | SP-050714 | 0003 | -- | Correct support qualifier - Align IS with requirements in 32.690 | F | 6.1.0 | 6.2.0 |
| Jun 2006 | SA_32 | SP-060257 | 0004 | -- | Correction of InventoryUnit missing VsDataContainer and Version Number | F | 6.2.0 | 6.3.0 |
| Jun 2006 | SA_32 | SP-060257 | 0005 | -- | Correct the TS reference number from 32.691 to 32.690 | F | 6.2.0 | 6.3.0 |
| Jun 2007 | SA_36 | SP-070269 | 0006 | -- | Delete the duplicated vendorUnitTypeNumber attribute definitions | F | 6.3.0 | 6.4.0 |
| Jun 2007 | SA_36 | -- | -- | -- | Automatic upgrade to Rel-7 (no CR) at freeze of Rel-7. | -- | 6.4.0 | 7.0.0 |
| Dec 2008 | SA_42 | -- | -- | -- | Upgrade to Release 8 | -- | 7.0.0 | 8.0.0 |
| Dec 2009 | - | - | - | - | Update to Rel-9 version | -- | 8.0.0 | 9.0.0 |
| Mar 2011 | - | - | - | - | Update to Rel-10 version (MCC) |  | 9.0.0 | 10.0.0 |
| May 2011 | SA_52 | SP-110295 | 0007 | 1 | Add missing common RAT inventory | C | 10.0.0 | 10.1.0 |
| May 2011 | SA_52 | SP-110290 | 0008 | 2 | Inventory Model Enhancements - IS | B | 10.0.0 | 10.1.0 |