# Use of TAP for the Single IMSI Wholesale Billing Interface

# Version 1.0

# 15 July 2013

# Introduction

## Overview

This document explains how to use TAP for the Single IMSI Wholesale Billing Interface (SI-IF6) between the DSP and ARP.

## Scope

TAP3.12 has been used for the purpose of this document.  Details of file structure and data dictionary can be found in TD.57 [2].

Note: TD.57 [2] has been defined for the VPMN/HPMN wholesale billing interface.  The use of the document also for the DSP/ARP wholesale billing interface therefore needs to consider that it has been written to define the rules on how the VPMN must populate TAP.  When for example TD.57 [2] specifies “available from the network”, this shall be interpreted as “available to the DSP at the time of creation of the ARP wholesale billing file”.

## Definitions

| Term | Description |
| --- | --- |
| Calls | All call events defined for the SI-IF6 |

## Abbreviations

| Term | Description |
| --- | --- |
| ABF | Alternative Billing Format |
| ARP | Alternative Roaming Provider |
| BID | Billing Identifier |
| BS | Bearer Service |
| CAMEL | Customised Application Mobile Enhanced Logic |
| CLIR | Calling Line Identification Restriction |
| CSD | Circuit Switched Data |
| DSP | Domestic Service Provider |
| GPRS | General Packet Radio Service |
| GGSN | Gateway GPRS Support Node |
| GSM | Global System for Mobile communications (originally Groupe Spécial Mobile) |
| GSMA | GSM Association |
| HSCSD | High Speed Circuit Switched Data |
| HPMN | Home PMN |
| IF | Interface |
| IMEI | International Mobile station Equipment Identity |
| IMS | IP Multimedia Subsystem |
| IMSI | International Mobile Subscriber Identity |
| IP | Internet Protocol |
| LTE | Long Term Evolution |
| MCC | Mobile Country Code |
| MMS | Multimedia Messaging Service |
| MMSC | Multimedia Messaging Service Centre (Server/Relay or Store and Forward Server) |
| MNC | Mobile Network Code |
| MO | Mobile Originated |
| MOC | Mobile Originated Call |
| MSISDN | Mobile Station International Subscriber Directory Number |
| MT | Mobile Terminated |
| MTC | Mobile Terminated Call |
| MVNO | Mobile Virtual Network Operator |
| NI | Network Identification |
| OI | Operator Identification |
| PDP | Packet Data Protocol |
| PMN | Public Mobile Network |
| PSTN | Public Switched Telephone Network |
| PRD | Permanent Reference Document |
| RAP | Returned Account Procedure |
| RFC | Request for Comments |
| SGSN | Serving GPRS Support Node |
| SI | Single IMSI |
| SMS | Short Message Service |
| SMSC | Short Message Service Centre |
| SS | Supplementary Service |
| TADIG | Transferred Account Data Interchange Group |
| TAP | Transferred Account Procedure |
| TS | Teleservice |
| USSD | Unstructured Supplementary Service Data |
| UTC | Universal Time Co-ordinated |
| VPMN | Visited PMN |
| VT | Video Telephony |

## References

| Ref | Doc Number | Title |
| --- | --- | --- |
|  | GSMA PRD TD.28 | File Transfer Methods |
|  | GSMA PRD TD.57 | TAP3 Format Specification |
|  | High Level Technical Specifications | EU Roaming Regulation III Structural Solutions High Level Technical Specifications |
|  | RFC 2119 | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997. Available at http://www.ietf.org/rfc/rfc2119.txt |

## Conventions

The key words “must”, “must not”, “required”, “shall”, “shall not”, “should”, “should not”, “recommended”, “may”, and “optional” in this document are to be interpreted as described in RFC 2119 [4].

# General Requirements

The ARP will adapt to local DSP practices (including local currency).

Voice over LTE is charged using the Mobile Session record.  The underlying CDRs are based on 3GPP R11 where the routed destination has been added in addition to the requested destination.  GSMA TADIG is planning an emergency release of TAP3.12 in support of this change and to enable operators to charge based on the routed destination as well as to receive also the requested destination.

SMS over IP is charged using the Messaging Event record.

SMS-MT, Service Centre Usage, Content Transaction, Location Services, and Wi-Fi services are not needed.

TAP does not require use of standard GSMA roaming agreements, in particular AA.14s and IOT.  The billing format is independent of the format by which rates are exchanged.  The minimum content of the rate and operator information will be confirmed with the Processes subgroup.

RAP is not used to report on errors as it would make the interface too expensive and complicated for new entrants.  Instead in the case of errors a manual credit note process should be used between the ARP and the DSP.  Note: this process is manual and not standardised.

The TAP data needs to be sent to the current ARP for that customer as per the start time of the call event.

# Required Data Items

This section lists all required data items per call scenario, and how they should be populated on the DSP to ARP interface.

## File Level

### Batch Control Information

| Data Item | How to populate | Comments |
| --- | --- | --- |
| Sender (DSP) | TADIG code of DSP |  |
| Recipient (ARP) | TADIG code of ARP |  |
| File Sequence Number | Maintain sequence numbers for the DSP/ARP relationship |  |
| Transfer Cut Off Timestamp (including UTC Time Offset) | The timestamp used to select calls for transfer. All call records available prior to the timestamp are transferred. |  |
| File Available Timestamp (including UTC Time Offset) | Set to the time when the file is sent |  |
| Specification Version Number | Set to 3 |  |
| Release Version Number | Set to 12 |  |

### Accounting Information

| Data Item | How to populate | Comments |
| --- | --- | --- |
| Tax Rate Code | Repeat with unique code per unique Tax Rate in the call records | Only necessary if taxes apply. |
| Tax Type | Set to 01 | Assume national tax (if applicable). |
| Tax Rate | Set to actual tax rate used |  |
| Tax Indicator | Set to 1 | Assume Value Added Tax. If other type of tax, don’t populate. |
| Local Currency | ISO code of the Local Currency used | For example EUR for Euro. |
| TAP Currency | Set to the Local Currency | Assuming no currency translation, but both rates and files are in the same currency. |
| Exchange Rate Code | Set to 1 |  |
| Number of Decimal Places | Set to 0 |  |
| Exchange Rate | Set to 1 |  |
| TAP Decimal Places | Set to the number of decimal places required by the charge fields | This depends on how many decimals are used in the rates. |

### Network Information

| Data Item | How to populate | Comments |
| --- | --- | --- |
| UTC Time Offset Code | Repeat with unique code per unique UTC Time Offset in the call records |  |
| UTC Time Offset | Repeat with unique offset per unique UTC Time Offset in the call records |  |
| Recording Entity Code | Repeat with unique code per unique Recording Entity in the call records |  |
| Recording Entity Type | Set to the type of Recording Entity |  |
| Recording Entity Identification | Repeat with unique identification per unique Recording Entity in the call records | Populate with real Recording Entity Identification if available. If not available, populate with “UNKNOWN” or VPMN TADIG Code, or when an IP address is required populate with “1.1.1.1”. |

### Audit Control Information

| Data Item | How to populate | Comments |
| --- | --- | --- |
| Total Charge | Calculate from individual call records |  |
| Total Tax Value | Calculate from individual call records |  |
| Total Discount Value | Set to 0 |  |
| Call Event Details Count | Calculate from individual call records |  |

## Call Detail Level

Notes:

- The “normal” TAP use for SMS-MO is to carry the roaming charge.  Interconnect charges can be carried as well within the Mobile Originated Call record.  TAP also has a unique record type for SMS interconnect charges, the Service Centre Usage record, but as that is not used today in the industry the approach of including the interconnect charges with the roaming charges (broken out or not) has been chosen.
Note: This is further explained in the High Level Technical Specifications [4], section 4.1.3.4.

- MMS has been included in the GPRS call record to be charged as data.  There is flexibility within the data record to charge everything as data, or to charge the interconnect charge separately from the roaming charge, for example a fixed charge for the interconnect part and a volume charge for the roaming part.

- Invocation of conditional call forwarding can be recorded just like in TAP, inline in a MOC or offline in a separate Supplementary Service Event.

### Mobile Originated Call

| Data Item | How to populate | Comments |
| --- | --- | --- |
| IMSI | IMSI | If MSISDN is not populated, then IMSI must be provided in full. If MSISDN is populated, then population of the IMSI is at the discretion of the DSP (as a minimum the MCC/MNC of the HPMN/DSP must be populated). |
| MSISDN | MSISDN if available |  |
| Called Number | Called Number if available | For SMS-MO, this will contain the SMSC address. |
| Dialled Digits | Dialled Digits if available |  |
| SMS Destination Number | SMS Destination Number if available |  |
| Call Event Start Timestamp (including UTC Time Offset) | Call Event Start Timestamp (including UTC Time Offset) |  |
| Total Call Event Duration | Total Call Event Duration |  |
| Cause for Termination | Cause for Termination if available |  |
| Recording Entity Code | Numeric code | Referring to table in the Network Information. |
| Call Reference | Call Reference or Message Reference where available |  |
| Serving Network | TADIG code of the VPMN | For anti-tromboning and callback, this would be the TADIG code of the HPMN. This could possibly be used as an indication that anti-tromboning or callback has taken place. |
| IMEI | IMEI if available |  |
| TeleService Code | TeleService Code if available | One of TS or BS Code must be present. |
| Bearer Service Code | Bearer Service Code if available | One of TS or BS Code must be present |
| User Protocol Indicator | User Protocol Indicator if available | Used for VT, not strictly necessary as BS37 can be used instead |
| Charged Item | Populate with A, D, E and/or F depending on the service and charging principle | If fixed components are being charged (for example for unregulated services), then the whole Charge Information group can be repeated. |
| Exchange Rate Code | Populate with 1 | Referring to the table in the Accounting Information |
| Call Type Level 1 | Populate as per the rate, with 1 for national calls, and 2 for international calls | Note: The Call Type Levels are not copied from the incoming TAP file. Rather they must be in line with the DSP rate. |
| Call Type Level 2 | Populate as per the rate, for example with 0 for unknown | 0 Unknown/Not Applicable 1 Mobile 2 PSTN 3 Non Geographic 4 Premium Rate 5 Satellite destination |
| Call Type Level 3 | Populate as needed to uniquely identify the rate |  |
| Charge Type | Populate with 00 | Assume only total charge is needed. |
| Charge | As per the rate | Excluding any taxes |
| Chargeable Units | Populate in accordance with the Charged Item. For Charged Item = D, populate with the duration in seconds. |  |
| Tax Rate Code | Populate with the relevant code | Referring to the table in the Accounting Information. Only present if taxes apply. |
| Tax Value | The Tax Value | Only present if taxes apply. |
| HSCSD Indicator | Populate with 1 if the service is HSCSD | Rarely used. |
| Supplementary Service Code | Supplementary Service Code if available | Call forwarding indicator. |
| Third Party Number | Third Party Number if available |  |
| CLIR Status Indicator | CLIR Status Indicator if available |  |
| CAMEL Service Key | CAMEL Service Key if available |  |
| CAMEL Destination Number | CAMEL Destination Number if available |  |
| Operator Specific Information | Populate with additional information as agreed (or at the discretion of the Sender) | Free text field. Could be used to identify callback and anti-tromboning by use of special keywords: Callback AntiTromboning |

### Mobile Terminated Call

| Data Item | How to populate | Comments |
| --- | --- | --- |
| IMSI | IMSI | If MSISDN is not populated, then IMSI must be provided in full. If MSISDN is populated, then population of the IMSI is at the discretion of the DSP (as a minimum the MCC/MNC of the HPMN/DSP must be populated). |
| MSISDN | MSISDN if available |  |
| Calling Number | Calling Number if available | Note: This is generally not very reliable on TAP. Not all HPMNs provide it for retail billing. |
| CLIR Status Indicator | CLIR Status Indicator if available |  |
| SMS Originator | SMS Originator if available | If available for SMS-MT. |
| Call Event Start Timestamp (including UTC Time Offset) | Call Event Start Timestamp (including UTC Time Offset) |  |
| Total Call Event Duration | Total Call Event Duration |  |
| Cause for Termination | Cause for Termination if available |  |
| Recording Entity Code | Numeric code | Referring to table in the Network Information. |
| Call Reference | Call Reference or Message Reference where available |  |
| Serving Network | TADIG code of the VPMN |  |
| IMEI | IMEI if available |  |
| TeleService Cde | TeleService Code if available | One of TS or BS Code must be present. |
| Bearer Service Code | Bearer Service Code if available | One of TS or BS Code must be present |
| User Protocol Indicator | User Protocol Indicator if available | Used for VT, not strictly necessary as BS37 can be used instead |
| Charged Item | Populate with D, E and/or F depending on the service and charging principle | Assuming duration based charge (for example for unregulated services). If fixed components are being charged, then the whole Charge Information group can be repeated. |
| Exchange Rate Code | Populate with 1 | Referring to the table in the Accounting Information |
| Call Type Level 1 | Populate as per the rate, with 1 for national calls, and 2 for international calls, or 0 or unknown |  |
| Call Type Level 2 | Populate as per the rate, for example with 0 for unknown | 0 Unknown/Not Applicable 6 Forwarded call 7 Non forwarded call |
| Call Type Level 3 | Populate as needed to uniquely identify the rate |  |
| Charge Type | Populate with 00 | Assume only total charge is needed. If there is a need to breakout roaming and interconnect charges separately (for example for SMS) then 01 and 03 can be used in addition to 00 by repeating the Charge Detail group. |
| Charge | As per the rate | Excluding any taxes |
| Chargeable Units | Populate in accordance with the Charged Item. For Charged Item = D, populate with the duration in seconds. |  |
| Tax Rate Code | Populate with the relevant code | Referring to the table in the Accounting Information. Only present if taxes apply. |
| Tax Value | The Tax Value | Only present if taxes apply. |
| HSCSD Indicator | Populate with 1 if the service is HSCSD | Rarely used. |
| CAMEL Service Key | CAMEL Service Key if available |  |
| Operator Specific Information | Populate with additional information as agreed | Free text field. |

### Supplementary Service Event

| Data Item | How to populate | Comments |
| --- | --- | --- |
| IMSI | IMSI | If MSISDN is not populated, then IMSI must be provided in full. If MSISDN is populated, then population of the IMSI is at the discretion of the DSP (as a minimum the MCC/MNC of the HPMN/DSP must be populated). |
| MSISDN | MSISDN if available |  |
| Recording Entity Code | Numeric code | Referring to table in the Network Information. |
| Call Reference | Call Reference where available |  |
| Serving Network | TADIG code of the VPMN |  |
| IMEI | IMEI if available |  |
| Supplementary Service Code | Supplementary Service Code |  |
| Action Code | Action Code |  |
| Supplementary Service Parameters | Supplementary Service Parameters if available |  |
| Charging Timestamp (including UTC Time Offset) | Charging Timestamp (including UTC Time Offset) |  |
| Charged Item | Populate with E |  |
| Exchange Rate Code | Populate with 1 | Referring to the table in the Accounting Information |
| Charge Type | Populate with 00 |  |
| Charge | As per the rate | Excluding any taxes |
| Tax Rate Code | Populate with the relevant code | Referring to the table in the Accounting Information. Only present if taxes apply. |
| Tax Value | The Tax Value | Only present if taxes apply. |
| TeleService Code | TeleService Code if available |  |
| Bearer Service Code | Bearer Service Code if available |  |
| Operator Specific Information | Populate with additional information as agreed (or at the discretion of the Sender) | Free text field. |

### GPRS Call

| Data Item | How to populate | Comments |
| --- | --- | --- |
| IMSI | IMSI | If MSISDN is not populated, then IMSI must be provided in full. If MSISDN is populated, then population of the IMSI is at the discretion of the DSP (as a minimum the MCC/MNC of the HPMN/DSP must be populated). |
| MSISDN | MSISDN if available |  |
| PDP Address | PDP Address if available |  |
| Access Point Name NI | Access Point Name NI |  |
| Access Point Name OI | Access Point Name OI if available |  |
| Call Event Start Timestamp (including UTC Time Offset) | Call Event Start Timestamp (including UTC Time Offset) |  |
| Total Call Event Duration | Total Call Event Duration |  |
| Cause for Termination | Cause for Termination if available |  |
| Partial Type Indicator | Partial Type Indicator if available |  |
| PDP Context Start Timestamp (including UTC Time Offset) | PDP Context Start Timestamp (including UTC Time Offset) if available | Only for partial records. |
| Network Init. PDP Context | Network Init. PDP Context if available |  |
| Charging Id | Charging Id |  |
| Recording Entity Code | Numeric code | Referring to table in the Network Information. |
| Serving Network | TADIG code of the VPMN |  |
| IMEI | IMEI if available |  |
| IMS Signalling Context | IMS Signalling Context if available |  |
| Data Volume Incoming | Data Volume Incoming |  |
| Data Volume Outgoing | Data Volume Outgoing |  |
| Charged Item | Populate according to charging principle |  |
| Exchange Rate Code | Populate with 1 | Referring to the table in the Accounting Information |
| Call Type Level 1 | Populate with 10 for HGGSN/HP-GW | Assuming home routing. |
| Call Type Level 2 | Populate as per the rate, for example with 0 for unknown or 10-15 for QoS charging | 0 Unknown/Not Applicable 10 Broadband 11 Narrowband 12 Conversational 13 Streaming 14 Interactive 15 Background |
| Call Type Level 3 | Populate as needed to uniquely identify the rate |  |
| Charge Type | Populate with 00 |  |
| Charge | As per the rate | Excluding any taxes |
| Chargeable Units | Populate in accordance with the Charged Item. For Charged Item = X, populate with the total volume in bytes. |  |
| Tax Rate Code | Populate with the relevant code | Referring to the table in the Accounting Information. Only present if taxes apply. |
| Tax Value | The Tax Value | Only present if taxes apply. |
| CAMEL Service Key | CAMEL Service Key if available |  |
| Access Point Name NI | CAMEL Access Point Name NI if available |  |
| Access Point Name OI | CAMEL Access Point Name OI if available |  |
| Operator Specific Information | Populate with additional information as agreed | Free text field. |

### Mobile Session

| Data Item | How to populate | Comments |
| --- | --- | --- |
| Mobile Session Service | Mobile Session Service | 1 MO Voice over LTE 2 MT Voice over LTE 3 Emergency call over LTE |
| IMSI | IMSI | If MSISDN is not populated, then IMSI must be provided in full. If MSISDN is populated, then population of the IMSI is at the discretion of the DSP (as a minimum the MCC/MNC of the HPMN/DSP must be populated). |
| MSISDN | MSISDN if available |  |
| Public User Id | Public User Id if available |  |
| IMEI | IMEI if available |  |
| Serving Network | TADIG code of the VPMN | For anti-tromboning and callback, this would be the TADIG code of the HPMN. This could possibly be used as an indication that anti-tromboning or callback has taken place. |
| Event Reference | Event Reference |  |
| Recording Entity Code | Numeric code | Referring to table in the Network Information. Could be repeated. |
| Service Start Timestamp (including UTC Time Offset) | Service Start Timestamp (including UTC Time Offset) |  |
| Cause for Termination | Cause for Termination if available |  |
| Total Call Event Duration | Total Call Event Duration |  |
| Non-Charged Party Number | Non-Charged Party Number if available |  |
| Non-Charged Public User Id | Non-Charged Party Public User Id if available |  |
| Requested Number | Requested Number if available |  |
| Requested Public User Id | Requested Public User Id if available |  |
| Charged Item | Populate with A, D, and/or F depending on the charging principle | If fixed components are being charged (for example for unregulated services), then the whole Charge Information group can be repeated. |
| Exchange Rate Code | Populate with 1 | Referring to the table in the Accounting Information |
| Call Type Level 1 | Populate as per the rate, with 1 for national calls, and 2 for international calls | Note: The Call Type Levels are not copied from the incoming TAP file. Rather they must be in line with the DSP rate. |
| Call Type Level 2 | Populate as per the rate, for example with 0 for unknown | 0 Unknown/Not Applicable 1 Mobile 2 PSTN 3 Non Geographic 4 Premium Rate 5 Satellite destination |
| Call Type Level 3 | Populate as needed to uniquely identify the rate |  |
| Charge Type | Populate with 00 | Assume only total charge is needed. |
| Charge | As per the rate | Excluding any taxes |
| Chargeable Units | Populate in accordance with the Charged Item. For Charged Item = D, populate with the duration in seconds. |  |
| Tax Rate Code | Populate with the relevant code | Referring to the table in the Accounting Information. Only present if taxes apply. |
| Tax Value | The Tax Value | Only present if taxes apply. |
| Operator Specific Information | Populate with additional information as agreed (or at the discretion of the Sender) | Free text field. Could be used to identify callback and anti-tromboning by use of special keywords: Callback AntiTromboning |

### Messaging Event

| Data Item | How to populate | Comments |
| --- | --- | --- |
| Messaging Event Service | Messaging Event Service | 1 MO SMS over IP 2 MT SMS over IP |
| IMSI | IMSI | If MSISDN is not populated, then IMSI must be provided in full. If MSISDN is populated, then population of the IMSI is at the discretion of the DSP (as a minimum the MCC/MNC of the HPMN/DSP must be populated). |
| MSISDN | MSISDN if available |  |
| Public User Id | Public User Id if available |  |
| IMEI | IMEI if available |  |
| Serving Network | TADIG code of the VPMN | For anti-tromboning and callback, this would be the TADIG code of the HPMN. This could possibly be used as an indication that anti-tromboning or callback has taken place. |
| Event Reference | Event Reference |  |
| Element Type | Element Type if available | For example SMSC |
| Element ID | Element ID if available | For example SMSC Address |
| Recording Entity Code | Numeric code | Referring to table in the Network Information. Could be repeated. |
| Service Start Timestamp (including UTC Time Offset) | Service Start Timestamp (including UTC Time Offset) |  |
| Non-Charged Party Number | Non-Charged Party Number if available |  |
| Non-Charged Public User Id | Non-Charged Party Public User Id if available |  |
| Exchange Rate Code | Populate with 1 | Referring to the table in the Accounting Information |
| Call Type Level 1 | Populate as per the rate, with 1 for national calls, and 2 for international calls | Note: The Call Type Levels are not copied from the incoming TAP file. Rather they must be in line with the DSP rate. |
| Call Type Level 2 | Populate as per the rate, for example with 0 for unknown | 0 Unknown/Not Applicable 1 Mobile 2 PSTN 3 Non Geographic 4 Premium Rate 5 Satellite destination |
| Call Type Level 3 | Populate as needed to uniquely identify the rate |  |
| Charge | As per the rate | Excluding any taxes |
| Tax Rate Code | Populate with the relevant code | Referring to the table in the Accounting Information. Only present if taxes apply. |
| Tax Value | The Tax Value | Only present if taxes apply. |
| Operator Specific Information | Populate with additional information as agreed (or at the discretion of the Sender) | Free text field. |

# Physical Format

The physical format is standard TAP as defined in TD.57 [2].

# File Names

The file naming convention should follow the TAP standard:

CD[DSP TADIG code][ARP TADIG code][Sequence number

Example: CDLUXPTLUXAR00123

The ARP will need to use an in-country TADIG code, whether they have a registered in-country presence or not.  The GSMA issues TADIG codes, and there are plenty of codes available within each European country.  For the avoidance of doubt, if the ARP is also a MNO with international roaming agreements, it cannot reuse that TADIG code in their role as an ARP.

# Timescales

The exceptional timescale for the VPMN to make the TAP records available to the HPMN is 30 days from call end time.  As at least some of the information will be sourced from the incoming TAP files, the HPMN will need additional time to make the records available to the ARP (potentially via a MVNO).

Introduce a new exceptional timescale for the DSP to ARP interface, 40 days from call end time.  The additional 10 days is needed to cater for exceptional cases of operational problems, validation, re-rating, transfer via a MVNO, etc.  This is easy to manage, and will work for all call records regardless of source.

# File Exchange Rules

The TAP files should be exchanged once per day, or more regularly if agreed between the DSP and ARP.

Notification files are exchanged when there is no traffic recorded during the collection period.  As the DSP will include calls from all VPMNs in the same TAP file stream to the ARP, with the exception of the period between signing of the DSP/ARP contract and the first active roamer it is highly unlikely that there will be any notification files produced (as all it will take is one ARP customer roaming anywhere in Europe per day).

Physical transfer methods are discussed in TD.28 [1].

# TAP Versions

There are 8 different versions of TAP used today over the VPMN/HPMN interface.  The versions are generally not backwards compatible, as shown in the following table:

| Version | Backwards Compatible | Comments, including changes relevant to the DSP/ARP interface |
| --- | --- | --- |
| 3.1 | No | The first version of TAP3, and contains many errors. The GSMA has since 2001 strongly recommended that no-one uses 3.1. |
| 3.2 | No | Corrected many of the errors with 3.1. Introduced charging for USSD. |
| 3.3 | Yes, with 3.2 | Many validation rule changes due to operational experiences. |
| 3.4 | No | Support for CAMEL within GPRS call, addition of Serving Network (where not the same as the Sender), Change of Call Type Level structure, change of IPv6 length, repeating Operator Specific Information. |
| 3.9 | Yes, with 3.4 | Many validation rule changes due to operational experiences. |
| 3.10 | No | Support for Video Telephony, increased some Integer sizes that had caused overflows in previous versions, Called Number in international format, addition of Dialled Digits. |
| 3.11 | No | Major simplification, restructure and removal of unnecessary data items, support for satellite calls, addition of SMS Destination Number, support for data over LTE. |
| 3.12 | No, but see comment | Support for VoLTE, enable differential MTC charging, standardisation of charge validation procedure to resolve operational issues. There are two new record types to support VoLTE in 3.12. If those are not used, then 3.12 is backwards compatible with 3.11 in terms of the physical format. Logically, it is not always backwards compatible with 3.11, as that depends on how the fields in 3.12 are used. One feature that would make it not backwards compatible would be if the Call Type Levels are significant within a Mobile Terminated Call. |

TAP Versions

Only TAP3.11 and TAP3.12 are allowed, in line with expected future GSMA rules on TAP versions.  The version exchanged (TAP3.11 or TAP3.12) is at the discretion of the DSP.

Conversions can take place on the private interface.

- Document Management

  - Document History

| Version | Date | Brief Description of Change | Approval Authority | Editor / Company |
| --- | --- | --- | --- | --- |
| 0.1 | 19 Feb 2013 | First draft | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.2 | 26 Feb 2013 | Updated with comments received via e-mail | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.3 | 26 Feb 2013 | Updated as agreed during B&P #3 | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.4 | 4 Mar 2013 | Further editorial changes | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.5 | 12 Mar 2013 | Updated as agreed during B&P #4 | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.6 | 21 Mar 2013 | Added IMSI when MSISDN is not available. Added discussion on TAP versions. | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.7 | 27 Mar 2013 | Updated as agreed during B&P #6 | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.8 | 18 Apr 2013 | Added support for SMS and MMS retail charging. Updated abbreviations. | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.9 | 24 Apr 2013 | Selected MOC/MTC for MMS retail charging. Referred to TD.28 for physical transfer. Removed USSD to refer only to callback. Other editorial changes. | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.10 | 17 May 2013 | Clarifications to sections 2 and 7, after comments received from DTAG. | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.11 | 22 May 2013 | Approved changes as agreed by B&P #10. Added general statement on availability of data from network. Added clarifications on MMS. Open issue: Is support for MMS event based retail charging needed? | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.12 | 28 June 2013 | Removed support for MMS event based retail charging as it is not needed. | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.13 | 28 June 2013 | Made SMS Destination Number optional. | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.14 | 4 July 2013 | Changes after TADIG TDS review. Added comment that the Recording Entity can be an IP address. Added two new fields for Mobile Session that will be added to TAP3.12. Some editorial changes. | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.15 | 8 July 2013 | Added explanation for VoLTE changes in 3GPP R11 and TAP3.12 emergency release | B&P | Christer Gullstrand / Syniverse Technologies |
| 1.0 | 12 July 2013 | Approved by B&P #16 | B&P | Christer Gullstrand / Syniverse Technologies |
|  | 15 July 2013 | Editorial change to remove BA.12 as it was not referenced in the document | B&P | Christer Gullstrand / Syniverse Technologies |