# ABF format specification for the Single IMSI Wholesale Billing Interface

# Version 1.0

# 15 July 2013

# Introduction

## Overview

This document defines the logical and physical data that must be transferred between DSP and ARP when used CSV data format.  Format name is ABF (Alternative Billing Format). It additionally contains all the validation rules that can be applied.

## Scope

This Document defines, in full, the format and validation of CSV files that can be used between DSP and ARP.

## Definitions

| Term | Description |
| --- | --- |
| Calls | All call events defined for the applicable format (for example TAP, NRTRDE) |

## Abbreviations

| Term | Description |
| --- | --- |
| ABF | Alternative Billing Format |
| APN | Access Point Name |
| ARP | Alternative Roaming Provider |
| CR | Carriage Return |
| CSV | Comma Separated Values |
| DSP | Domestic Service Provider |
| GPRS | General Packet Radio Service |
| HPMN | Home Public Mobile Network |
| LF | Line Feed |
| MMS | Multimedia Messaging Service |
| MOC | Mobile Originated Call |
| MTC | Mobile Terminated Call |
| NRTRDE | Near Real Time Roaming Data Exchange |
| PRD | Permanent Reference Document |
| RAP | Returned Account Procedure |
| Sender | Sender of the data |
| SMS | Short Message Service |
| TAP | Transferred Account Procedure |
| USSD | Unstructured Supplementary Service Data |
| UTC | Universal Time Co-ordinated |
| VPMN | Visited Public Mobile Network |

## References

| Ref | Doc Number | Title |
| --- | --- | --- |
|  | CSV-1203 | “CSV File Format Specification”. Available at http://mastpoint.curzonnassau.com/csv-1203/ |
|  | GSMA PRD TD.13 | TADIG Code Naming Conventions |
|  | GSMA PRD TD.28 | File Transfer Methods |
|  | RFC 4180 | “Common Format and MIME Type for Comma-Separated Values Files”, Y. Shafranovich, October 2005. Available at http://tools.ietf.org/html/rfc4180 |
|  | RFC 2119 | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997. Available at http://www.ietf.org/rfc/rfc2119.txt |

## Conventions

The key words “must”, “must not”, “required”, “shall”, “shall not”, “should”, “should not”, “recommended”, “may”, and “optional” in this document are to be interpreted as described in RFC 2119 [5].

# Deviations from the document “WS Billing Required Data Items” requirements

Added SS event record type.

No separate record types for “Batch Control” and “Audit Control” were created. Information from “Batch Control” and “Audit Control” was moved to file name. This was done for following reasons:

- CSV format requires that all records have the same number of fields throughout the file.

- Standard utilities importing CSV files to databases require that the same column fields to have the same data type (string, number, timestamp) throughout the file.

- Standard utilities exporting database table to CSV file will produce records with the same column fields number and type for all CSV file.

- Putting header and trailer information to this field structure is complicated task.

- Having only call records in CSV file body will allow to create or read CSV files from/to database with one line of code using standard utilities. Having header and trailer in CSV file will require to program custom field by field parsing.

- Having header and trailer will force us to handle additional error situations.

- Header in CSV file usually means first line of the file containing list of fields’ names. It could be confusing.

- Call-back should be presented by two records MTC and MOC.

- MMS is charged as data within the GPRS record.

# CSV Format Specification

## Main Principles of Development

Main principles of the CSV format development are:

- Format should be as simple as possible.

- Format should give possibility to transmit calls data between DSP and ARP databases by most simple and efficient way. In simple case existing database utilities could be used to generate and import CSV files without any programming.

- Current TAP specification will be used as the base for the first version of CSV format and TAP data dictionary will be imported with small changes.

## Physical Specifications

CSV is ideal format for exchange structured homogeneous tabular data. In order to ensure compatibility with most CSV file processing systems - following rules should be strictly executed. The rules are based on RFC4180 [4] and CSV-1203 [1].

- File extension is .csv.

- US-ASCII character set must be used.

- Records must be ended with CR+LF or LF.

- Each record within CSV file must contain the same number of field columns.

- Field separator is comma.

- A field column must not have leading or trailing whitespace.

- The header record must not be used.

- Trailer records must not be used.

- If field’s payload contains one or more commas or double-quotes, then it must be delimited by pair of double-quotes.

- If a field’s payload contains a double-quote character, it must be represented by two consecutive double-quotes.

- Field’s payload must not contain characters CR and LF, even delimited by pair of double-quotes.

- Double-quote protection should not be applied where it is not required.

- Decimal symbol for numeric data is decimal point. Example correct: 3.14159,  wrong: 3,14159.

- Digit grouping symbols must not be used. Example correct: 1234567.89, wrong: 1,234,567.89.

- Leading zeros should be displayed. Example correct: 0.7, wrong: .7.

- Timestamp format for usage in file body is YYYY-MM-DDThh:mm:ss±hhmm. Where time range: 00:00:00 to 23:59:59. UTC offset range: -13:00 to +14:00.

- Timestamp format for usage in file name is YYYYMMDDhhmmss±hhmm. Different format used because symbol ‘:’ is reserved for special use in some Operating systems. Where time range: 000000 to 235959. UTC offset range: -1300 to +1400.

- Charge and tax values must contain no more than 6 decimal places. Example correct: 9.123456, wrong: 9.1234567.

## Physical transfer methods

Physical transfer methods are discussed in TD.28 [3]. It is recommended to send ABF files using FTP protocol over secure VPN tunnel. It is recommended that FTP transfer type should be set to binary.

## CSV data fields per record type

This section lists the CSV data fields per record type.

| No | Field Name | MOC | MTC | GPRS | SS |
| --- | --- | --- | --- | --- | --- |
| 1 | Call type | O | I | G | S |
| 2 | Serving Network | TADIG Code |  |  |  |
| 3 | Source File Identification | Source File Identification |  |  |  |
| 4 | Subscriber Identification Type | I – IMSI, M – MSISDN, P – Public User ID |  |  |  |
| 5 | Subscriber Identification | IMSI or MSISDN or Public User ID |  |  |  |
| 6 | Number or APN | Called Number | Calling Number | APN NI |  |
| 7 | Dialled Digits or APN | Dialled Digits |  | APN OI |  |
| 8 | Call Time (including UTC offset) | Call Event Start Timestamp |  |  |  |
| 9 | Total Call Event Duration | Total Call Event Duration |  |  |  |
| 10 | Partial Type Indicator |  |  | Partial Type Indicator |  |
| 11 | PDP Context Start Timestamp |  |  | PDP Context Start Timestamp |  |
| 12 | Data Volume Incoming |  |  | Data Volume Incoming |  |
| 13 | Data Volume Outgoing |  |  | Data Volume Outgoing |  |
| 14 | Basic Service Code (TS/BS) | Basic Service Code | Basic Service Code | Basic Service Code | Basic Service Code |
| 15 | Supplementary Service Code | Suppl. Service Code |  |  | Suppl. Service Code |
| 16 | Cause for Termination | Cause for Termination |  |  |  |
| 17 | Charge (excluding tax) | Charge |  |  |  |
| 18 | Tax Value | Tax Value |  |  |  |
| 19 | Call Reference | Call Reference | Call Reference | Charging Id | Call Reference |
| 20 | CAMEL Service Key | CAMEL Service Key |  |  |  |
| 21 | CAMEL Destination Number or APN | CAMEL Destination Number |  | APN NI (CAMEL) |  |
| 22 | CAMEL APN OI |  |  | APN OI (CAMEL) |  |
| 23 | Operator Specific Information | Operator Specific Information |  |  |  |

Notes:

Fields order selected to ease file reading by human.

Messaging Event and Mobile Session could be identified using values at “Basic Service Code” field – ME1, ME2, MS1, MS2 and MS3.

- LTE usage could be identified using value LTE at GPRS record “Basic Service Code” field.

## Version Control Principles

Format specification updates should be done by adding new fields at the end of the records.

Operators, who are not compliant with the new version, will ignore extra fields at the end of the records.

Parties who want to implement private format extension on bilateral level advised to use Operator Specific Information field. Information could be passed as list of parameters pn=pv separated by blanks. Where pn - parameter name, pv – parameter value. Example of Operator Specific Information field population – “Seq=00123 Rec=32”.

## File Naming Convention

CSV files must follow the following file naming convention:

CD_xxxxx_yyyyy_seq_transfer_available _vers_currency_charge_tax_ncalls.csv

where

CD or TD		CD identifies the file as containing chargeable data, TD – identifies the file as containing test data

xxxxx		contains the Sender TADIG Code in uppercase (5 positions)

yyyyy		contains the Recipient TADIG Code in uppercase (5 positions)

seq			contains the Sequence Number (5 positions)

transfer		contains Transfer Cut Off Timestamp in format YYYYMMDDhhmmss±hhmm

available		contains File Available Timestamp in format YYYYMMDDhhmmss±hhmm

vers		contains the Specification Version Number

currency		contains the Local Currency (ISO 4217)

charge		contains the Total Charge

tax			contains the Total Tax Value

ncalls		contains the Call Event Details Count

.csv		file extension

Example: CD_LVALM_ARP01_00001_20130321112000+0300_20130321112000+0300_1_EUR_3.338_0_7.csv

# Data Dictionary

The data dictionary gives a full listing of all data items within the Alternative Billing Format (ABF).

In addition to providing descriptions, derivations and values where appropriate for all groups and items, it describes conditionality and validation in detail.

## File Name Elements

| Element | Description | Error Code | Context | Severity Level | Validation Description |
| --- | --- | --- | --- | --- | --- |
| Call Events Count | This item contains a count of the number of call events in the file. If Call Events Count is equal to 0 than it is a Notification file. A Notification file is sent where the transfer mechanism is electronic file transfer and there is no data available for transfer. Conditionality: Mandatory. Values: >= 0 | CNT1 CNT2 CNT3 CNT5 | File Name File Name File Name File Name | Fatal Fatal Fatal Fatal | Syntax error. Value out of range. Missing. Value does not match the count of Call Events. Note that any rejected Call Events must be included for the purposes of this validation. |
| File Available Timestamp | The date and time at which the file was made available to the Recipient. Note: Where the Sender uses an agent for file transfer, this is the date and time the agent makes the file available to the Recipient or the Recipient’s agent. Physically this will normally be the timestamp when the file transfer commenced to the Recipient, i.e. start of push, however on some systems this will be the timestamp when the file was made available to be pulled. The time is given in the local time of the Sender PMN. There must be a UTC Time Offset associated with the item. Note: Where the Sender uses an agent for file transfer, the Timestamp and the UTC Time Offset could be that of the agent. Note that where a file is being resubmitted, for example where the file contained a fatal error on earlier submission, this timestamp must not contain the original timestamp, i.e. the item contains the timestamp when the file was made available on resubmission. Conditionality: Mandatory Format: YYYYMMDDhhmmss±hhmm | AVL1 AVL3 AVL5 | File Name File Name File Name | Fatal Fatal Fatal | Syntax error. Missing. Timestamp is more than one hour after the file received time at the Recipient. Note: The tolerance of one hour has been introduced as it cannot be guaranteed that Sender and Recipient system clocks are synchronised, in particular around daylight saving (wintertime/summertime) changes. |
| File Sequence Number | A unique reference which identifies each Data Interchange sent by one PMN to another, specific, PMN. The sequence commences at 1 and is incremented by one for each subsequent Data Interchange sent by the Sender PMN to a particular Recipient PMN. Separate sequence numbering must be used for Test Data and Chargeable Data. Having reached the maximum value (99999) the number must recycle to 1. Note that in the case of retransmission for any reason this number is not incremented. Note that where the file sequence number is being reused with a file already received and successfully processed and the content of the two files is different, then the received file is to be considered as ‘invalid reused sequence number’ and error message should be generated. Where the content of the two files is the same then the file is to be considered as a ‘copy’ of the original file and should be ignored (i.e. not rejected). Conditionality: Mandatory. Range: 00001 – 99999 for Test Data 00001 – 99999 for Chargeable Data | SEQ1 SEQ2 SEQ3 SEQ5 | File Name File Name File Name File Name | Fatal Fatal Fatal Fatal | Syntax error. Value out of range. Missing. File sequence number of the received file has already been received and successfully processed and the file is not a copy of the original file. Note: This error can also be raised if the original file details are not available (for example DCH migration scenario). |
| Local Currency | Contains the Currency Code which identifies the currency used to price the calls and/or events. This currency has to be the one used in the Sender’s IOT. Local Currency may be or may be not the same as Sender’s country local currency. Derivation: ISO 4217 Currency Codes standard. Conditionality: Mandatory. Example: Currency Code; Currency Name. AUD; Australian Dollars. CLP; Chilean Pesetas. GBP; Pounds Sterling. | LCR3 LCR4 | File Name File Name | Fatal Fatal | Missing. Value not as specified in the Sender’s Inter Operator Tariff (IOT). |
| Recipient | A TADIG code uniquely identifies file recipient. Derivation: GSM Association PRD TD.13 [2]. Conditionality: Mandatory. Examples: GBRCN GBRVF DEUD1 DEUD2 Note: These codes must be in uppercase (as defined in TD.13 [2]). | RCP2 RCP3 | File Name File Name | Fatal Fatal | Value out of range. Missing. |
| Sender | A TADIG code uniquely identifies file sender. The full list of codes in use is given in TADIG PRD TD.13 [2]. Conditionality: Mandatory. Example content: GBRCN GBRVF DEUD1 DEUD2 Note: These codes must be in uppercase (as defined in TD.13 [2]). | SND2 SND3 | File Name File Name | Fatal Fatal | Value out of range. Missing |
| Specification Version Number | To enable a PMN to encode and/or read a file it is necessary to uniquely identify the format. This is achieved through the Specification Version Number. Conditionality: Mandatory. Value: 1 | VER1 VER2 VER3 | File Name File Name File Name | Fatal Fatal Fatal | Syntax error. Value out of range. Missing. |
| Total Charge | The sum of all the charges. The Total Charge must not contain a negative value. Note that charges from records with severe errors must also be included where possible. Conditionality: Mandatory. Values: > or = 0 (zero) | TCH1 TCH2 TCH3 TCH5 | File Name File Name File Name File Name | Fatal Fatal Fatal Fatal | Syntax error. Value out of range. Missing. Value does not match the calculated sum of charges from file records. |
| Total Tax Value | The sum of the Tax Values contained in the batch. The Total Tax Value must not contain a negative value. This is present for audit purposes only. Note that taxes from records with severe errors should also be included where possible. Conditionality: Mandatory. Values: > or = 0 (zero) | TTX1 TTX2 TTX3 TTX5 | File Name File Name File Name File Name | Fatal Fatal Fatal Fatal | Syntax error. Value out of range. Missing. Value does not match the calculated sum of taxes from file records. |
| Transfer Cut Off Timestamp | The timestamp used to select calls for transfer. All call records available prior to the timestamp are transferred. This gives an indication to the recipient as to how ‘up-to-date’ the information is. Where the timestamp is present within empty file it is nominal showing that no data available for transfer are being held by the Sender PMN with a Charging Timestamp prior to this. The time is given in the local time of the Sender PMN. There must be a UTC Time Offset associated with the item. Conditionality: Mandatory. Format: YYYYMMDDhhmmss±hhmm | TCO1 TCO3 | File Name File Name | Fatal Fatal | Syntax error. Missing. |

## File Content Elements

| Element | Description | Error Code | Context | Severity Level | Validation Description |
| --- | --- | --- | --- | --- | --- |
| Access Point Name NI | The Network Identifier part of the Access Point Name (APN) in dot notation. Derivation: In the case of GPRS usage without CAMEL invocation: The actual connected APN NI as defined in PS Domain Charging – see Annex C. Pre GSM Release ’98 from the GSM item AccessPointName (see conditionality note below). In case of GPRS usage with CAMEL invocation: The actual connected APN NI as modified by the CSE in APN NI field as defined in PS Domain Charging – see Annex C. In the case of Wi-Fi usage this element must contain the Venue Class where available, otherwise defaults to 0 (zero). Conditionality: Mandatory within GPRS record. Networks which have not yet upgraded to at least GSM Release ’98 do not yet support the separation of the APN into the Network Identifier and Operator Identifier parts and, therefore, must populate a concatenation of both parts within this item. Values: This field is encoded as a string comprising of up to 63 characters. The actual contents are for further study. | ANI1 ANI3 ANI5 | GPRS GPRS GPRS | Severe Severe Severe | Syntax error. Missing. APN is EUInternet and session is not home routed. Note: This error cannot be raised when Access Point Name NI (CAMEL) is present. |
| Access Point Name NI (CAMEL) | The Network Identifier part of the Access Point Name (APN) in dot notation. Populated in case of GPRS usage with CAMEL invocation. Derivation: In case of GPRS usage with CAMEL invocation: The original APN NI before modification by the CSE in CAMEL APN NI field available within CAMEL Information as defined in PS Domain Charging – see Annex C. In the case of Wi-Fi usage this element must contain the Venue Class where available, otherwise defaults to 0 (zero). Conditionality: Mandatory within GPRS record. Networks which have not yet upgraded to at least GSM Release ’98 do not yet support the separation of the APN into the Network Identifier and Operator Identifier parts and, therefore, must populate a concatenation of both parts within this item. Values: This field is encoded as a string comprising of up to 63 characters. The actual contents are for further study. | ANC1 ANC5 | GPRS GPRS | Severe Severe | Syntax error. APN is EUInternet and session is not home routed. |
| Access Point Name OI | The Operator Identifier part of the Access Point Name (APN) in dot notation. Derivation: In the case of GPRS usage without CAMEL invocation: The actual connected APN OI as defined in PS Domain Charging – see Annex C. Pre GSM Release ’98 from the GSM item AccessPointName (see conditionality note below). From GSM Release ’98 to 3GPP Release 7 from the GSM item AccessPointNameOI (see conditionality note below). From 3GPP Release 8 from the GSM item P-GW PLMN Identifier (see conditionality note below). In the case of GPRS usage with CAMEL invocation: The actual connected APN OI as modified by CSE in APN OI field as defined in PS Domain Charging – see Annex C. Conditionality: Mandatory where the GPRS Call represents an SGSN or S-GW ticket or an assembled SGSN/GGSN or S-GW/P-GW ticket. This item is only available within networks which have upgraded to at least GSM Release ’98 and not available for Wi-Fi usage. Values: This field is encoded as a string comprising of up to 37 characters. The actual contents are for further study. | AOI1 | GPRS | Severe | Syntax error. . |
| Access Point Name OI (CAMEL) | The Operator Identifier part of the Access Point Name (APN) in dot notation. Populated in the case of GPRS usage with CAMEL invocation. Derivation: The original APN OI field available within CAMEL Information as defined in PS Domain Charging – see Annex C. Conditionality: This item is only available within networks which have upgraded to at least GSM Release ’98 and not available for Wi-Fi usage. Values: This field is encoded as a string comprising of up to 37 characters. The actual contents are for further study. |  |  |  |  |
| Basic Service Code | Basic Service Code uniquely defines TeleService, Bearer Service, Messaging Event Service, Mobile Session Service and LTE usage at GPRS sessions. TeleService and Bearer Service Codes, identifiable by the description ‘All’, should only be used where details of the specific services affected are not available. Note that compound TeleService codes can only occur in call independent supplementary service operations. Note that compound Bearer Service codes can only occur in conjunction with HSCSD usage or call independent supplementary service operations. Note that codes 27 & 37 (General Data A/Synchronous Service) are used where a data rate greater than 9600bps is used, the data rate used is not identifiable/specified on the network CDRs. Derivation: GSM items TeleServiceCode and BearerServiceCode (See Annex C – CS Domain Charging). TeleService code is derived by taking the MAP encoding (See Annex C – Teleservice Codes) and translating bits 8-5 into a ‘hex’ character and bits 4-1 into a second ‘hex’ character. Derivation: BearerService code is derived by taking the MAP encoding as specified for BearerServiceCode (See Annex C – CS Domain Charging) and translating bits 7-4 into a ‘hex’ character and bits 3-1 into a second ‘hex’ character. Conditionality: Must be present where the type of service used was a teleservice or bearer service. Tele Service Values (prefix 0 added to identify Tele Service): 000 All teleservices 010 All Speech transmission services 011 Telephony 012 Emergency calls 020 All SMS Services 021 Short Message MT/PP 022 Short Message MO/PP 060 All Fax Services 061 Facsimile Group 3 & alternative speech 062 Automatic Facsimile Group 3 063 Automatic Facsimile Group 4 070 All data teleservices (compound) 080 All teleservices except SMS (compound) 090 All voice group call services 091 Voice group call 092 Voice broadcast call Bearer Service Values (prefix 1 added to identify Bearer Service): 100 All Bearer Services 120 All Data Circuit Asynchronous Services 121 Duplex Asynch. 300bps data circuit 122 Duplex Asynch. 1200bps data circuit 123 Duplex Asynch. 1200/75bps data circuit 124 Duplex Asynch. 2400bps data circuit 125 Duplex Asynch. 4800bps data circuit 126 Duplex Asynch. 9600bps data circuit 127 General Data Circuit Asynchronous Service 130 All Data Circuit Synchronous Services 132 Duplex Synch. 1200bps data circuit 134 Duplex Synch. 2400bps data circuit 135 Duplex Synch. 4800bps data circuit 136 Duplex Synch. 9600bps data circuit 137 General Data Circuit Synchronous Service 140 All Dedicated PAD Access Services 141 Duplex Asynch. 300bps PAD access 142 Duplex Asynch. 1200bps PAD access 143 Duplex Asynch. 1200/75bps PAD access 144 Duplex Asynch. 2400bps PAD access 145 Duplex Asynch. 4800bps PAD access 146 Duplex Asynch. 9600bps PAD access 147 General PAD Access Service 150 All Dedicated Packet Access Services 154 Duplex Synch. 2400bps PAD access 155 Duplex Synch. 4800bps PAD access 156 Duplex Synch. 9600bps PAD access 157 General Packet Access Service 160 All Alternat Speech/Asynchronous Services 170 All Alternate Speech/Synchronous Services 180 All Speech followed by Data Asynchronous Services 190 All Speech followed by Data Synchronous Services 1A0 All Data Circuit Asynchronous Services (compound) 1B0 All Data Circuit Synchronous Services (compound) 1C0 All Asynchronous Services (compound) 1D0 All Synchronous Services (compound) Messaging Event Service Values: ME1 MO SMS over IP (IMS based SMS) ME2 MT SMS over IP (IMS based SMS) Mobile Session Service Values: MS1 MO Voice over LTE MS2 MT Voice over LTE MS3 Emergency call over LTE LTE usage identification: - LTE GPRS session over LTE | BSV2 BSV3 | MOC MTC GPRS MOC MTC | Severe Severe | Value out of range. Note: Values 021 and MS2 are only allowed for MTC. Values 022 and MS1 are only allowed for MOC. Missing. |
| Call Event Start Timestamp | The timestamp gives the start of the call event. The time is given in the local time of the Sender PMN (or Serving Network where this is not the Sender). There must be a UTC Time Offset associated with the timestamp. Note that local time is the local time at the location of the chargeable subscriber. Where the location is not available, as in some call forwarding scenarios, this will be a notional ‘network local time’. Note that this timestamp is the event start time as provided by the network. This will be either the call answer time or the channel seizure time. If charging is from call answer time then this item can be populated with either call answer time of channel seizure time. If charging is from channel seizure time then Call Event Start Timestamp must be populated with channel seizure time. In the case of an intermediate or last partial record of the same GPRS Call (GPRS Context) the timestamp must be populated with the record start time and not the context start timestamp (see item PDP Context Start Timestamp for identification of GPRS context start time in each partial record). Derivation: GSM item answerTime or seizureTime (See Annex C – CS Domain Charging and PS Domain Charging). This will normally be the equivalent of the start of charging or, where the call event is non chargeable, the equivalent point. IMS item Service Delivery Start Time Stamp (See Annex C – IMS Domain Charging For Wi-Fi usage item event-timestamp (RFC 2869) in UTC time. Conditionality: Mandatory within MOC MTC GPRS SS Example: > PDP Context activated at 2009-07-08T23:50:00+00:00 > Two partials raised on TAP (cut across midnight) First partial: Call Event Start Timestamp = PDP Context Start Timestamp = 2009-07-08T23:50:00 Second partial: Call Event Start Timestamp = 2009-07-09T00:00:00+00:00 PDP Context Start Timestamp = 2009-07-08T23:50:00+00:00 | TIM1 TIM3 TIM5 | Calls Calls Calls | Severe Severe Severe | Syntax error. Missing Call older than 40 calendar days or than allowed by bilateral agreement. Call age is calculated as difference between the call completion time or, where the call/event has no duration, the timestamp associated with the call/event and the File Available Timestamp normalized to use the same time basis (for example UTC). |
| Call Reference | The identifier distinguishes between transactions on the same mobile station. For SMS and Voice over LTE - Event Reference to be used. Derivation: GSM item CallReference (See Annex C – CS Domain Charging). GSM item Message Reference (see Annex C – CS Domian Charging) for SMS over circuit switched. GSM item Message Reference (See Annex C – PS Domain Charging) for SMS over GPRS. IMS item IMS Charging Identifier (See Annex C - IMS Charging Domain) for SMS and Voice over LTE. Conditionality: Must be present when available. | REF1 | MOC MTC SS | Severe | Syntax error. |
| Call Type | The type of record. Conditionality: Mandatory Values: O MOC record I MTC record G GPRS record S SS record Validation rules for duplicated records: MOC Subscriber Identification, Call Event Start Time, Called Number, Service Code and Total Call Duration match call record already processed and the call reference is identical in both call records. If Called Number is not present Dialled Digits must be used. MTC Subscriber Identification, Call Event Start Time, Calling Number, Service Code and Total Call Duration match call record already processed and the Call Reference is identical in both call records. GPRS Subscriber Identification, Charging Id and Call Event Start Time match call record already processed. SS Subscriber Identification, Charging Timestamp, Supplementary Service Code and Action match call record already processed and the Call Reference is identical in both call records. | CTP2 CTP3 CTP4 CTP5 | Calls Calls Calls Calls | Severe Severe Severe Severe | Value out of range. Missing. Call type, call destination or call service is not as agreed. Duplicated record |
| Called Number | The called number is the international representation of the destination and contains different information depending on the call scenario: Circuit Switched MO Call initiated by the roamer: The international representation of the number dialled by the roamer in establishing the call Circuit Switched MO Call initiated as a result of an incoming call and invocation of conditional call forwarding: The international representation of the number to which the call is forwarded SMS MO: The international representation of the SMSC address used The called number must always start with the valid country code as listed in E.164 “Assigned Country Codes” (spare and reserved codes are not valid, except the reserved code 970 for Palestine which is valid), and contain only numeric digits. All other characters must be removed. In the following cases it is acceptable for the Called Number (if present) to contain only the country code: Short code call: Only country code, or country code followed by short code Emergency call: Only country code, or country code followed by the emergency service access code Unsuccessful call attempt: Only country code, or full number starting with the country code Specially routed call (for example using ‘*’ or ‘#’): Only country code In all other cases the Called Number must contain the full long number as defined by E.164 “International public telecommunication number structure. Derivation: Derived from GSM item CalledNumber (See Annex C – CS Domain Charging) but it is represented in TAP in International format, i.e. the number begins with the Country Code. Conditionality: Must be present except for one of the following: use of the basic service emergency call unsuccessful call attempt when the call destination has been modified by CAMEL (CAMEL Destination Number is present) If Called Number is not present then, in case of unsuccessful call attempt or CAMEL destination modification, item Dialled Digits must be filled in. In the case of SMS-MO usage this field must always be present. Values: Each digit is numeric (0-9) | CDN1 CDN2 CDN3 | MOC MOC MOC | Severe Severe Severe | Syntax error and call does not represent an emergency call (Basic Service code is 012 or MS3). Value out of range or number does not start with a valid country code as listed in E.164 “Assigned Country Codes”, and either CAMEL Destination Number or Dialled Digits is not present. Note: Spare and reserved codes are NOT valid country codes, with the exception of reserved code 970 for Palestine which is valid. Missing and call does not represent an emergency call (Basic Service code is 012 or MS3). |
| Calling Number | The calling number is the number from which the call was originated in the case of mobile terminated calls. For SMS MT this item must contain the SMSC MSISDN. It is represented in international format, i.e. the number begins with the country code. Derivation: GSM item CallingNumber (See Annex C – CS Domain Charging). Conditionality: Must be present where available. Values: Each digit is numeric (0 – 9) |  |  |  |  |
| CAMEL Destination Number | The destination number returned by the CAMEL server to the interrogating (visited) MSC. Derivation: Derived from GSM item CalledNumber (See Annex C – CS Domain Charging and (for SMS over PS only) PS Domain Charging) but it is represented in International format, i.e. the number must start with a valid Country Code for the routed destination. Conditionality: Mandatory where Called Number not present. Values: Each digit is numeric (0-9) or contains ‘A’ (representing *) or contains ‘B’ (representing #) or contains ‘C’ (representing a) or contains ‘D’ (representing b) or contains ‘E’ (representing c) | CDN1 CDN2 CDN5 | MOC MOC MOC | Severe Severe Severe | Syntax error. Value out of range or number does not start with a valid country code as listed in E.164 “Assigned Country Codes”. Note: Spare and reserved codes are NOT valid country codes with the exception of reserved code 970 for Palestine which is valid. CAMEL Destination Number different from what the Home CAMEL Server returned. Note: The Sender may need to insert a country code in front of what the Home CAMEL Server returned. |
| CAMEL Service Key | The identification of the CAMEL service logic to be applied to the IN subscriber. Derivation: GSM item ServiceKey (See Annex C – CS Domain Charging and PS Domain Charging). Conditionality: Mandatory when CAMEL service used. Range: 0 – 2147483647 (derived from 0 to 2^31) |  |  |  |  |
| Cause For Termination | Indicates the reason for termination of the call where that reason is anything other than normal. Derivation: GSM items CauseForTerm and CauseForRecClosing(See Annex C – CS Domain Charging and PS Domain Charging). For Wi-Fi usage mapped from acct-terminate-cause (RFC 2866). For Voice over LTE network usage, IMS item CauseForRecordClosing (see Annex C - IMS Domain Charging) Conditionality: Must be present within MOC, MTC and GPRS where the call release is abnormal, see values below. Where the cause for termination is normal release, i.e. value 0, then the item is not present. Values: 1 Unsuccessful Service Delivery 3 Unsuccessful call attempt 4 Stable call abnormal termination 5 CAMEL initiated call release/Management intervention 20 Management intervention 21 Intra SGSN intersystem change 24 SGSN PLMNIDS change Values 4, 5 (CAMEL initiated call release), 20, 21 and 24 are valid within a GPRS Call context only where the call did not terminate normally. Where partial GPRS Calls are generated then only the last partial can contain a cause for termination where appropriate. Values 3, 4 and 5 (CAMEL initiated call release) are valid within a MOC/MTC only where the call did not terminate normally. Values 4 and 20 are valid within a Wi-Fi context only where the call did not terminate normally. Where partial GPRS Calls for Wi-Fi usage are generated then only the last partial can contain a cause for termination where appropriate. Values 1 and 5 (Management intervention) are valid for Voice over LTE only where the session did not terminate normally. Note: Only the values as defined above are valid in TAP. There are other valid 3GPP values (listed in Annex C), however, they are not valid in TAP. Following is a mapping between the RFC2866 Acct-Termination-Cause values and Cause for Termination values: Cause For Termination Values; RFC2866 Values; RFC2866 Short Description. <not present>; 1; User Request. 4; 2; Lost Carrier. 4; 3; Lost Service. 4; 4; Idle Timeout. 4; 5; Session Timeout. 20; 6; Admin Reset. 20; 7; Admin Reboot. 4; 8; Port Error. 4; 9; NAS Error. 4; 10; NAS Request. 4; 11; NAS Reboot. 4; 12; Port Unneeded. 4; 13; Port Preempted. 4; 14; Port Suspended. 4; 15; Service Unavailable. 4; 16; Callback. 4; 17; User Error. <not present>; 18; Host Request. | CFT1 CFT2 | MOC MTC GPRS MOC MTC GPRS | Severe Severe | Syntax error. Value out of range. |
| Charge | The charge in local currency before any tax is added (if applicable). The Charge must not contain a negative value. Charge calculation and validation principles: Charge calculation has to be precise. Charge calculation must be done with maximum computer capability. Intermediate rounding is not allowed. Only final charge could be rounded. It is recommended to round up to 6 fractional digits. Charge tolerance must be not more than half of smallest fractional position provided in the charge. Examples: Charge: 1.2345 Allowed tolerance: ±0.00005 Charge: 1.23456 Allowed tolerance: ±0.000005 If in case on national regulation or technical inability Sender cannot fulfil these charge calculation rules, Sender must inform Recipient about actual charge calculation procedure to ensure possibility validate charge with tolerance allowed above. Conditionality: Mandatory Values: > or = 0 (zero) | CHG1 CHG2 CHG3 CHG4 | Calls Calls Calls Calls | Severe Severe Severe Severe | Syntax error. Value out of range. Missing Charge not in line with agreement. |
| Charging ID | A charging identifier which can be used to identify all records produced in SSGN(s) and GGSN or in S-GW(s) and P-GW involved in a single PDP context. For Wi-Fi usage this item is unique as a Wi-Fi record always represents a complete Wi-Fi session. Derivation: GSM item ChargingID (PS Domain Charging – see Annex C). For Wi-Fi the derivation is at the discretion of the Sender. Note that Charging Id must remain unique during a significant amount of time. Conditionality: Mandatory Range: 0 – 4294967295 (derived from 0 to 2^32-1) | CID1 CID2 CID3 | GPRS GPRS GPRS | Severe Severe Severe | Syntax error. Value out of range. Missing |
| Data Volume Incoming | The Data Volume Incoming identifies the number of incoming octets (bytes). Derivation: GSM item dataVolumeGPRSDownlink (PS Domain Charging – see Annex C). For Wi-Fi usage items Acct-Input-Octets (RFC 2866) and Acct-Input-Gigawords (RFC 2869) converted into octets. Conditionality: Mandatory. Values: > or = 0 (zero) | DVI1 DVI2 DVI3 | GPRS GPRS GPRS | Severe Severe Severe | Syntax error. Value out of range. Missing. |
| Data Volume Outgoing | The Data VolumeOutgoing identifies the number of outgoing octets (bytes). Derivation: GSM item dataVolumeGPRSUplink (PS Domain Charging – see Annex C). For Wi-Fi usage items Acct-Output-Octets (RFC 2866) and Acct-Output-Gigawords (RFC 2869) converted into octets. Conditionality: Mandatory. Values: > or = 0 (zero) | DVO1 DVO2 DVOI3 | GPRS GPRS GPRS | Severe Severe Severe | Syntax error. Value out of range. Missing. |
| Dialled Digits | The Dialled Digits item contains the actual digits as dialled by the subscriber, i.e. unmodified, in establishing a call. This will contain ‘+’ and ‘#’ where appropriate. The Dialled Digits item should contain everything the subscriber has entered before the send button is pushed (for example, pre-entered menu selections for voicemail). For emergency calls operators may optionally insert the digits ‘112’ or their national emergency number into the Dialled Digits. Conditionality: Must be present when available. If Called Number is not present then, in the case of either unsuccessful call attempt or CAMEL call with presence of CAMEL Destination Number, Dialled Digits must be present. If Dialled Digits is not available then Called Number must be present. Values: Each digit is numeric (0-9) or contains ‘+’, ‘*’ or ‘#’ Note that the network may translate a dialled * to A and a dialled # to B. Where this is done then TAP must contain the original values of * or # as actually dialled by the subscriber. | DIA1 | MOC | Severe | Syntax error. |
| Operator Specific Information | This is an optional item which will contain information which has been bilaterally agreed. The content of the item is defined by bilateral agreement and may vary according to the context. Optionality: On discretion of sender. |  |  |  |  |
| Partial Type Indicator | The Partial Type Indicator identifies the first and last partials in a sequence. Conditionality: Must be present within GPRS records where the Call Event Detail represents a partial. Not present for Wi-Fi usage as there are no partial records for Wi-Fi sessions. Values: F first partial I intermediate partial L last partial | PTI2 | GPRS | Severe | Value out of range. |
| PDP Context Start Timestamp | The timestamp gives the start of the PDP context in the case where the Call Event Details (GPRS Call) represents an intermediate or last partial of a PDP context. The time is given in the local time of the Sender PMN (or Serving Network where this is not the Sender). There must be a UTC Time Offset associated with the timestamp. Note: The local time is the local time at the location of the chargeable subscriber. Where the location is not available, as in some call forwarding scenarios, this will be a notional ‘network local time’. Optionality: Operators may decide not to supply this information (normally this will be because it is not available from the billing system). Conditionality: Must be present within GPRS record where available and Partial Type Indicator is present identifying the GPRS Call as either an intermediate or last partial (subject to optionality). |  |  |  |  |
| Serving Network | A unique identifier used to determine the network where the call event was originally created. This would be a TADIG PMN code (see TADIG PRD TD.13 [2]) Conditionality: Mandatory. | SVN2 SVN3 | Calls Calls | Severe Severe | Value out of range. Missing. |
| Source File Identification | A reference which allows track the record back to source file or multiple source files. Source files could be identified by TAP sequence number, file name or file id. Identification may contain multiple source files references – original TAP file sequence, MNO to MVNO file name and DSP to ARP file id. In case of multiple source files – references should be separated by blank and newest file reference should be at the beginning of the string. Conditionality: Mandatory if received in source file, otherwise on discretion of sender. |  |  |  |  |
| Subscriber Identification | The subscriber identification as defined by Subscriber Identification Type. IMSI It must be full IMSI (not only MCC+MNC) which uniquely identifies the subscriber who has used the network and is liable for any charges that may be incurred. Derivation: GSM item servedIMSI (See Annex C – CS Domain Charging and PS Domain Charging). MSISDN The Mobile Subscriber ISDN number. It is represented in international format, i.e. the number begins with the country code. Derivation: GSM item MSISDN (See Annex C – CS Domain Charging and PS Domain Charging). Public User ID An address identifying the subscriber, either in the form of a SIP URI or TEL URI. Derivation: IMS item SubscriptionID (See Annex C – IMS Domain Charging) Examples: sip:alice@atlanta.com sip:alice:secretword@atlanta.com;transport=tcp sips:alice@atlanta.com?subject=project%20x&priority=urgent sip:+1-212-555-1212:1234@gateway.com;user=phone sips:1212@gateway.com sip:alice@192.0.2.4 sip:atlanta.com;method=REGISTER?to=alice%40atlanta.com sip:alice;day=tuesday@atlanta.com Subscriber Identification conditionality: Mandatory. | SID1 SID3 SID4 | Calls Calls Calls | Severe Severe Severe | Syntax error. Missing. Referenced subscriber does not belong to recipient at the corresponding call event date. |
| Subscriber Identification Type | The type of subscriber identification. Conditionality: Mandatory Values: I IMSI M MSISDN P Public User ID | SIT2 SIT3 | Calls Calls | Severe Severe | Value out of range. Missing. |
| Supplementary Service Code | A code which uniquely defines the supplementary service, a group of supplementary services, or a USSD message. It is combined with Action Code. The Action Code qualifies the way in which the supplementary service is used. Derivation for Supplementary Service: GSM items SupplementaryServiceCode and SS ActionType (See Annex C – CS Domain Charging). The code is derived by taking the MAP encoding (See Annex C – Supplementary Service Codes) and translating bits 8-5 into a ‘hex’ character and bits 4-1 into a second ‘hex’ character. Conditionality: Mandatory within SS records. Mandatory within MOC records where a supplementary service has been used in conjunction with the call unless supplied in a separate SS record. Supplementary Service Codes Values: 00 All supplementary services 10 All line identification services 11 Calling number identification presentation 12 Calling number identification restriction 13 Connected number identification presentation 14 Connected number identification restriction 18 All name identification SS 19 Calling name presentation 20 All call forwarding 21 Call forwarding unconditional 24 Call deflection 28 All conditional Call Forwarding 29 Call forwarding on mobile subscriber busy 2A Call forwarding on no reply 2B Call forwarding on subscriber not reachable 30 All call offering services 31 Call transfer 40 All call completion services 41 Call waiting 42 Call hold 43 Completion of calls to busy subscribers – origination side 44 Completion of calls to busy subscribers – destination side 45 Multicall 50 All multi party services 51 Multi party service 60 All community of interest services 61 Closed user groups 70 All charging supplement services 71 Advice of charge (information) 72 Advice of charge (charging) 80 All additional info transfer services 81 UUS1 user to user signalling 82 UUS2 user to user signalling 83 UUS3 user to user signalling 90 All call barring 91 All Barring of outgoing Call Services 92 Barring of all outgoing calls 93 Barring of all outgoing international calls 94 Barring of all OG international except HPMN 99 All Barring of incoming Call Services 9A Barring of all incoming calls 9B Barring of all IC calls when outside HPMN FF USSD Message Action Codes Values: 0 Registration SS only 1 Erasure SS only 2 Activation SS only 3 Deactivation SS only 4 Interrogation SS only 5 Invocation SS only 6 Registration of Password SS only 7 USSD Invocation USSD only Examples: 2A5 – Invocation of call forwarding on no reply 292 – Activation of call forwarding on mobile subscriber busy | SSV2 SSV3 SSV5 | SS SS MOC | Severe Severe Severe | Value out of range. (Including invalid or non transferable Supplementary Service Code and Action code combination) Missing. Value ‘FF’. |
| Tax Value | The actual value. The Tax Value must be of a value greater than or equal to zero. Conditionality: Mandatory. Values: >= 0 (zero) | TAX1 TAX2 TAX3 TAX4 | Calls Calls Calls Calls | Severe Severe Severe Severe | Syntax error. Value out of range. Missing. Tax Value is not in line with the agreement at the corresponding call event date. |
| Total Call Event Duration | The item contains the actual total duration of a call event as a number of seconds. The Total Call Event Duration must always contain the call duration calculated from the call end time (channel release) minus the Call Event Start Timestamp. The item is used, in conjunction with the Call Event Start Timestamp (and UTC Time Offset) to calculate the call event end time. This is needed for ageing calculations and validation against the File Available Timestamp. Conditionality: Mandatory within MOC MTC GPRS Values: >= 0 (zero) Note: for SMS calls the value must be 0 (zero). | DUR1 DUR2 DUR3 DUR5 | MOC MTC GPRS MOC MTC GPRS MOC MTC GPRS MOC MTC | Severe Severe Severe Severe | Syntax error. Value out of range Missing. Total Call Event Duration is greater than 0 (zero) for SMS call. |

# Error Handling Procedure

## General Rules

- ARP should communicate errors to DSP by email.

- Charging errors should be handled by issuing credit notes. Records with charging errors should not be stopped.

- Relaxed rules should be applied during validation:

Leading and trailing whitespace should be ignored.

Fields not relevant to validated record type should be ignored.

Extra fields at the end of the records should be ignored.

## Error Codes

- Error codes consist of two parts: three letters abbreviation of element name and error number for the element. Error numbers from 1 to 4 used for following types of errors:

- 1 – Syntax error

- 2 – Value out of range

- 3 – Missing

- 4 – Value is not as agreed

- Supplementary Services

The following table contains the relevant combinations of Service Codes and Actions.

For the purposes of the table a notation is used as follows:

‘-‘	An invalid combination of Service Code and Action (as defined by 3GPP TS 22.004)

‘Y’	A valid and relevant combination of Service Code and Action which must be transferred

‘O’	A valid and relevant combination of Service Code and Action which optionally can be transferred in the TAP for customer care purposes

‘N’	A valid and relevant combination of Service Code and Action which must not be transferred

| Supplementary Service |  | Action |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Registration | Erasure | Activation | Deactivation | Interrogation | Invocation | Password |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| 00 | All supplementary services | - | - | - | - | - | - | O |
| 10 | All line identification service | - | - | - | - | - | - | - |
| 11 | Calling number identification presentation | - | - | - | - | N | O | - |
| 12 | Calling number identification restriction | - | - | O | - | N | N | - |
| 13 | Connected number identification presentation | - | - | - | - | N | N | - |
| 14 | Connected number identification restriction | - | - | N | - | N | N | - |
| 18 | All name identification SS | - | - | - | - | N | N | - |
| 19 | Calling name presentation | - | - | - | - | N | O | - |
| 20 | All call forwarding | O | O | O | O | - | - | - |
| 21 | Call forwarding unconditional | O | O | O | O | N | - | - |
| 24 | Call deflection | - | - | - | - | - | O | - |
| 28 | All conditional call forwarding | O | O | O | O | - | - | - |
| 29 | Call forwarding on mobile subscriber busy | O | O | O | O | N | Y | - |
| 2A | Call forwarding on no reply | O | O | O | O | N | Y | - |
| 2B | Call forwarding on subscriber not reachable | O | O | O | O | N | Y | - |
| 30 | All call offering services | - | - | - | - | - | - | - |
| 31 | Call transfer | - | - | - | - | - | O | - |
| 40 | All call completion services | - | - | - | - | - | - | - |
| 41 | Call waiting | - | - | O | O | N | N | - |
| 42 | Call hold | - | - | - | - | - | O | - |
| 43 | Completion of calls to busy subscribers – origination side | - | O | O | O | N | O | - |
| 44 | Completion of calls to busy subscribers – destination side | - | O | O | O | N | O | - |
| 45 | Multicall | N | - | - | - | N | O | - |
| 50 | All multi party services | - | - | - | - | - | - | - |
| 51 | Multi party service | - | - | - | - | - | O | - |
| 60 | All community of interest services | - | - | - | - | - | - | - |
| 61 | Closed user groups | - | - | - | - | - | O | - |
| 70 | All charging supplementary services | - | - | - | - | - | - | - |
| 71 | Advice of charge – information | - | - | - | - | - | O | - |
| 72 | Advice of charge – charging | - | - | - | - | - | O | - |
| 80 | All additional info transfer services | - | - | - | - | - | - | - |
| 81 | UUS1 user to user signalling | - | - | O | O | - | O | - |
| 82 | UUS2 user to user signalling | - | - | O | O | - | O | - |
| 83 | UUS3 user to user signalling | - | - | O | O | - | O | - |
| 90 | All call barring | O | - | - | O | - | - | N |
| 91 | All barring of outgoing call services | - | - | - | O | - | - | - |
| 92 | Barring of all outgoing calls | - | O | O | O | N | - | - |
| 93 | Barring of all outgoing international calls | - | O | O | O | N | - | - |
| 94 | Barring of all outgoing international except HPMN | - | O | O | O | N | - | - |
| 99 | All barring of incoming call services | - | - | - | O | - | - | - |
| 9A | Barring of all incoming calls | - | O | O | O | N | - | - |
| 9B | Barring of all IC calls when outside HPMN | - | O | O | O | N | - | - |

- Changes from Previous Specification Version Number

This section lists the changes from previous Specification Version Number.

| Description | Originator | Specification Version | Comments |
| --- | --- | --- | --- |

- Description of 3GPP Releases and Specifications

  - Charging Specifications

The table below provides a description of the development of 3GPP releases and the corresponding charging specifications applicable to every GSM/3GPP release.

Note: All 3GPP specifications (including the old GSM specifications previously maintained by ETSI) can be found on the 3GPP ftp site: http://www.3gpp.org/ftp/Specs/archive/.

|  | Phase 2 and Release 96 | Releases 97, 98 | Releases 99, 4 | Release 5 | Releases 6, 7, 8, 9, 10, 11 |
| --- | --- | --- | --- | --- | --- |
| CS domain charging | TS 12.05 | TS 12.05 | TS 32.005 | TS 32.205 | TS 32.298 |
| PS domain charging |  | TS 12.15 | TS 32.015 | TS 32.215 | TS 32.298 |
| IMS domain charging |  |  |  | TS 32.225 | TS 32.298 |
| SMS Charging |  |  |  |  | TS 32.298 & 32.274 |
| LCS charging |  |  |  |  | TS 32.298 |

- Step by Step Procedure How to Upload Example ABF File into Database Table

This annex will illustrate how records from example ABF file could be uploaded into database table. This annex is intended as a help for Developers. It does not set any requirements or obligations.

As a database was used Microsoft SQL Server 2008 R2 SP2 Express edition installed on Windows 7 PC. This database could be downloaded free of charge from Microsoft site.

You need to execute following steps:

Download and install Microsoft SQL Server 2008 R2 SP2 Express edition on your PC.

Store example ABF file into directory c:\tmp.

Run SQL Management Studio (installed with SQL Server).

Create database TempDb.

Create table calls in TempDb

Upload example ABF file to calls table.

View uploaded records from calls table.

In order to install SQL Server Express database - run installation file (in my case it was SQLEXPR_x86_ENU.exe), select new installation and accept default settings all the installation time.

Put example ABF file CD_LVALM_ARP01_00001_20130321112000+0300_

20130321112000+0300_1_EUR_3.338_0_7.csv into directory c:\tmp.

Run SQL Management Studio. Press New Query button (upper left corner) and enter SQL commands to execute following steps (commands are in bold).

Enter and execute following commands to create database TestDb:

USE master

GO

CREATE DATABASE TestDb

GO

Enter and execute following commands to create table calls in TestDb database:

USE [TestDb]

GO

CREATE TABLE [calls](

[Call_Type] [char](1) NOT NULL,

[Serv_Netw] [char](5) NOT NULL,

[Source_File] [varchar](100) NULL,

[Subscr_Id_Type] [char](1) NOT NULL,

[Subscr_Id] [varchar](100) NOT NULL,

[Number_or_APN] [varchar](63) NULL,

[Diall_Dig] [varchar](50) NULL,

[Call_Time] [datetimeoffset](7) NOT NULL,

[Tot_Duration] [bigint] NULL,

[Part_Ind] [char](1) NULL,

[PDP_Cont_Time] [datetimeoffset](7) NULL,

[Data_Vol_Inc] [bigint] NULL,

[Data_Vol_Out] [bigint] NULL,

[Basic_Serv_Code] [char](3) NULL,

[Suppl_Serv_Code] [char](3) NULL,

[Cause_for_Term] [int] NULL,

[Charge] [decimal](18, 6) NULL,

[Tax_Value] [decimal](18, 6) NULL,

[Call_Ref] [bigint] NULL,

[CAMEL_Serv_Key] [int] NULL,

[CAMEL_Dest_Number] [varchar](63) NULL,

[CAMEL_APN_OI] [varchar](37) NULL,

[Op_Spec_Inf] [varchar](MAX) NULL,

) ON [PRIMARY]

GO

Enter and execute following commands to upload test file CD_LVALM_ARP01_00001_

20130321112000+0300_20130321112000+0300_1_EUR_3.338_0_7.csv into table calls:

BULK INSERT [TestDb].[dbo].[calls] FROM 'C:\tmp\CD_LVALM_ARP01_00001_20130321112000+0300_20130321112000+0300_1_EUR_3.338_0_7.csv' WITH (FIELDTERMINATOR = ',', ROWTERMINATOR = '\n')

GO

Enter and execute following commands to view records uploaded from example file:

SELECT * FROM [TestDb].[dbo].[calls]

GO

- Document Management

  - Document History

| Version | Date | Brief Description of Change | Approval Authority | Editor / Company |
| --- | --- | --- | --- | --- |
| 0.1 | 21 Mar 2013 | First draft based on Billing Required Data Items | B&P | Sergejs Rusaks / LMT |
| 0.2 | 26 Mar 2013 | Updated as agreed during B&P #6 | B&P | Sergejs Rusaks / LMT |
| 0.3 | 10 Apr 2013 | Data dictionary, annexes A, B and C were added. Validation rules for duplicated records were moved to data dictionary at Call Type element. Comment on error codes added. New annex D – Step by step procedure how to upload example ABF file into database table. | B&P | Sergejs Rusaks / LMT |
| 0.4 | 17 Apr 2013 | Validation rule added on Call Type - Call type, call destination or call service is not as agreed. Charge calculation and validation principles were clarified at Charge element description. Clarified that Local Currency may be or may be not the same as Sender’s country local currency. Basic Service Code – MMS was added to give possibility to charge MMS origination and termination. | B&P | Sergejs Rusaks / LMT |
| 0.5 | 30 Apr 2013 | Removed reference to USSD Added information regarding MMS Added APN NI validation rules for EUInternet Added reference to TD.28 | B&P | Sergejs Rusaks / LMT |
| 0.6 | 2 May 2013 | Made it clearer that binary ftp is a recommendation. Clarified the APN errors with CAMEL. | B&P | Sergejs Rusaks / LMT |
| 0.7 | 23 May 2013 | Clarified that symbol ‘:’ in file name is reserved for special use in some Operating systems. TD was added to file name (test files). Clarified definitions for Recipient and Sender. Text “available from the network” was changed to “available”. Clarified that Annex D is intended as a help for Developers. | B&P | Sergejs Rusaks / LMT |
| 0.8 | 27 June 2013 | Clarification about notification files. Added validation error for old calls. Removed support for MMS retail charging as is not needed. | B&P | Sergejs Rusaks / LMT |
| 1.0 | 8 July | All changes agreed. Final version released. | B&P | Sergejs Rusaks / LMT |
|  | 15 July | Editorial changes to add TD.13 as reference and remove TD.58. | B&P | Sergejs Rusaks / LMT |