# Use of NRTRDE for the Single IMSI Fraud Interface

# Version 1.0

# 8 July 2013

# Introduction

## Overview

This document explains how to use NRTRDE for the optional Single IMSI Fraud Interface (SI-IF8) between the DSP and ARP.

## Scope

This document needs to be read in conjunction with TD.35 [3].

Voice over LTE and SMS over IP is out of scope for the SI-IF8 fraud interface and for further study (will not be received on the VPMN/HPMN interface).

Note: TD.35 [3] has been defined for the VPMN/HPMN fraud prevention interface.  The use of the document also for the DSP/ARP fraud prevention interface therefore needs to consider that it has been written to define the rules on how the VPMN must populate NRTRDE.  When for example TD.35 [3] specifies “available from the network”, this shall be interpreted as “available to the DSP at the time of creation of the ARP fraud prevention file”.

## Definitions

| Term | Description |
| --- | --- |
| Calls | All call events defined for the SI-IF8 |

## Abbreviations

| Term | Description |
| --- | --- |
| ARP | Alternative Roaming Provider |
| BS | Bearer Service |
| DSP | Domestic Service Provider |
| GPRS | General Packet Radio Service |
| GGSN | Gateway GPRS Support Node |
| GSM | Global System for Mobile communications (originally Groupe Spécial Mobile) |
| GSMA | GSM Association |
| HPMN | Home PMN |
| IF | Interface |
| IMEI | International Mobile station Equipment Identity |
| IMSI | International Mobile Subscriber Identity |
| IP | Internet Protocol |
| LTE | Long Term Evolution |
| MCC | Mobile Country Code |
| MNC | Mobile Network Code |
| MO | Mobile Originated |
| MOC | Mobile Originated Call |
| MSISDN | Mobile Station International Subscriber Directory Number |
| MT | Mobile Terminated |
| MTC | Mobile Terminated Call |
| NI | Network Identification |
| NRTRDE | Near Real Time Roaming Data Exchange |
| OI | Operator Identification |
| PMN | Public Mobile Network |
| PRD | Permanent Reference Document |
| RFC | Request for Comments |
| SGSN | Serving GPRS Support Node |
| SI | Single IMSI |
| SMS | Short Message Service |
| SMSC | Short Message Service Centre |
| TADIG | Transferred Account Data Interchange Group |
| TAP | Transferred Account Procedure |
| TS | Teleservice |
| UTC | Universal Time Co-ordinated |
| VPMN | Visited PMN |

## References

| Ref | Doc Number | Title |
| --- | --- | --- |
|  | GSMA PRD BA.20 | Fraud Prevention Procedures |
|  | GSMA PRD TD.28 | File Transfer Methods |
|  | GSMA PRD TD.35 | NRTRDE Format Specification |
|  | RFC 2119 | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997. Available at http://www.ietf.org/rfc/rfc2119.txt |

## Conventions

The key words “must”, “must not”, “required”, “shall”, “shall not”, “should”, “should not”, “recommended”, “may”, and “optional” in this document are to be interpreted as described in RFC 2119 [4].

# General Requirements

## Considerations If Not Implementing This Interface

If the DSP chooses not to implement this interface according to the requirements in this section, it is liable (towards the ARP) for any related proven roaming fraud.  It is recognised that the DSP could have implemented other methods to combat roaming fraud, or (in case the VPMN has failed to comply with the NRTRDE process towards the HPMN) liability for the fraudulent calls could be transferred to the VPMN.

## Requirements On Top Of On-line Interfaces

The DSP must always forward to the ARP SMS-MO records where a third party SMSC is used (due to a potential fraud concern), even where on-line (real-time interfaces) are in place.

## Requirements Where No On-line Interfaces Exist

Where there are no on-line (real-time) retail charging interfaces with the ARP, the DSP must forward to the ARP MOC, MTC and SMS-MO records for all services/destinations that are being managed by the ARP.

## Additional Provisions

The NRTRDE data needs to be sent to the current ARP for that customer as per the start time of the call event.

Sending of other NRTRDE data not listed as mandatory in sections 2.2 and 2.3 (for example SMS-MT and GPRS data) is optional at the discretion of the DSP.  Unless agreed otherwise between the DSP and the ARP, the DSP is doing this at best effort and only in the interest to help the ARP stop fraud, however is not liable in the case it fails to comply with the NRTRDE rules (for example timescales).

For the purpose of fraud prevention, the DSP is also allowed to analyse the NRTRDE data sent to the ARP.  The DSP and ARP should agree on how to manage the fraud process.

The source of the NRTRDE data can be the NRTRDE data received from the VPMN, or any other information (for example SMSC or home GGSN information) at the discretion of the DSP.

# Required Data Items

This section lists all required data items (names as in TD.35 [3]) per call scenario, and how they should be populated on the DSP to ARP interface.

## File Level

| Data Item | How to populate | Comments |
| --- | --- | --- |
| Specification Version Number | Set to 2 |  |
| Release Version Number | Set to 1 |  |
| Sender | TADIG code of DSP |  |
| Recipient | TADIG code of ARP |  |
| Sequence Number | Maintain sequence numbers for the DSP/ARP relationship |  |
| File Available Timestamp (including UTC Time Offset) | Set to the time when the file is sent |  |
| Call Events Count | Calculate from individual call records |  |

## Call Detail Level

### MOC

| Data Item | How to populate | Comments |
| --- | --- | --- |
| IMSI | IMSI | If MSISDN is not populated, then IMSI must be provided in full. If MSISDN is populated, then population of the IMSI is at the discretion of the DSP (as a minimum the MCC/MNC of the HPMN/DSP must be populated). |
| IMEI | IMEI if available |  |
| Call Event Start Timestamp (including UTC Time Offset) | Call Event Start Timestamp (including UTC Time Offset) |  |
| Total Call Event Duration | Total Call Event Duration |  |
| Cause for Termination | Cause for Termination |  |
| TeleService Code | TeleService Code if available | One of TS or BS Code must be present |
| Bearer Service Code | Bearer Service Code if available | One of TS or BS Code must be present |
| Supplementary Service Code | Supplementary Service Code if available | Call forwarding indicator. |
| Dialled Digits | Dialled Digits if available |  |
| Connected Number | Connected Number if available | For SMS-MO, this will contain the SMSC address. |
| Third Party Number | Third Party Number if available |  |
| Recording Entity Identification | Recording Entity Identification | Populate with real Recording Entity Identification if available. If not available, populate with UNKNOWN or VPMN TADIG Code |
| Call Reference | Call Reference or Message Reference where available |  |
| Serving Network | If Serving Network is present in the incoming record, then populate with that. Otherwise populate with the TADIG code of the VPMN. |  |
| MSISDN | MSISDN | Populate if available. |

### MTC

| Data Item | How to populate | Comments |
| --- | --- | --- |
| IMSI | IMSI | If MSISDN is not populated, then IMSI must be provided in full. If MSISDN is populated, then population of the IMSI is at the discretion of the DSP (as a minimum the MCC/MNC of the HPMN/DSP must be populated). |
| IMEI | IMEI if available |  |
| Call Event Start Timestamp (including UTC Time Offset) | Call Event Start Timestamp (including UTC Time Offset) |  |
| Total Call Event Duration | Total Call Event Duration |  |
| Cause for Termination | Cause for Termination |  |
| TeleService Code | TeleService Code if available | One of TS or BS Code must be present |
| Bearer Service Code | Bearer Service Code if available | One of TS or BS Code must be present |
| Calling Number | Calling Number if available | For SMS-MT, this will contain the SMSC address. |
| Recording Entity Identification | Recording Entity Identification | Populate with real Recording Entity Identification if available. If not available, populate with UNKNOWN or VPMN TADIG Code |
| Call Reference | Call Reference or Message Reference where available |  |
| Serving Network | If Serving Network is present in the incoming record, then populate with that. Otherwise populate with the TADIG code of the VPMN. |  |
| MSISDN | MSISDN | Populate if available. |

### GPRS

| Data Item | How to populate | Comments |
| --- | --- | --- |
| IMSI | IMSI | If MSISDN is not populated, then IMSI must be provided in full. If MSISDN is populated, then population of the IMSI is at the discretion of the DSP (as a minimum the MCC/MNC of the HPMN/DSP must be populated). |
| IMEI | IMEI if available |  |
| Call Event Start Timestamp (including UTC Time Offset) | Call Event Start Timestamp (including UTC Time Offset) |  |
| Total Call Event Duration | Total Call Event Duration |  |
| Cause for Termination | Cause for Termination |  |
| Access Point Name NI | Access Point Name NI |  |
| Access Point Name OI | Access Point Name OI if available |  |
| Data Volume Incoming | Data Volume Incoming |  |
| Data Volume Outgoing | Data Volume Outgoing |  |
| SGSN Address | SGSN Address | Populate with real SGSN Address if available. If not available, populate with “1.1.1.1”. |
| GGSN Address | GGSN Address | Populate with real GGSN Address if available. If not available, populate with “1.1.1.1”. |
| Charging Id | Charging Id |  |
| Serving Network | If Serving Network is present in the incoming record, then populate with that. Otherwise populate with the TADIG code of the VPMN. |  |
| MSISDN | MSISDN | Populate if available. |

# Physical Format

The physical format is standard NRTRDE as defined in TD.35 [3].

# File Names

The file naming convention should follow the NRTRDE standard:

NR[DSP TADIG code][ARP TADIG code][Sequence number

Example: NRLUXPTLUXAR0012345

The ARP will need to use an in-country TADIG code, whether they have a registered in-country presence or not.  The GSMA issues TADIG codes, and there are plenty of codes available within each European country.  For the avoidance of doubt, if the ARP is also a MNO with international roaming agreements, it cannot reuse that TADIG code in their role as an ARP.

# Timescales and Liability

The timescale for the VPMN to make the NRTRDE records available to the HPMN is 4 hours from call/partial end time.  As at least some of the information will be sourced from the incoming NRTRDE files, the HPMN will need additional time to make the records available to the ARP (potentially via a MVNO).

The DSP must make correct NRTRDE records (subject to the requirements in section 2) available to the ARP within 8 hours from call/partial end time.  The additional 4 hours is needed to cater for exceptional cases of operational problems, validation, re-rating, transfer via a MVNO, etc.  The actual time can also be shorter than 8 hours as agreed between the DSP and ARP.

The liability rules that apply to the VPMN and HPMN over the VPMN/HPMN interface as defined in BA.20 [1], apply also to the DSP and ARP over the DSP/ARP interface.  The two interfaces are independent of one another.

# Error Reports and File Delivery Reports

File Delivery Reports and Error Reports as defined in TD.35 [3] are mandatory on the VPMN/HPMN interface, and must (as requested by the Fraud Forum) also be used over the DSP/ARP interface.  If not implemented, then the ARP will accept the data as sent by the DSP.

# File Exchange Rules

Physical transfer methods are discussed in TD.28 [2].

- Document Management

  - Document History

| Version | Date | Brief Description of Change | Approval Authority | Editor / Company |
| --- | --- | --- | --- | --- |
| 0.1 | 19 Feb 2013 | First draft | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.2 | 4 Mar 2013 | Updated in line with the comments received on the TAP document | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.3 | 12 Mar 2013 | Updated as agreed during B&P #4 | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.4 | 20 Mar 2013 | Updated as agreed during B&P #5 | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.5 | 25 Mar 2013 | Updated as agreed during B&P #6 | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.6 | 27 Mar 2013 | Correction of editorial mistake in section 5 (2 hours instead of 10 days) | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.7 | 12 Apr 2013 | Updated with information related to liability for non-regulated destinations | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.8 | 18 Apr 2013 | Clarified that MSISDN is the main customer identification. Clarified that the physical format is standard NRTRDE. Updated abbreviations. | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.9 | 19 Apr 2013 | Clarified that the DSP must send NRTRDE data for the services and destinations that are managed by the ARP | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.10 | 24 Apr 2013 | Increased timescale to 8 hours. Clarified liability statements and that the DSP may also use the NRTRDE data for fraud analysis purposes. Refer to TD.28 for file transfer methods. | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.11 | 17 May 2013 | Clarifications to sections 2 and 7, after comments received from DTAG. | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.12 | 22 May 2013 | Approved changes as agreed by B&P #10. Added general statement on availability of data from network. | B&P | Christer Gullstrand / Syniverse Technologies |
| 0.13 | 4 July 2013 | Changes after TADIG TDS review. Added comment that the Recording Entity can be an IP address. Some editorial changes. | B&P | Christer Gullstrand / Syniverse Technologies |
| 1.0 | 8 July 2013 | TADIG TDS changes agreed. Final version released | B&P | Christer Gullstrand / Syniverse Technologies |