# 3GPP TSG SA WG5 (Telecom Management) Meeting #90	S5-131289

# 26-30 August 2013, Valencia, Spain

# Source:	NEC

# Title:	pCR 32.501 Clarify MvPnC EM Connectivity

# Document for:	Approval

Agenda Item:	6.6.2-Multi-Vendor Plug and Play eNB connection to the network (560033)

# Decision/action requested

The group is asked to agree to the proposed changes.

# 2	References

[1]	S5-131057	Post-SA5#89 stand of MUPPET Super-CR to 32.501

[2]	S5-130303	MvPnC Use Case Clarifications

[3]	S5-130934 	 pCR 32.501 Clarify MvPnC EM Resolution

[4]	IETF RFC 2131	Dynamic Host Configuration Protocol

[5]	IETF RFC 2132	DHCP Options and BOOTP Vendor Extensions

[6]	IETF RFC 3315	Dynamic Host Configuration Protocol for IPv6 (DHCPv6)

# 3	Rationale

The current version of agreed Super-CR on multi-vendor plug and play [1] still has one instance of “FFS” on how the eNB can obtain the IP address of its EMS once it is connected to the Secure Operator Network. The main objective of the proposed change is to finalise this open “FFS”.

During session #89 in Sophia Antipolis this topic was discussed to some extent of details but without reaching a final agreement. It was apparent from the discussion that more than one option will have to be considered for the process of enabling the eNB connectivity to its EMS. Each of these options would allow and facilitate deployment depending on operators’ specific implementation preference and existing infrastructure.

The options that are being considred for acquiring the the correct initial EM’s IP address are based on the original discussion [2, 3] and are listed below;

- The IP address or FQDN is provided by the IP Autoconfiguration service, with FQDN to be resolved to an IP address by DNS.

- A FQDN is pre-configured in the eNB and resolved to an IP address through DNS.

The first option above would mean that DHCP server is used to provide the IP address or FQDN of the eNB’s EM. This requires a DHCP server response to be conditioned on the eNB’s vendor.  DHCP servers support adding information in responses conditioned on the “Client Identifier”, the “User Class”, and “Vendor Class” options in queries.  A simple and straight-forward mapping would be to encode the eNB’s vendor into the “Vendor Class” option (option 60 for DHCPv4 [4] , or option 16 for DHCPv6 [5]).

The proposed change is to clarify that the eNB’s EM is configured via DHCP, resolving obtained FQDNs via DNS if necessary.

For the second option, a FQDN of the EM would have to be pre-configured at the eNB and resolved to an IP address via DNS in the Secured Operator Network.

# 4	Detailed proposal

First Modified Section

### 6.4.3	Use case Multi Vendor Plug and Connect eNB to network

| Use Case Stage | Evolution / Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | After physical installation, connect the eNB to its initial Element Manager and to the Core Network(s) as automatically as possible. |  |
| Actors and Roles | eNB as user |  |
| Telecom resources | eNB; IP networks: Non-Secure Operator Network, External Network, and its elements like DHCP server optionally DNS, CA/RA servers, Security Gateway(s) (each protecting one or more Secure Operator Networks), Secure Operator Network(s) including Element Manager(s), Core Network(s) |  |
| Assumptions | There is a functional power supply for the eNB. There may be one or more IP Autoconfiguration Services like DHCP and Router Advertisements and zero or more DNS servers. |  |
| Pre conditions | The eNB is physically installed. IP connectivity exists between the involved telecom resources. The involved telecom resources are functional. The relevant information is stored and available: - Vendor Certificate at the eNB - Operator Certificate at the CA/RA - For the External Network or Non-Secure Operator Network: - (Outer) IP autoconfiguration information at the IP Autoconfiguration Service - FQDN of the initial OAM SeGW at the eNB and/or FQDN or IP address of the initial OAM SeGW at the IP Autoconfiguration Service - FQDN of the CA/RA servers at the eNB and/or FQDN or IP address of the CA/RA servers at the IP Autoconfiguration Service - If FQDNs need to be resolved, corresponding IP address(es) at the DNS server(s) - For the Secure Operator Network: - (Inner) IP autoconfiguration information at the IP Autoconfiguration Service or at the initial OAM SeGW - FQDN or IP address of the initial EMS at the eNB and/or DHCP Server of the Secure Operator Network. - If FQDNs need to be resolved, corresponding IP address(es) at the DNS server(s) - Configuration and software for the eNB at the EM(s) |  |
| Begins when | The eNB is powered up. |  |
| Step 1 (M) | The eNB uses the native VLAN to start communicating on, where PnP traffic is sent and received untagged. |  |
| Step 2 (M) | The eNB acquires its IP address through stateful or stateless IP autoconfiguration. This may provide 0 or more DNS server addresses. |  |
| Step 3 (M) | The eNB acquires the IP address of the CA/RA server. The FQDN of the CA/RA server may be pre-configured in the eNB or the FQDN or IP address of the CA/RA server may be provided by the IP Autoconfiguration Service. FQDNs are resolved through the DNS if necessary. |  |
| Step 4 (M) | The eNB performs Certificate Enrolment. |  |
| Step 5 (M) | The eNB acquires the IP address of the OAM SeGW. The FQDN of the OAM SeGW may be pre-configured in the eNB or the FQDN or the IP address of the OAM SeGW may be provided by the IP Autoconfiguration Service. FQDNs are resolved through the DNS if necessary. |  |
| Step 6 (M) | The eNB establishes a secure connection (tunnel) to the Security Gateway given by Step 5. The eNB receives its (inner) IP autoconfiguration information (which may be the same as the outer IP address obtained in step2) and optionally the address of one or more DNS servers within the Secure Operator Network from the Configuration Parameters of IKEv2 during tunnel establishment. |  |
| Step 7 (M) | The eNB acquires the IP address of the correct Element Manager by either, issuing a DHCP request including the eNB’s vendor information, resolving FQDNs via DNS if necessary, or by having a pre-configured FQDN (including the eNB’s vendor information) resolved via DNS. | Secure connection |
| Step 8 (M) | The eNB establishes a connection to the provided EM and acquires its configuration and software if any. The configuration may contain an address to another EM that this specific node shall use as EM. The configuration may contain an address to another SeGW that should be used before connecting to the EM. The eNB may then - release the connection to the current EM and OAM SeGW and then restart (returning to step 1), - release the connection to the current EM and OAM SeGW and then return to step 6, - release the connection to the current EM and then repeat step 8, or - continue with step 9. | Secure connection |
| Step 9 (M) | The eNB establishes a connection to the Core Network(s) using the transport (VLAN ID, IP addresses) and security parameters provided in step 8. |  |
| Ends when | Ends when all mandatory steps identified above are successfully completed or when an exception occurs. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post Conditions | One or more secure connections exist between the eNB and the Element Manager and the Core Network(s). Via the connection to the Element Manager the eNB can receive further instructions to become operational and carry user traffic, e.g. the administrativeState is set to “unlocked”. |  |
| Traceability | All requirements of clause 5.2 and 6.5.3. |  |

Table 6.4.3-1

Editor’s note: The CA/RA server itself may be protected by a SeGW.

Security aspects – e.g. prevention of unauthorized network access and of fake parameters supplied to the eNBs etc.  -have special importance. Security related sub-steps to establish secure connections are not shown in table 6.4.3-1. More security aspects are described in a specific chapter (see clause 4.3.3).

End of Modifications