3GPP TSG-SA5 Meeting #97	S5-145290

# Venice, Italy October 20-24,  2014														revision of S5-145173

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.628 | CR | 0008 | rev | 1 | Current version: | 12.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Change “esState” in the Annex C text with the correct name of the attribute |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM12 |  |  |  |  |  |  | Date: |  | 2014-10-12 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | “esState” is not defined in the specification but used in the text in Annex C. This should really be “energySavingState” which is defined in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace “esState” with “energySavingState”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Can cause confusion to the reader of the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First modification

Annex C (informative):
State diagram for Distributed and EM-centralized Energy Saving Management

Energy Saving Management “state” attributes are available at various levels:

-	SubNetwork IOC level, via the attribute esSwitch defined in the IOC SonControl;

-	ENBFunction IOC level, via the attribute esSwitch defined in the IOC SonControl;

-	EUtranGenericCell IOC level, via the locally defined attribute isChangeForEnergySavingAllowed, and via the attribute energySavingState defined in the IOC EnergySavingProperties.

The following figure provides an overview of Energy Saving Management related IOCs, state attribute names and possible values, for Distributed and EM-centralized Energy Saving Management:

# Figure C-1: IOCs and state attributes for Distributed and EM-centralized ESM

Dependencies exist between distributed ESM state attributes. The diagram below shows which allowed combinations of attribute state values, and which state transitions, are valid and which are not.

Figure C-2: State diagram for Distributed and EM-centralized ESM

Description:

a)	The ESM-related state attribute esSwitch is used by IRPManager to switch on / off the ESM functionality, in the distributed and EM-centralized architectures. This attribute has two possible values: On, Off. It can be set at the sub-network level and at the eNodeB level;

b)	The ESM-related state attribute isChangeForEnergySavingAllowed is used by IRPManager to prohibit or allow configuration changes of cells individually, within a eNodeB / sub-network, for ESM purposes by the IRPAgent. This attribute has two possible values: Yes, No.

c)	The ESM-related state attribute energySavingState is used to reflect the actual status of a cell regarding the energy saving. This attribute has two possible values: isEnergySaving, isNotEnergySaving.

Figure C-2 describes the following state transitions:

1)	Transition from (esSwitch = On and isChangeForEnergySavingAllowed = Yes and energySavingState = isNotEnergySaving) to((esSwitch = On and isChangeForEnergySavingAllowed = Yes and  energySavingState = isEnergySaving): happens when:

ES Policies attached to the subject cell and to candidate for compensation cells, e.g. threshold and duration, are satisfied and allow entering Energy Saving state.

2)	Transition from (esSwitch = On and isChangeForEnergySavingAllowed = Yes and energySavingState = isEnergySaving) to(esSwitch = On and isChangeForEnergySavingAllowed = Yes and  energySavingState = isNotEnergySaving): happens when:

ES Policies attached to the subject cell and to candidate for compensation cells, e.g. threshold and duration, are satisfied and allow leaving Energy Saving state.

3)	Transition from (esSwitch = On and isChangeForEnergySavingAllowed = Yes) to (esSwitch = On and isChangeForEnergySavingAllowed = No and energySavingState = isNotEnergySaving): happens when:

IRP Manager sets the attribute isChangeForEnergySavingAllowed to “No”.

4)	Transition from (esSwitch = On and isChangeForEnergySavingAllowed = No and energySavingState = isNotEnergySaving) to isChangeForEnergySavingAllowed = Yes: happens when:

IRP Manager sets the attribute isChangeForEnergySavingAllowed to “Yes”.

5)	Transition from (esSwitch = On and isChangeForEnergySavingAllowed = Yes) to( esSwitch = Off and isChangeForEnergySavingAllowed = Yes and energySavingState = isNotEnergySaving): happens when:

IRP Manager sets the attribute esSwitch to “Off”.

6)	Transition from (esSwitch = Off and isChangeForEnergySavingAllowed = Yes and energySavingState = isNotEnergySaving) to (esSwitch = On and isChangeForEnergySavingAllowed = Yes and energySavingState = isNotEnergySaving): happens when:

IRP Manager sets the attribute esSwitch to “On”.

7)	Transition from (esSwitch = On and isChangeForEnergySavingAllowed = No and energySavingState = isNotEnergySaving) to (esSwitch = Off and isChangeForEnergySavingAllowed = No and energySavingState = isNotEnergySaving): happens when:

IRP Manager sets the attribute esSwitch to “Off”.

8)	Transition from (esSwitch = Off and isChangeForEnergySavingAllowed = No and energySavingState = isNotEnergySaving) to (esSwitch = On and isChangeForEnergySavingAllowed = No and energySavingState = isNotEnergySaving): happens when:

IRP Manager sets the attribute esSwitch to “On”.

End of modifications