# 3GPP TSG SA WG5 (Telecom Management) Meeting #99	S5-151127

# 2-6 February 2015 Taipei, Taiwan	revision of S5-140abc

# Source:	Alcatel-Lucent

# Title:	Discussion paper for TR196 changes

# Document for:	Discussion

Agenda Item:	5.3

# 1	Decision/action requested

Discussion and response to BBF

# 2	References

[1] S5-151041 - bbf2014.1291.01 - Proposed TR-196i2a1 Statistics and Energy Saving Parameters for 3GPP Release 11, 12 and 13  - http://www.3gpp.org/ftp/Inbox/LSs_from_external_bodies/Broadband_Forum/bbf2014.1291.01.zip

# 3	Rationale

The following contribution provides BBF to define in response to their liaison to 3GPP (bbf2014.1291.01).

Item 1: Data Model Corrections:

In developing the profiles we discovered a parameter in the model provided by the BBF was incorrect.

FAPService.{i}.CellConfig.UMTS.CN.CSDomain.RAB.Stats.FailEstabPS should have been FAPService.{i}.CellConfig.UMTS.CN.CSDomain.RAB.Stats.FailEstabCS

Resolution:

FAPService.{i}.CellConfig.UMTS.CN.CSDomain.RAB.Stats.FailEstabPS to FAPService.{i}.CellConfig.UMTS.CN.CSDomain.RAB.Stats.FailEstabCS

Item 2: Proposed profiles

The BBF liaison requests 3GPP provide feedback on how we would like the profiles to be defined.

Profiles in TR-069 are used for 2 purposes:

- As a deprecated mechanism for CPEs to report their capabilities

- For SPs to use when requesting feature support from vendors. This is the most common usage.

The approach to the profiles taken by this contribution was to utilize the most granular set of profiles based on the requested parameters in TS 32.452, TS 32.592, TS 32.453). This is similar to how the profiles were defined for previous features.

Resolution:

Agree to the following profiles:

### UMTSCSRABMonitoring:1 Profile

This table defines the UMTSCSRABMonitoring:1 profile for the FAPService:2 data model. The minimum REQUIRED version for this profile is FAPService:2.99.

| Name | Requirement |
| --- | --- |
| FAPService.{i}.CellConfig.UMTS.CN.CSDomain.RAB. | P |
| FAPService.{i}.CellConfig.UMTS.CN.CSDomain.RAB.Stats. | P |
| AttEstabCS | R |
| FailEstabCS | R |
| SuccEstabCS | R |
| RelDropCS | R |

### UMTSCSHHOMonitoring:1 Profile

This table defines the UMTSCSHHOMonitoring:1 profile for the FAPService:2 data model. The minimum REQUIRED version for this profile is FAPService:2.99.

| Name | Requirement |
| --- | --- |
| FAPService.{i}.CellConfig.UMTS.CN.CSDomain.HHO. | P |
| FAPService.{i}.CellConfig.UMTS.CN.CSDomain.HHO.Stats. | P |
| Att3GMacroToHnbCSHO | R |
| Succ3GMacroToHnbCSHO | R |
| Att2GMacroToHnbCSHO | R |
| Succ2GMacroToHnbCSHO | R |

### UMTSPSRABMonitoring:1 Profile

This table defines the UMTSPSRABMonitoring:1 profile for the FAPService:2 data model. The minimum REQUIRED version for this profile is FAPService:2.99.

| Name | Requirement |
| --- | --- |
| FAPService.{i}.CellConfig.UMTS.CN.PSDomain.RAB. | P |
| FAPService.{i}.CellConfig.UMTS.CN.PSDomain.RAB.Stats. | P |
| AttEstabPS | R |
| FailEstabPS | R |
| SuccEstabPS | R |
| RelDropPS | R |
| ULNumUserBitsEDCH | R |
| ULNumUserBits64K | R |
| ULNumUserBits128K | R |
| ULNumUserBits384K | R |
| DLNumUserBitsHSDPA | R |
| DLNumUserBits64K | R |
| DLNumUserBits128K | R |
| DLNumUserBits384K | R |

### UMTSPSRABLGWMonitoring:1 Profile

This table defines the UMTSPSRABLGWMonitoring:1 profile for the FAPService:2 data model. The minimum REQUIRED version for this profile is FAPService:2.99.

| Name | Requirement |
| --- | --- |
| FAPService.{i}.CellConfig.UMTS.CN.PSDomain.RAB. | P |
| FAPService.{i}.CellConfig.UMTS.CN.PSDomain.RAB.Stats. | P |
| LGWNumberOfEntries | R |
| FAPService.{i}.CellConfig.UMTS.CN.PSDomain.RAB.Stats.LGW.{i}. | P |
| CorrelationID | R |
| EstabDirectPathLGWAttNbr | R |
| EstabDirectPathLGWSuccNbr | R |
| EstabDirectPathLGWFailNbr | R |

### UMTSHHOMonitoring:1 Profile

This table defines the UMTSHHOMonitoring:1 profile for the FAPService:2 data model. The minimum REQUIRED version for this profile is FAPService:2.99.

| Name | Requirement |
| --- | --- |
| FAPService.{i}.CellConfig.UMTS.CN.HHO. | P |
| FAPService.{i}.CellConfig.UMTS.CN.HHO.Stats. | P |
| SuccHnbUmtsIntraFreq | R |
| AttHnbUmtsIntraFreq | R |
| AttHnbUmtsInterFreq | R |

### UMTSRANRRCMonitoring:1 Profile

This table defines the UMTSRANRRCMonitoring:1 profile for the FAPService:2 data model. The minimum REQUIRED version for this profile is FAPService:2.99.

| Name | Requirement |
| --- | --- |
| FAPService.{i}.CellConfig.UMTS.RAN.RRC. | P |
| FAPService.{i}.CellConfig.UMTS.RAN.RRC.Stats. | P |
| AttConnEstab | R |
| FailConnEstab | R |
| SuccConnEstab | R |

### LTEEPCHOMonitoring:1 Profile

This table defines the LTEEPCHOMonitoring:1 profile for the FAPService:2 data model. The minimum REQUIRED version for this profile is FAPService:2.99.

| Name | Requirement |
| --- | --- |
| FAPService.{i}.CellConfig.LTE.EPC.HO. | P |
| FAPService.{i}.CellConfig.LTE.EPC.HO.Stats. | P |
| ToeNBAtt | R |
| ToeNBSucc | R |
| ToeNBFail | R |
| FromeNBAtt | R |
| FromeNBSucc | R |
| FromeNBFail | R |
| ToUtranAtt | R |
| ToUtranSucc | R |
| ToUtranFail | R |
| ToGeranAtt | R |
| ToGeranSucc | R |
| ToGeranFail | R |

### LTERANCSGMonitoring:1 Profile

This table defines the LTERANCSGMonitoring:1 profile for the FAPService:2 data model. The minimum REQUIRED version for this profile is FAPService:2.99.

| Name | Requirement |
| --- | --- |
| FAPService.{i}.CellConfig.LTE.RAN.CSG. | P |
| FAPService.{i}.CellConfig.LTE.RAN.CSG.Stats. | P |
| MeanNbrUsr | R |
| AttInboundMobility | R |
| SuccInboundMobility | R |
| FailedInboundMobility | R |

### LTERANRRCMonitoring:1 Profile

This table defines the LTERANRRCMonitoring:1 profile for the FAPService:2 data model. The minimum REQUIRED version for this profile is FAPService:2.99.

| Name | Requirement |
| --- | --- |
| FAPService.{i}.CellConfig.LTE.RAN.RRC. | P |
| FAPService.{i}.CellConfig.LTE.RAN.RRC.Stats. | P |
| AttConnEstab | R |
| FailConnEstab | R |
| SuccConnEstab | R |

### LTERANERABMonitoring:1 Profile

This table defines the LTERANERABMonitoring:1 profile for the FAPService:2 data model. The minimum REQUIRED version for this profile is FAPService:2.99.

| Name | Requirement |
| --- | --- |
| FAPService.{i}.CellConfig.LTE.RAN.ERAB. | P |
| FAPService.{i}.CellConfig.LTE.RAN.ERAB.Stats. | P |
| EstabInitAttNbr | R |
| EstabInitSuccNbr | R |
| EstabInitFailNbr | R |
| EstabAttNbr | R |
| EstabSuccNbr | R |
| EstabFailNbr | R |
| RelEnbNbrQCI | R |
| RelEnbNbrCause | R |

### LTERANERABLGWMonitoring:1 Profile

This table defines the LTERANERABLGWMonitoring:1 profile for the FAPService:2 data model. The minimum REQUIRED version for this profile is FAPService:2.99.

| Name | Requirement |
| --- | --- |
| FAPService.{i}.CellConfig.LTE.RAN.ERAB. | P |
| FAPService.{i}.CellConfig.LTE.RAN.ERAB.Stats. | P |
| LGWNumberOfEntries | R |
| FAPService.{i}.CellConfig.LTE.RAN.ERAB.Stats.LGW.{i}. | P |
| CorrelationID | R |
| EstabDirectPathLGWAttNbr | R |
| EstabDirectPathLGWSuccNbr | R |
| EstabDirectPathLGWFailNbr | R |

### LTERANEDRBMonitoring:1 Profile

This table defines the LTERANEDRBMonitoring:1 profile for the FAPService:2 data model. The minimum REQUIRED version for this profile is FAPService:2.99.

| Name | Requirement |
| --- | --- |
| FAPService.{i}.CellConfig.LTE.RAN.DRB. | P |
| FAPService.{i}.CellConfig.LTE.RAN.DRB.Stats. | P |
| PdcpSduBitrateDl | R |
| PdcpSduBitrateUl | R |
| PdcpSduBitrateDlMax | R |
| PdcpSduBitrateUlMax | R |
| PdcpSduDelayDl | R |
| PdcpSduDropRateDl | R |
| PdcpSduAirLossRateDl | R |
| PdcpSduLossRateUl | R |
| UEActiveDl | R |
| UEActiveUl | R |

### LTERANERRUMonitoring:1 Profile

This table defines the LTERANERRUMonitoring:1 profile for the FAPService:2 data model. The minimum REQUIRED version for this profile is FAPService:2.99.

| Name | Requirement |
| --- | --- |
| FAPService.{i}.CellConfig.LTE.RAN.RRU. | P |
| FAPService.{i}.CellConfig.LTE.RAN.RRU.Stats. | P |
| PrbTotDl | R |
| PrbTotUl | R |

Item 3: Proposed data model

The XML provided in the BBF liaison was modified with the profile entries in Item 2 and the correction in Item 1.

This data model should be sent back to the BBF to be used for the TR-196 specification.

Resolution:

Include the il-196-2-99-0_S5.xml, il-196-2-99-0_S5-diffs.html, il-196-2-99-0_S5.html files in the response liaison to BBF.

# 4	Detailed proposal

TR-196 changes attached. il-196-2-99-0_S5.zip.

Proposal is to send LS back to BBF with comments if any on the proposed TR196 changes.