3GPP TSG-SA WG6 Meeting #8	S6-151350

Anaheim, USA, 16th – 20th November 2015	(revision of S6-151xxx)

Title:	Simplified introduction to configuration data annex

Agenda Item:	7.1.1

Source:	Vodafone

Contact:	peter.dawes@vodafone.com

Abstract: The introduction to Annex B that lists configuration parameters etc. is simplified. Also, it is proposed to mandate that the UE is pre-provisioned with parameters only if such pre-provisioning is essential for the MCPTT service to work.

*** first change ***

Annex B (normative):
MCPTT data

## B.1	Introduction

This Annex contains tables of the parameters (e.g. maximum values, timer values) and authorizations that adjust and control the MCPTT service. Reference tables are divided into items that apply to both on- and off-network and those that apply to on-network only. Table columns are as follows:

-	Corresponding 3GPP TS 22.179 [2] requirement. Only one reference is given even if several requirements are relevant.

-	Definition: Description of the parameter, authorization etc.

-	UE provisioned:Must be provided to the UE before the MCPTT service can be used. .

-	UE notified: During service operation, provided by the network to the UE when required.

NOTE:	UE notified data isassumed to be provided using SIP signalling.

-	Network managed/enforced: Controlled and/or enforced by the network. The MCPTT service can work if this item is not available to the UE.

Editor's note: it is FFS if another SIP reference point is needed.

Editor's note: Following tables are incomplete.

## B.2	MCPTT UE data

Tables B.2-1, B.2.-2 list UE specific data.

Table B.2-1: MCPTT UE data (both on network and off network)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| 5.5.2-005 | Max number of simultaneous group calls (N4) | X |  | X (CSC-4) |
| 5.5.2-006 | Max number of transmissions (N5) in a group | X |  | X(CSC-4) |
| 5.5.2-007 | Max number of private calls(N10) | X |  | X(CSC-4) |

Table B.2-2: MCPTT UE related data (on network only)

| Reference | Definition | UE provisioned (CSC-4) | UE notified () | Network enforced/managed |
| --- | --- | --- | --- | --- |
| 6.13.4-001 | Enabling/disabling of UE |  | X | X(CSC-4) |

## B.3	MCPTT user data

Tables B.3-1, B3-2 summarise data specific to an MCPTT user.

Table B.3-1: MCPTT user data (both on network and off network)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| 5.5.2-008 | Max number of simultaneously received group calls (N6) | X |  | X(CSC-2) |
| 5.5.2-009 | Max number of transmissions (N7) | X |  | X CSC-2) |
| 5.6.5-001 | Authorised to make private call with manual commencement | X |  | X(CSC-4) |
| 5.6.5-003 | User which can be called in private call | X |  | X(CSC-4) |
| 5.6.5-002 | Authorised to make private call with automatic commencement | X |  | X(CSC-4) |
| 5.6.5-004 | Max private call duration | X |  | X(CSC-4) |
| 5.6.5-005 | Max hang time for private calls | X |  | X(CSC-4) |
| 5.6.5-006 | Restriction in notification of call failure reason for private call | X |  | X(CSC-4) |
| 5.7.2.2.2-002 | Authorisation for imminent in- peril cancelation | X |  | X(CSC-4) |
| 5.7.2.3.1-001 | Authorisation to make an emergency private call | X |  | X(CSC-4) |
| 5.7.2.3.2-001 | Authorisation to cancel emergency priority in a private emergency call | X |  | X(CSC-4) |
| 5.7.2.4.1-002 | Authorisation to activate emergency alert | X |  | X(CSC-4) |

Table B.3-2: MCPTT user data

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |

## B.4	Group call data

Tables B.4-1, B4-2 summarise data specific to MCPTT group calls.

Table B.4-1: MCPTT group calldata (both on network and off network)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| 5.7.2.1.2-002 | Timeout value for the cancellation of an in progress emergency for a group call | ? |  | X(CSC-2) |
| 5.7.2.2.2-004 | Timeout value for the cancellation of an in progress imminent-peril group call | ? |  | X(CSC-2) |

Table B.4-2: MCPTT group call data (on network only)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| 6.2.4-003 6.4.9-002 | Group call Hang timer |  |  | X(CSC-4) |

Table B.4-3: MCPTT group calldata (off network only)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-7.4-002] [R-7.4-003] | Group call Hang timer |  |  | X(CSC-4) |

## B.5	Service data

Tables B.5-1, B.5-2 and B.5-3 summarise the data related to an instance of an MCPTT service.

Table B.5-1: Service data (both on-network and off-network)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| 5.7.2.3.2-002 | Timeout value for the cancellation of an in progress emergency for a private call | X |  | X(CSC-2) |

Table B.5-2: Service data (on-network only)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
|  | Hang timer for private calls |  |  |  |

Table B.5-3: Service data (off-network only)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-7.4-002] [R-7.4-003] | Hang timer for private calls in off-network |  |  |  |