3GPP TSG-SA WG6 Meeting #12	S6-160791

Tenerife - Santa Cruz, Spain, 25th – 29th July 2016	(revision of S6-160747)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on MCPTT user logging from multiple UEs

Spec:	3GPP TS 23.PTT

Agenda item:	7.5

Document for:	Agreement

1. Introduction

This contribution will focus on the requirement and identify the possible requirement to change the existed signalling flow to support MCPTT user logging in from multiple UEs.

2. Reason for Change

According to the following SA1 requirements from TS 22.280, an MCPTT User can simultaneously have several active MCPTT UEs: “

5.11	Support for multiple devices

[R-5.11-001] The MCX Service shall allow an MCPTT User to log in to multiple MCPTT UEs concurrently.

[R-5.11-002] The MCX Service shall ensure that the MCPTT User logs into each MCPTT UE separately.”

For each MCPTT user, it may perform the following signalling procedure for getting MCPTT service.

- Service registration

- Affiliation

- Group call setup( originating side or terminating side)

- Private call setup

- Group management( Being notified about the creation of new group).

These signalling flow may need to be changed to cover the case that one MCPTT user logging from mutiples MCPTT Ues.

This contribution mainly focus on the possible impact to the Service registration, group call setup and private call setup procedure. The impact to service registration is that MCPTT server need to store multiple registraion information each may have different service setting.

TS 22.280 also has the following requirement about mutiple UEs.

“ 6.11	Support for multiple devices

[R-6.11-001] The MCPTT Service shall provide a notification to the MCPTT User if the MCPTT User is already logged on to another MCPTT UE.”

Accordingly, it is proposed to change the flow as the following CR.

3. Conclusions and proposal

It is proposed to agree the following changes to 3GPP TS 23.PTT.

* * * First Change * * * *

10.x Support for multiple devices

10.x.1	General

An MCPTT user may be authorized to use the MCPTT service from multiple MCPTT UEs as per procedure in subclause 10.2.

If an MCPTT server receives a service authorization request for an MCPTT user already authorized to use the MCPTT service from another MCPTT UE, the MCPTT server shall process the new service authorization request and include in the confirmation an indication that the MCPTT user is already authorized to use the MCPTT service from another MCPTT UE.

Editor’s Note:	How to log off remotely from MCPTT clients on other MCPTT UEs that the MCPTT user is already authorized to use the MCPTT service from is FFS.

******************************End of the CR*******************************************