3GPP TSG-SA WG6 Meeting #26	S6-181362

Vilnius, Lithuania, 15th – 19th October 2018	(revision of S6-18xxxx)

Source:	Motorola Solutions

Title:	pCR Solution for discreet listening group membership

Spec:	3GPP TR 23.784 v020

Agenda item:	10.5

Document for:	Approval

Contact:	david.chater-lea@motorolasolutions.com

1. Introduction

This pCR adds a solution to key issue 2, group membership of authorized user for discreet listening.

2. Reason for Change

To provide a solution for group membership for discreet listening and logging.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.784 v020.

* * * First Change * * * *

## 6.x	Solution x:	Group membership for discreet listening

### 6.x.1	Description

Key issue 2 raises the issue of whether the authorized MC user should be a pre-defined member of the MC service group in order to provide discreet listening of calls in the MC service group which include the target MC user.

If the authorized MC user is required to become a member of an MC service group in order to perform DL on a target user when otherwise that authorized user would not have been a member of that MC service group, other group members including the target of the DL may be aware that the authorized user is now a group member (temporarily or permanently). This would contravene the requirement stated in 3GPP TS 22.280 [2] and reproduced in Annex A that the DL service is provided without the target being aware of the DL.

Therefore, provision of DL needs to be independent of the group membership of the authorized MC user, and will be a separate service provided by the MC service server which provides MC service to that authorized MC user.

### 6.x.2	Impacts on existing nodes and functionality

This solution results in additional functionality in the MC service server which provides service of the authorized MC user to provide the content of group communications to the authorized MC user without the authorized MC user needing to become a member of the MC service group.

Additionally, interactions between MC service servers may be needed to satisfy the scenario where the target user is receiving MC service from a different serving MC service server from the target of the DL.

### 6.x.3	Solution Evaluation

This solution allows discreet listening to be achieved, satisfying the requirement that the target is not made aware of the DL operation. It provides resolution to key issue 2.