3GPP TSG-SA WG6 Meeting #26	S6-181443

Vilnius, Lithuania, 15th – 19th October 2018	(revision of S6-181354)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0151 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | User regroup procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh2MCPTT |  |  |  |  |  |  | Date: |  | 2018-10-08 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The temporary user regroup procedure does not have a security solution in 3GPP TS 33.180. To resolve this, a revised procedure is provided which makes use of existing group creation and group call solutions, which enables the security solutions for these to be reused. There is not a performance issue in reusing the existing group creation procedure for a temporary user regroup, as the human authorized user or dispatcher has to assemble a list of users prior to the start of the call, and therefore human factors dictate that the process takes some finite time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The existing temporary user regroup procedure is modified by reusing existing procedures for group creation and group call. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The user regroup procedure will not satisfy the need for a realizable security procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.2.8.1, 10.6.2.8.2, 10.6.2.8.3, 10.6.2.2.7, 10.6.2.2.9, 10.6.2.2.14, 10.6.2.2.14a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.6.2.8	Temporary group call – user regroup

##### 10.6.2.8.1	General

The temporary group call performed by user regroup can be initiated by an authorized user inviting a list of MCPTT users. The group ID for this temporary group is obtained prior to call setup.. The release of the call may be followed by release of the temporary group ID.

##### 10.6.2.8.2	Temporary group setup and group call

Figure 10.6.2.8.2-1 below illustrates the temporary group call creation and call setup procedure initiated by an authorized user.

Pre-conditions:

1.	The authorized user is aware of the MCPTT users who will be invited to the temporary group call.

2.	The MCPTT server is allowed to perform implicit affiliation/de-affiliation operation for this type of group call.

3.	The MCPTT users are allowed to be implicitly affiliated for this type of group call.

4.	A group ID is available prior to group call setup.

Figure 10.6.2.8.2-1: Group call setup for temporary group call – user regroup

1.	The authorized user of MCPTT UE 1 makes use of the group management client of MCPTT UE 1 to create the temporary group according to the group creation procedure in 3GPP TS 23.280 [16] subclause 10.2.3. The configuration identifies the group as a temporary group. As part of this procedure, the MCPTT users are notified of their membership to the temporary group, and the MCPTT server is notified about the creation of the group and the list of group members.

2.	The user of MCPTT UE 1 initiates a group call according to the procedures described in subclauses 10.6.2.3.1.1 or 10.6.2.3.1.2 of the present document. The receiving MCPTT clients of MCPTT UEs 2 and 3 are implicitly affiliated to the group and are notified of this affiliation during the call setup.

##### 10.6.2.8.3	Group call end (by authorized user)

The temporary group call is ended by following the call release procedures described in subclauses 10.6.2.3.1.1.3 or 10.6.2.3.1.2.3 of the present document.

If the authorized user wishes to end the use of the temporary group, the procedure for group deletion described in 3GPP TS 23.280 [16] is followed.

* * * Next Change * * * *

##### 10.6.2.2.7	Group call request (MCPTT client – MCPTT server)

Table 10.6.2.2.7-1 describes the information flow group call request from the MCPTT client to the MCPTT server.

Table 10.6.2.2.7-1 Group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID (NOTE) | O | The MCPTT group ID of the group on which the call is requested |
| SDP offer | M | Media parameters of MCPTT clients |
| Implicit floor request | O | When originating client requests the floor, this element shall be included |
| Broadcast indicator | O | Indicates that the group call request is for a broadcast group call |
| NOTE: the MCPTT group ID shall not be present for the temporary group call - user regroup. |  |  |

* * * Next Change * * * *

##### 10.6.2.2.9	Group call request (MCPTT server – MCPTT client)

Table 10.6.2.2.9-1 describes the information flow group call request from the MCPTT server to the MCPTT client.

Table 10.6.2.2.9-1 Group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is initiated |
| SDP offer | M | Media parameters of MCPTT server |
| Broadcast indicator | O | Indicates that the group call request is for a broadcast group call |

* * * Next Change * * * *

##### 10.6.2.2.14	Group call release request (MCPTT server – MCPTT client)

Table 10.6.2.2.14-1 describes the information flow group call release request from the MCPTT server to the MCPTT client.

Table 10.6.2.2.14-1 Group call release request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the MCPTT group member |
| Functional alias | O | The functional alias of the MCPTT group member |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is released |

* * * Next Change * * * *

##### 10.6.2.2.14a	Group call release request (MCPTT client – MCPTT server)

Table 10.6.2.2.14a-1 describes the information flow group call release request from the MCPTT client to the MCPTT server.

Table 10.6.2.2.14a-1 Group call release request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the MCPTT group member |
| Functional alias | O | The functional alias of the MCPTT group member |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is released |