3GPP TSG-SA WG6 Meeting #31	S6-190899

Bruges, Belgium, 20th – 24th May 2019	(revision of S6-19xxxx)

Source:	Motorola Solutions

Title:	Update solution 1 for multiple UEs

Spec:	3GPP TR 23.784 v110

Agenda item:	10.3

Document for:	Approval

Contact:	david.chater-lea@motorolasolutions.com

1. Introduction

This pCR updates solution 1, 'Functionality for discreet listening', to support discreet listening where the target user is receiving MC service from multiple MC service UEs.

2. Reason for Change

Solution 1 provides a solution for discreet listening, where the communications of a user are requested and provided according to MC service ID. If a user is receiving service from multiple devices, an additional client ID can be used to identify which MC service client the communications are originating from or being sent to. This will provide a solution for discreet listening aspect of key issue 9: 'Discreet listening and logging for user receiving MC service on multiple devices'.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.784 v110.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.280:	"Mission Critical Services Common Requirements (MCCoRe)"

[3]	3GPP TS 22.281:	"Mission Critical Video services"

[4]	3GPP TS 22.179: "Mission Critical Push To Talk (MCPTT); Stage 1"

[5]	3GPP TS 22.282: "Mission Critical Data services"

[6]	3GPP TS 33.180: "Security of the mission critical service"

[7]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2"

[8]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[9]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[10]	3GPP TS 24.379: "Mission Critical Push To Talk (MCPTT) call control; Protocol specification"

[11]	3GPP TS 24.281: "Mission Critical Video (MCVideo) signalling control; Protocol specification"

[12]	3GPP TS 24.282: "Mission Critical Data (MCData) signalling control; Protocol specification"

* * * Next Change * * * *

##### 6.1.1.3.2	Discreet listening invocation and revocation procedures

The procedure allows the MC service client of the authorized user to request discreet listening of a target MC service user.  The procedure is illustrated in figure 6.1.1.3.2-1 below.

Figure 6.1.1.3.2-1: Discreet listening invocation

The information flows needed to support such a procedure are shown in tables 6.1.1.3.2-1 and 6.1.1.3.2-2 below.

Table 6.1.1.3.2-1: Discreet listening activation request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | MC service ID of the target of the discreet listening request |

Table 6.1.1.3.2-2: Discreet listening activation response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | MC service ID of the target of the discreet listening request |
| Result | M | Indicates the success or failure of the discreet listening request |

Invocation of discreet listening of a target MC service user will result in all communications sent or received to any MC service client on which the target MC service user is receiving MC service being provided to the authorized user.

A similar procedure, not illustrated here, is required for revocation of the discreet listening function.

##### 6.1.1.3.3	Provision of discreet listening content for private communications

When the target user makes or receives a private call or initiates individual communications with a server, the authorized user needs to be informed and provided with the media. Figure 6.1.1.3.3-1 below illustrates a procedure where discreet listening target MC service client 1 sets up a call.

Figure 6.1.1.3.3-1: Discreet listening of target initiated private call

The procedure is similar if the target of the discreet listening is the recipient of the private call.

If the target initiated the private call, the discreet listening call request may include the client ID associated with the MC service client on which the target initiates the call. If the target receives a request for a private call, the discreet listening call request may include the client ID(s) of the MC service clients on which the private call request is received.

NOTE:	If the MC service server does not receive the 'discreet listening call response' for any reason, the call should still proceed so that the requirement that the call is not affected and the target user is not aware of the discreet listening process is satisfied.

##### 6.1.1.3.4	Provision of discreet listening content for group communications

When the target user makes or receives a group call, the authorized user needs to be informed and provided with the media. Figure 6.1.1.3.4-1 below illustrates a procedure where discreet listening target MC service client 2 is a receiving party in a group call initiated by another affiliated group member.

Figure 6.1.1.3.4-1: Discreet listening of target initiated group call

The procedure is similar if the target of the discreet listening is the initiator of the group call.

If the target initiated the group call, the discreet listening call request may include the client ID associated with the MC service client on which the target initiates the call. If the target receives a request for a group call, the discreet listening call request may include the client ID(s) of the MC service clients on which the group call request is received.

NOTE 1:	If the MC service server does not receive the 'discreet listening call response' for any reason, the call should still proceed so that the requirement that the call is not affected and the target user is not aware of the discreet listening process is satisfied.

NOTE 2:	It is irrelevant whether the authorized user is or is not a member of the MC service group.

##### 6.1.1.3.5	Information flows for discreet listening

A single pair of information flows can support provision of discreet listening content of individual communications and group communications. Suitable information elements are shown in tables 6.1.1.3.5-1 and 6.1.1.3.5-2 below.

Table 6.1.1.3.5-1: Discreet listening call request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | MC service ID of the target of discreet listening |
| MC service client ID list (see NOTE) | O | List of MC service client IDs of the target user on which the call is requested |
| MC service ID | M | MC service ID of the other party in individual call, or MC service ID of talking party in group call |
| Call direction | M | Indicates whether the target or another party initiated the call |
| Call type | M | Indicates the type of communication (private call, group call etc) |
| MC service group ID | O | Provided if the communication is a group call |
| NOTE: MC service client ID identifies a specific installation of an MC service client on an MC service UE and is provided to the MC service server at service authorization of the MC service user. It may be equivalent to the client IDs specified in 3GPP TS 24.379 [10], 3GPP TS 24.281 [11] and 3GPP TS 24.282 [12]. |  |  |

Table 6.1.1.3.5-2: Discreet listening call response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | MC service ID of the target of discreet listening |
| Result | M | Indicates the success or failure of the discreet listening call request |

* * * Next Change * * * *

### 6.1.3	Solution Evaluation

This solution allows discreet listening to be achieved with minimal changes to the functional model as it only requires enhancement to existing functional entities and reference points. It also allows discreet listening to be invoked on a service dependent basis as it is achieved as a function of the MC service specific MC service clients. It supports the situations where the MC service user is receiving MC service on multiple MC service clients.