3GPP TSG-SA WG6 Meeting #31	S6- 191153

Bruges, Belgium, 20th – 24th May 2019	(revision of S6-190963)

Source:	Samsung, Deutsche Telekom AG

Title:	Solution for key issue 5 – Location API

Spec:	3GPP TR 23.758 v0.1.0

Agenda item:	10.7

Document for:	Approval

Contact:	Sunghoon Kim (s.hun.kim@samsung.com)

1. Introduction

According to Key issue 5: Capability Exposure to Edge Application, the Edge Application can utilize capability exposure services provided by the Edge Enabler Server. This proposal focuses on when the Edge Enabler Server exposes capability exposure API(s) which are compliant for interworking with existing 3GPP system feature.

2. Reason for Change

This paper proposes Location Service API exposed by the Edge Enabler Server to the Edge Application, based on the interworking with 3GPP network (e.g., NEF, SCEF). Location API can be used by the Edge Application to track or check the valid location of the UE for their location based services, e.g., in case of gaming, there will be specific region for playing certain instance. If the UE supports GPS, the application client can use the GPS information locally collected by the UE, and the application client can provide this information to the Application Server via application traffic. However, GPS spoofing can easily override the correct GPS information of the UE, so the GPS information collected by the application client may not be a valid location information. In addition, not all UE shall support GPS, for example, IoT devices, so there is no way to retreive the UE location information from the UE. Therefore, it is necessary to provide alternative way to check or track UE location without relying on the interaction with the UE. 3GPP network supports location reporting feature since release 12, and it is obvious that the UE using Edge Computing Service connects to the 3GPP network, so that utilizing 3GPP network feature for location reporting is feasible way.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.758 v0.1.0.

* * * First Change (All New Text)* * * *

7.X	Solution #X: Location Reporting API

7.X.1	Solution description

The following solution corresponds to the key issue #5 on Capability Exposure to Edge Application as specified in subclause 4.5, especially use case for the exposure of UE location information.

The Edge Enabler Server exposes Location Reporting API to the Edge Application in order to support tracking or checking the valid location of the UE. Location Reporting API exposed by the Edge Enabler Server is relying on the SCEFN/NEF northbound API for monitoring event of UE location.

The Edge Application can request Location Reporting API for one-time reporting to check current UE location. The Edge Application can also request Location Reporting API for continuous reporting to track UE location.

7.X.1.1	Procedure

Pre-conditions:

- The Edge Application is authorized to discover and to use Location Reporting API provided by the Edge Enabler Server

- Edge Enabler Server is authoized to use Nnef Event Exposure API for Location Reporting, based on SLA with MNO.

- UE Identifier between Edge Application and the Edge Enabler Server is authorized for the Location Reporting API.

- It is assumed there is user consent between the UE and the Edge Enabler Server for exposing location information of the UE to the Edge Application.

7.X.1.1.1	Request-Response model

Figure 7.X.1.1-1 illustrates the interactions between the Edge Enabler Server, the Edge Application, and 3GPP Network Function for one-time location reporting.

Figure 7.X.1.1-1: Solution X – Location Reporting API: Request-Response model

- The Edge Application requests Location Reporting API (UE Identifier, Location Granularity) to the Edge Enabler Server based on the decision from the Edge Application. The Edge Application shall include UE Identifier. Location granularity is optional parameter to indicate format of location e.g. GPS Coordinates, Cell ID, Tracking Area ID, or civic addresses (e.g. streets, districts, etc.), which can be understood by the Edge Application.

NOTE:	The trigger condition of the Location Reporting API is up to application service logic, which is out of scope of this specification.

Edtiror’s Note:	It is FFS how to determine UE Identifier which should be addressed by other solutions.

- The Edge Enabler Server reuqests Nnef_Event_Exposure service opreation to the NEF for location reporting as specified in TS 23.502 [xy].

If there is a request of Location Reporting from the Edge Application, the Edge Enabler Server determines to trigger Nnef_EventExposure_Subscribe operation for location reporting. If the request from the Edge Application includes the location granularity, the Edge Enabler Server can include the location granularity parameter in the Nnef_EventExposure_Subscribe operation.

Editor’s Note: It is FFS which UE Identifier the Edge Enabler Server uses for Nnef_Event_Exposure service operation for location reporting.

Upon receiving the request from the Edge Enabler Server, the NEF checks if the Edge Enabler Server is authorized to request location reporting monitoring event, and the NEF performs Monitoring Event operation for UE location. The NEF responds to the Edge Enabler Server with UE location.

Editor’s Note:	This step follows existing operation for location reporting as specified in TS 23.502[xy].

NOTE:	Even if there is no request from the Edge Application for Location Reporting, the Edge Enabler Server may determine to trigger the Nnef_EventExposure_Subscribe operation for location reporting in order to cache the latest UE location information continuously. The Edge Enabler Server can use the cached location information to respond to the Location Reporting API request from the Edge Application.

3.	The Edge Enabler Server caches locally the UE location information recevied in the step 2. The Edge Enabler Server may store the UE location with timestamp which indicates the time of receving the UE location in the step 2.

4.	If the Edge Enabler Server has received Location Reporting API request in the step 1, the Edge Enabler Server responds to the Edge Application with the UE location and timestamp.

The Edge Enabler Server may modify the format of location information to fit to the location granularity requested from the Edge Application in the step 1, if supported. For example, if the Edge Enabler Server receives the UE location from the NEF in a format of tracking area ID or cell ID, the Edge Enabler Server can modify the representation of the location information to the GPS Coordinates, or civic addresses (e.g. streets, districts, etc.) as requested by the Edge Application.

7.X.1.1.2	Subscribe-Notify model

Figure 7.X.1.1-2 illustrates the interactions between the Edge Enabler Server, the Edge Application, and 3GPP Network Function for continuous location reporting.

Figure 7.X.1.1-1: Solution X – Location Reporting API: Subscribe-Notify model

- The Edge Application requests Location Reporting subscribe operation (UE Identifier, Location Granularity) for trakcing the UE location continuously. The Edge Application shall include UE Identifier. Location granularity is optional parameter to indicate format of location e.g. GPS Coordinates, Cell ID, Tracking Area ID, or civic addresses (e.g. streets, districts, etc.), which can be understood by the Edge Application.

- The Edge Enabler Server reuqests Nnef_Event_Exposure service opreation to the NEF for location reporting as specified in TS 23.502 [xy]. The Edge Enabler Server requests continuous location reporting to the NEF to keep informed up to date UE location information in order to avoid repeatiton of location reporting request to the NEF whenever receives the request from Edge Applications.

If there is a request of Location Reporting from the Edge Application, the Edge Enabler Server determines to trigger Nnef_EventExposure_Subscribe operation for location reporting. If the request from the Edge Application includes the location granularity, the Edge Enabler Server can include the location granularity parameter in the Nnef_EventExposure_Subscribe operation.

Upon receiving the request from the Edge Enabler Server, the NEF checks if the Edge Enabler Server is authorized to request location reporting monitoring event, and the NEF performs Monitoring Event operation for UE location.

- The NEF responds to the Edge Enabler Server with UE location.

Editor’s Note:	Step 2 and Step 3 follow existing operation for location reporting as specified in TS 23.502[xy].

4.	The Edge Enabler Server caches locally the UE location information recevied in the step 3. The Edge Enabler Server may store the UE location with timestamp which indicates the time of receving the UE location in the step 3.

5.	The Edge Enabler triggers Location Reporting Notify service operation to the Edge Application which has subscribed Location Reporting. The Edge Enabler includes the lastest UE location information received from the NEF.

The Edge Enabler Server may modify the format of location information to fit to the location granularity requested from the Edge Application in the step 1, if supported. For example, if the Edge Enabler Server receives the UE location from the NEF in a format of tracking area or cell ID, the Edge Enabler Server can modify the representation of the location information to the GPS Coordinates, or civic addresses (e.g. streets, districts, etc.).

7.X.1.2	Service Operations

Table 7.x.1.2-1 describes the service operations of Location Reporting API provided by the Edge Enabler Server.

Table 7.x.1.2-1: Location Reporting Services provided by the Edge Enabler Server

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| EDGE3_LocationReporting | Subscribe | Subscribe/Notify | Edge Application |
|  | Unsubscribe |  | Edge Application |
|  | Notify |  | Edge Application |
|  | Request | Request/Response | Edge Application |

##### 7.X.1.2.1	EDGE3_LocationReporting_Subscribe operation

Service operation name: EDGE3_LocationReporting_Subscribe

Description: the consumer subscribes to receive a location reporting.

Inputs (required): target of location reporting (UE Identifier), Notification Target Address.

Inputs (optional): Location Granularity (e.g., format of range of GPS information, or civic addresses), Expiry time.

Outputs (required): The indication that the subscription is accepted, Expiry time (required if the subscription can be expired based on the local policy), and Subscription Correlation ID.

Outputs (optional): None.

##### 7.X.1.2.2	EDGE3_LocationReporting_Unsubscribe operation

Service operation name: EDGE3_LocationReporting_unsubscribe

Description: The consumer uses this service operation to unsubscribe the location reporting.

Input, Required: Subscription Correlation ID.

Input, Optional: None.

Output, Required: Operation execution result indication.

Output, Optional: None.

The consumer unsubscribes the location reporting subscription by invoking EDGE3_LocationReporting_Unsubscribe (Subscription Correlation ID) to the Edge Enabler Server.

7.X.1.2.3	EDGE3_LocationReporting_Notify operation

Service operation name: EDGE3_LocationReporting_Subscribe

Description: Provides the location reporting information to the Consumer which has subscribed to that event before.

Input, Required: Target of location reporting (UE Identifier), Location information, timestamp.

Input, Optional: Location Granularity (e.g., format of range of GPS information, or civic addresses)

Output, Required: None.

Output, Optional: None.

##### 7.X.1.2.4	EDGE3_LocationReporting_Request operation

Service operation name: EDGE3_LocationReporting_Requset

Description: the consumer requests to receive a location reporting for the UE.

Inputs (required): target of location reporting (UE Identifier), Notification Target Address.

Inputs (optional): Location Granularity (e.g., range of GPS information, or civic addresses)

Outputs (required): Target of location reporting (UE Identifier), Location information, timestamp.

Outputs (optional): Location Granularity (e.g., format of range of GPS information, or civic addresses)

7.X.2	Solution Evaluation

Editor’s Note:	To be added