3GPP TSG- SA WG6 Meeting #34	S6-192093

Reno NV, USA, November 11 – 15, 2019

Source:	Ericsson

Title:	pCR 23.758 – New solution

Document for:	Agreement

Agenda Item: 	11.5

Work Item / Release:	FS_EDGEAPP / Rel-17

Abstract of the contribution: This contribution proposes a solution that expands over several Key Issues

# 1	Introduction

This pCR proposes a solution where the discovery of the optimal Edge Aplication server is done on a per need basis, that is at the moment the Application Client requires it. With this proposal, the UE connects to the Edge DN only when the Edge DN applications servers are going to be used, and the Edge Application Server discovery can take into account the latest network status.

This solution does not require the UE to select and register in the Edge DN: there is no need to create and maintain a UE context in the EES and an Edge DN context in the EEC. Still, the EEC may keep Application server discovery information for efficiency. As a result, some of the key issues do not apply:

KIs #2 & #8: “Edge Data Network discovery and registration” and “Edge Data Network Selection”. They are Not Applicable. It is the optimal Edge Application Server that is selected. There is no Edge data network discovery and registration procedure as such that need to take place outside the actual Edge Application Server selection.

KI #10: “Dynamic availability”. Does not apply since it is the network (5GC, EEC, and actual AS) that handles this.

This pCR expands over a number of Key Issues, namely:

KI#1: “Service provisioning and configuration”.

KI #4: Edge Application Server discovery

KI #6: Edge Computing Service authorization

KI #7: “Flexible deployment”

KI#13: Provision of QoS information of the Edge Application Server

For the solution to be optimal the selection of the Edge Application Server needs to be paired with the 3GPP 5GC connectivity and the steering of the traffic of that application. This aspect has also been included in this pCR.

During the discussion on the ad-hoc meeting oct. 10, a chicken and egg issue raised for QoS. The QoS is set up for the UE to AS, i.e. a QoS flow may be established for this communication, and the EES or UE will trigger this prior UE starts communicating with the AS.

# 2	Proposal

## ***************** START CHANGES *****************

## 7.x	Solution #x: Per UE Stateless Edge enabling Service

This solution addresses Key Issues #1, #4, #6, #7 and #13. In this solution. The key characteristics of this solution is that the Edge enabling server does not keep any registration state of the UE, thus no need to solve KI #2. Furthermore, the EDGE 4 as defined in the architecture does not apply.

It is also assumed that it is the network side of the Edge Enabling Application Architecture that assigns a certain application server for a UE, and thus the UE does not need to do any selection among application servers, by this KI #8 is not needed.

### 7.x.1	Solution description

#### 7.x.1.1	Configuration of the UE

With regards to the Configuration of the UE, two models are assumed:

1.	The user downloads an edge data network application. From the user’s perspective, this is like any other app. The application itself contains necessary configuration data to run in an edge data network. The typical case for this model is that the Edge Data Network service is provided over the Internet DNN and slice.

2.	The UE is a specific device for edge data network services. Since it is a special device, it may already have the necessary application software and associated configurations, or the UE is configured to download the application software and associated configurations from a pre-configured site. The typical case for this model is that the Edge Data Network service is provided over a dedicated DNN and slice.

URSP Rules in 5GC is used to configure the UE’s usage of DNNs and slices.

This solves the KI#1: the above described mechanisms for models 1 and 2 and URSP rules provide all necessary configurations needed.

It is further assumed that necessary EEC functions are either part of the application (e.g. provided in a library in an SDK) or are provided by the operating system of the UE (also part of the SDK). This depends on OS.

By this, KI #7 is solved since each application will be tied to a certain edge data network.

#### 7.x.1.2	Edge Data network service execution

Pre-requisites:

UE has the edge application SW according to 7.x.1.1 Since the application SW and the OS in the UE act as the EEC and the internals of the UE is not assumed to be standardized, the flow below uses the term EEC from the architecture, but EEC is not really a separate thing. The interplay between application and OS to realize the EEC is a UE internal matter and is not to be standardized in 3GPP.

NOTE:	Prior to discovery, the UE does not register with the EES, thus the UE has no knowledge about the topology and existance of edge computing. It is only the application that may have a certain behaviour due to edge computing.

An Edge Application Server deployed in the Edge Hosting environement either registers in the EES (Key Issue #3) using for example, solution #12, where the solution includes Edge Enabler Server authorization check to verify whether the Edge Application Server has the proper privileges to register as part of the priocess. Or, the EES is provitioned with the ASes by means of O&M. The latter has no impact on “existing” cloud applications

If there is a need for a specific QoS class for the communication to the EES, then this is established as part of the PDU session establishment. Thus, this must be configured in the 5GC.

Figure 7.x.1.2-1 Edge Application Server Discovery

There is a trigger to execute an application (internal or by a user). If no established PDU session can be reused, the UE establishes a new PDU session to the corresponding DNN and slice. That session is used for the communication described below:

- The AS is identified by a FQDN, the AS-FQDN. The EEC does a discovery of an AS over EDGE-1 by sending a discovery request to the EES with the AS-FQDN as query parameter. There are examples how this is done in clause 7.x.1.2.1. The EES authenticates and authorizes the UE for this application, for example by using 3GPP credentials.

2.	If uplink classifier (UL-CL) and/or IPv6 multihoming are used to provide connectivity, the IP address may not be used to determine the Edge Application Server closest to the UE, this mean that the EES needs to get the UE location from the 5GC via e.g. NEF or PCF as per TS 23.502. E.g. using the Npcf_PolicyAuthorization_Subscribe service operation with immediate reporting flag set.

3.	 If UL-CL and/or IPv6 multihoming are to be used, the EES influences routing as per TS 23.502, see clause 4.3.6.3 step 2d.

4.	Depending on how the application is built, i.e. if UE or network should initiate QoS flow establishment and if a certain QoS flow is needed for the application. EES may initiate establishment of a QoS flow for this application as per TS 23.502, e.g as in 4.16.5.2. The EES may have gotten the QoS information when AS registers to EES

NOTE:	step 3 and 4 can happen after step 5.

5.	EES responds with the address of the AS.

6.	Depending on how the application is built, the UE may initiate a QoS flow establishment (see step 4), according to 23.502 clause 4.3.3.2 step 1a.

7.	Applicaton traffic starts between Application Client and Edge Application Server (over the QoS flow associated with the default QoS rule, or if QoS flow was established over the established QoS flow)

##### 7.x.1.2.1	Example Discovery methods

See two examples of Edge Applcation Server discovery using IETF protocols in the following sub-clauses

NOTE:	The SA2 WI FS_enh_EC has a KI for discovery. This can also result in more solutions for discovery of the Edge Application Server.

###### 7.x.1.2.1.1	HTTPS based protocol

The application/OS uses a HTTPS protocol (e.g. DNS over HTTPS) to discover the AS by using the AS-FQDN as a query parameter. A HTTPS connection is set up to the EES. The EES authenticates the UE and authorizes if UE is allowed to use this application as an edge application. How to authenticate (what client credentials to use) is an SA3 issue.

There are several ways how route the HTTPS connection to the EES:

-	the EES has an anycast address known by the application/OS in the UE. The IP network routes the request to the closest EES;

-	the EES has a FQDN, the EES-FQDN. The DNS in the 5GC has knowledge of the location of the UE and returns to the UE the address of the local EES; or

-	the EES has an EES-FQDN. The EES-FQDN is resolved to any EES. This EES resolves the AS location as follows:

1)	the EES may have the address of the AS for the UE location;

2)	the EES may use HTTP re-direct to an EES that can respond with the address of the AS in the location of the UE; or

3)	the EES may further interact with local EES (e.g. via normal DNS) to resolve the AS-FQDN and answer back to the UE using secured HTTP with the address of the AS.

###### 7.x.1.2.1.1	Using DNS

In this case it is assumed the the EES is operated by the operator and that the connection between the UPF and EES is secure. If the Internet DNN and slice are used, the EES forwards DNS request the Internet DNS Hierarchy for non-edge applications. There are a number of options:

- the EES may resolve the request and answer back with the AS for the UE Location; or

- the EES may forward the request to the EES that can resolve it for the UE location.

The EES authenticates the UE and authorizes if UE is allowed to use this pplication as an edge application. Since 5GC gives the UE an IP address and the connection between EES and UPF is secure, the EES can use the IP address to authenticate and authorize the UE. The EES can get location of the UE and directly query the site DNS for the address of the AS.

### 7.x.2	Solution Evaluation

This solution has minimum impact on any application in the UE. If DNS would be used and no QoS initiation in UE, any currently working application in a UE could be an Edge Application. If DoH is used and no QoS initiation in UE it has very minor impact on existing applications.

From 23.758, “Application Client portability: Changes in logic of Application Clients to interact with Edge Application Servers, compared to existing cloud environment, are avoided.”

The solution has solved and handled many of the key issues with a simplistic approach.

Therefore, it is proposed that this solution and solution #12 should be the bases for normative work ****************** END CHANGES ******************