3GPP TSG-SA WG6 Meeting #36 BIS-e meeting	S6-200436

# 31st March – 8th April 2020

Source:	InterDigital

Title:	Disc on Clarification of the definition of a UAS

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_UASAPP

# Discussion

It is currently not clear from the 3GPP documentation whether the UAS may consist of on one UAV-Controller and “one UAV” or “one or more UAVs”.

The definition of an UAS in 3GPP TS 22.125 is:

Unmanned Aerial System (UAS): Composed of Unmanned Aerial Vehicle (UAV) and related functionality, including command and control (C2) links between the UAV and the control station, the UAV and the network, and for remote identification. An UAS may comprise of a UAV and a UAV controller.

This approach, that a UAS consist of a UAV-C and a UAV, is documented in most part of 3GPP TS 22.125.

It can also be noted that in TS 22.125, Clause 4.1, a UAS is defined as the following:

An Unmanned Aerial System (UAS) is the combination of an Unmanned Aerial Vehicle (UAV), sometimes called a drone, and a UAV controller.

Similar definition was also used in most SA1 use cases, such as those captured in 3GPP TR 22.825 and 3GPP TR 22.829.

It was however at one point added an informative annex to 3GPP TS 22.125 which has been the root of some confusion. This annex states “an UAS is composed by one UAV controller and one or more UAV(s)”. This statement from the informative annex has been incorporated into parts of the stage 2 TRs 23.755 in SA6 and 23.754 in SA2 as a normative requirement.

It can also be noted that this informative annex contains normative requirements.

___

The UAV-Controller as an entity can control multiple UAVs. It is, however, questionable whether the definition of a UAS shall include more than one UAV. As 3GPP specify logical functions, it is correct to state that each UAV is controlled by an (instance of the) UAV-logical function.

Moreover, in the case of UAVs with autonomous flying capabilities i.e. using UTM-Navigated C2 communications the UAS may be defined as not comprising a UAV-Controller.

In a lot of governmental regulatory documents and industry literatures, we also observe that a UAS is invariably defined as a pair of UAV and its controller. A few examples are given here.

In “14 CFR 1.1” ( https://www.govinfo.gov/content/pkg/CFR-2019-title14-vol1/xml/CFR-2019-title14-vol1-sec1-1.xml):

Small unmanned aircraft system (small UAS) means a small unmanned aircraft and its associated elements (including communication links and the components that control the small unmanned aircraft) that are required for the safe and efficient operation of the small unmanned aircraft in the national airspace system.

FAA and DOT’s Proposed Rule for Remote Identification of Unmanned Aircraft Systems (https://www.federalregister.gov/documents/2019/12/31/2019-28100/remote-identification-of-unmanned-aircraft-systems) also use the UAS definition in 14 CFR 1.1.

In ARC’s recommendation for UAS Identification and Tracking,

Unmanned aircraft system (UAS) means an unmanned aircraft and associated elements (including communication links and the components that control the unmanned aircraft) that are required for the pilot in command to operate safely and efficiently in the national airspace system.

# Proposal

The definition of UAS introduced in SA6 and SA2, which allows more than one UAVs in a UAS, conflicts with the normative text in the stage 1 specification and the definition widely used by the industry or organizations outside 3GPP. The future work resulted from this definition may risk creating confusion and/or being incompatible with the USS/UTM eco-system outside 3GPP.

It is proposed that SA1 review 3GPP TS 22.125 and align the informative Annex A “UAS Reference Model” with the normative text. As a result of this, SA6 and SA2 can align the stage 2 accordingly.

To initiate this alignment, it is proposed that SA6 send an LS to SA1 and SA2. A draft LS on is available in S6-200437.

The corresponding changes to TR 23.755 v.0.6.0 are provided in a pCR in S6-200438.