3GPP TSG-SA WG6 Meeting #36-BIS-e	S6-200567

# E-meeting, 31st March – 8th April 2020											(revision of S6-200536)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0248 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Replacing the usage of MCPTT with MC Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT |  |  |  |  |  | Date: |  |  | 2020-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In some of the procedures MCPTT is being used which should be replaced with MC Service since TS 23.280 is for common services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replaced MCPTT with MC Service |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be inlined with the principle being followed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.7.2.12, 10.10.1.2.1, 10.10.1.2.4, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

#### 10.7.2.12	Group status notification

Table 10.7.2.12-1 describes the information flow for the group status notification from MC service server to MC service client.

Table 10.7.2.12-1: Group status notification

| Information element | Status | Description |
| --- | --- | --- |
| MC service group ID | M | The MC service group ID of the group |
| Status of the group | M | Indicate the group has on-going call or not |

* * * * * * * SECOND CHANGE * * * * * * *

##### 10.10.1.2.1	MC service emergency alert initiation

The procedure focuses on the case where an MC service client is initiating an MC service emergency alert unicast signalling for communicating the alert with the affiliated MC service group members of that MC service group. An MC service client in the MC service emergency state gains elevated access privilege for all of the MC service user's mission critical applications. This procedure will place the MC service client in the MC service emergency state if the MC service client is not already in that state.

Procedures in figure 10.10.1.2.1-1 are the signalling control plane procedures for the MC service client initiating an MC service emergency alert with an MC service group i.e., MC service users on MC service client 1, MC service client 2 and MC service client 3 belong to the same MC service group which is defined on group management server.

NOTE 1:	For simplicity, a single MC service server is shown in place of a user home MC service server and a group hosting MC service server.

Pre-conditions:

1.	The MC service group to be used for emergency communications by MC service client 1 is previously defined on the group management server and MC service client 2 and MC service client 3 are affiliated to that MC service group.

NOTE 2:	Alternatively, the client could have been provisioned for emergency behaviour on the currently selected group.

2.	All members of the MC service group belong to the same MC system.

3.	The initiating MC service client 1 is affiliated with one or more MC service groups.

4.	The initiating MC service client 1 may not have carried out an explicit affiliation procedure with the MC service group designated as the MC service emergency group.

5.	Optionally, MC service client 1 may use an activated functional alias for the group communication.

6.	The MC service server may have subscribed to the MC service functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 10.10.1.2.1-1 MC service emergency alert

1.	The MC service user at the MC service client 1 initiates an MC service emergency alert. MC service client 1 sets its MC service emergency state. The MC service user at MC service client 1 may select a functional alias used for the call. The MC service emergency state is retained by the MC service client 1 until explicitly cancelled.

2.	MC service client 1 requests the MC service server to send an MC service emergency alert request to the MC service group designated as the MC service emergency group.

3.	MC service server checks whether the MC service user of MC service client 1 is authorized for initiation of MC service emergency alerts for the indicated MC service group. The MC service server checks whether the provided functional alias, if present, can be used and has been activated for the user.

4.	MC service server resolves the MC service group ID to determine the members of that MC service group and their affiliation status, based on the information from group management server.

5.	The MC service server sends the MC service emergency alert response to the MC service user 1 to confirm the MC service emergency alert request. MC service group calls made by the MC service client 1 will be sent as emergency calls until the emergency state on the MC service client 1 is cancelled.

6.	The MC service server sends an MC service emergency alert request towards the MC service clients of each of those affiliated MC service group members. The MC service emergency alert request message shall contain the following information: Location, MC service ID and MC service group ID (i.e., MC service user's selected MC service group or dedicated MC service emergency group, as per user profile configuration) and the MC service user's mission critical organization name. If in step 2, the MC service client 1 does not include the location information in the MC service emergency alert request to the MC service server, the MC service server acquires the location information of the MC service user at the MC service client 1 from the location management server. If the location information is included in step 2, then the MC service server uses the location information from MC service client 1.

7.	MC service users are notified of the MC service emergency alert. The functional alias of the group call initiating MC service user may be displayed.

8.	The receiving MC service clients send the MC service emergency alert response to the MC service server to acknowledge the MC service emergency alert.

9.	The MC service server implicitly affiliates the client to the emergency group if the client is not already affiliated.

NOTE 3:	Sending the emergency alert without making a request to also start an emergency call does not put the group into the in-progress emergency state.

NOTE 4:	Sending the emergency alert does not put the other UEs in the group into an emergency state.

NOTE 5:	The MC service client 1 need not initiate a group call. For example, the MC service client can be configured to only allow alerts or the MC service user can choose not to make an MC service emergency group call.

NOTE 6:	It is configured whether MC service client 1 automatically triggers a subsequent MC service emergency group call.

Editor's note:	How emergency alert is synchronized between MC services is FFS.

* * * * * * * THIRD CHANGE * * * * * * *

##### 10.10.1.2.4	Leaving MC service emergency alert area

This procedure focuses on the case where a notification is sent when an MC service UE moves out of a pre-defined area. This procedure will notify the MC service client that it is now out of the pre-defined emergency alert area. It is up to the MC service user at that MC service client to determine if the emergency state is to be cleared or not.

Procedures in figure 10.10.1.2.4-1 are the signalling control plane procedures for the MC service server sends an MC service emergency alert area notification. It is optional if the MC service user wants to initiate the MC service emergency alert cancel procedure to inform the members in the MC service group that its emergency condition is over.

NOTE 1:	For simplicity, a single MC service server is shown in place of a user home MC service server and a group hosting MC service server.

Pre-conditions:

1.	The MC service group is previously defined on the group management server with MC service client 2 and MC service client 3 affiliated to that MC service group. This MC service group has been pre-defined for communicating the emergency alert in the user profile.

2.	All members of the MC service group belong to the same MC service system.

3.	An area in which the emergency alert for the MC service client should remain active is pre-defined in MC service server.

Figure 10.10.1.2.4-1 Leaving MC service emergency alert area

1.	MC service server acquires the latest location information of user at MC service client 1 from location management server as described in subclause 10.9.3.6.

2.	MC service server checks whether MC service client 1 has moved out a pre-defined area.

3.	If so, MC service server sends an MC service emergency alert area notification to MC service client 1 to notify that it has moved out of the area in potential danger.

4.	MC service client 1 notifies the MC service user about the MC service emergency alert area cancel. It is up to the MC service user at the MC service client 1 to determine if the emergency state is clear.

5.	Optionally, the MC service user of MC service client 1 initiates MC service emergency alert cancel procedure as described in 10.10.1.2.2.

NOTE 2:	It is up to the MC service user to clear the local emergency state on the client after the MC service UE has moved out of the emergency alert area, the MC service server cannot clear the client's local emergency state.

* * * * * * * END CHANGES * * * * * * *