3GPP TSG-SA WG6 Meeting #37-e	S6-200642

# e-meeting, 14th – 26th May 2020											(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0254 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On-demand location history reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS, BMWi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT |  |  |  |  |  | Date: |  |  | 2020-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently existing information flows and procedures do not support the on-demand request of locally stored location reports of an MC service user after returning from off-network operation. [R-5.11-009] in 3GPP TS 22.280 in clause 5.11 for on-network and off-network location reports based on triggered events. Clause 7.1 in 3GPP TS 22.280 describes in general that MC services available during off-network are functional comparable to MC services during on-network and this includes location management, as an essential feature of MC communication. While triggered location reports are available during on-network operation, the same or modified off-network triggers allow such MC service continuity while off-network operation. The location reports provided after returning to on-network operation. Use case #5, solutions #6 and #7 discussed in 3GPP TR 23.744. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New information flow and new procedure for the on-demand request of locally stored location reports after returning to on-network operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The operational analysis of location information triggered during off-network operation cannot be included into operative-tactical decisions after returning to on-network operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.2.12 (new), 10.9.3.9.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.9.2.12	Location information history request

Table 10.9.2.12-1 describes the information flow from the MC service server or location management client to the location management server and from the location management server to the location management client for the location history requesting.

Table 10.9.2.12-1: Location information history request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID list (see NOTE 1) | M | List of identities (e.g. MCPTT ID, MCData ID, MCVideo ID) of the MC service user from whom reports are requested |
| Reporting number of stored reports (see NOTE 2) | O | Indicates the number of stored reports to be sent |
| Reporting start time (see NOTE 2) | O | Indicates to send all reports having this start time and newer |
| Reporting end time (see NOTE 2) | O | Indicates to send all reports having this end time and older |
| Reporting triggering criteria (see NOTE 2) | O | Indicates to send all reports triggered by this condition |
| Minimum time between consecutive reports | O | Defaults to 0 if absent |
| NOTE 1: The MC service server may only use the MC service ID according to the MC service. NOTE 2: If none of these information elements is present, all stored location information reports shall be reported. Reporting number of stored reports shall not be present, if either reporting start time or reporting end time is present. |  |  |

* * * Next Change * * * *

##### 10.9.3.9.3	On-demand location history reporting procedure

NOTE 1: This procedure is valid for single MC system operation only.

The location management server can request off-network location reports at any time by sending a Location information history request to the location management client.

Figure 10.9.3.9.3-1 illustrates the procedure for the on-demand based usage of location history reporting.

Pre-conditions:

- The location management client 1 is configured to locally store location information report.

- The location management client 1 locally stored location information reports, based on triggered events.

- The location management client 1 marked these location information reports as off-network location information reports.

Figure 10.9.3.9.3-1: On-demand based usage of location history procedure

- The location management client 1 enters on-network operation, establishes connection to the location management server and enters the state to transmit location information reports.

- The location management client 1 provides the status of the available stored location information reports to the location management server.

- The location management server distributes the status to the MC service server and other entities like location management client 2, e.g. subscribed MC service user.

- The MC service server or the location management client 2 initiate location information history request to the location management server, based on the received status information.

- The location management server checks the authorization of this request, e.g. authorized MC service user.

- The location management server forwards the location information history request to the location management client 1.

- The location management client 1 decides to send one or several location information reports as determined by location management server (e.g. period of time, last N reports, all stored reports).

- The location management client 1 responds to the location management server with one or several off-network location information reports.

NOTE 2:	The transmission of requested off-network location information reports does not interrupt the on-network location information reports.

NOTE 3:	The last stored location information report is transmitted first.

NOTE 4:	The transmission may stopped at any time with the Cancel location history reporting procedure, as described in clause 10.9.3.9.4.

- The location management server updates the available location information with one or several off-network location information reports, e.g. by sending notifications to the subscribed MC servicer users or just to the on-demand requesting entity.

* * * Next Change * * * *

* * * End of Change * * * *