3GPP TSG-SA WG6 Meeting #37-e	S6-200682

# e-meeting, 14th – 26th May 2020											(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0214 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Pre-emption of EPS bearers by a new MCData bearer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData3 |  |  |  |  |  | Date: |  |  | 2020-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current text has some misleading and incomplete statements. Not only “other application” bearers but also other MCData bearers can be pre-empted by a new MCData bearer. Pre-emption can occur based on total capacity used up in the cell, not only based on number of bearers. It is not sufficient for a bearer to be higher priority in order to pre-empt other bearers: it has to also be allocated as pre-emption capable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarifying text addressing the points above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current text may lead to incorrect implementation, configuration and operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 5.8.3	EPS unicast bearer considerations for MCData

For an MCData session request, resources shall be requested utilising interaction with dynamic PCC. The MCData system shall request resources over Rx to a PCRF. The dedicated bearer for MCData media shall utilise the QCI value of 70 (as specified in 3GPP TS 23.203 [14]). The request of resources over Rx shall include an application identifier for MCData in order for the PCRF to evaluate the correct QCI.

The UE is required to support at minimum one bearer, which is used for MCData (see annex A in 3GPP TS 36.331 [15]).

Depending on operator policy, for media plane:

-	the MCData system may be able to request modification of the priority (ARP) of an existing bearer without the need to initiate a new dedicated GBR bearer; or

-	the allocation of EPS bearers of desired priority for MCData communications may cause the pre-emption of lower priority pre-emptible EPS bearers (for MCData or for other applications), if the maximum number of bearers or maximum traffic capacity has been reached, in favour of the newly initiated MCData EPS bearer. In this case, if the new EPS bearer to be used for MCData communication has higher priority level (ARP) than other bearer(s), is allocated with a capability to pre-empt other bearers and the other bearer(s) are pre-emptible, then the EPS bearer for MCData communication pre-empts one (or more) of the existing EPS bearer(s).

NOTE:	Operator policy takes into account regional/national requirements.

The EPS bearer(s) for MCData emergency communications shall have highest priority level among MCData communication types. The EPS bearer(s) for MCData imminent peril communications shall have higher priority level than for normal MCData communications, but lower than the priority level for MCData emergency communications.

* * * End of Change * * * *