3GPP TSG-SA WG6 Meeting #37-e	S6-200686

# e-meeting, 14th – 26th May 2020											(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 23.280 | CR | <0256> | rev | <Rev#> | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding equipment type in functional alias activation request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Trunking |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMONASTERY2 |  |  |  |  |  | Date: |  |  | 2020-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to TR22.289, it states that “These different types of FRMCS Equipment are relevant for the registration process of functional identities. For example a train driver cannot register to a functional identity using a FRMCS Equipment which is not fit for the train driver to perform his Role. So a train driver cannot perform a registration as train driver of train 123 if he uses a catering staff FRMCS Equipment (where there is no train emergency button available, no loudspeaker and handset, etc.). The registration request will be rejected by the FRMCS System.” Currently the functional alias activation procedure does not support FRMCS equipment type check. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding equipment type information in functional alias activation request for equipment type check by the MC service server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements of FRMCS are not fully satisfied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.13.2.3, 10.13.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * First Change * * * *

10.13.2.3	Functional alias activation request

Table 10.13.2.3-1 describes the information flow functional alias activation request from the MC service client to the MC service server.

Table 10.13.2.3-1: Functional alias activation request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the originator who triggers the functional alias activation request. |
| Functional alias list | M | A list of one or more functional aliases which the originator intends to activate. |
| Equipment type | O | The equipment type information of originator MC service UE which triggers the functional alias activation request. |

# * * * Next Change * * * *

10.13.4	MC service user activates functional alias(es) within an MC system

The procedure for MC service user activates functional alias(es) within an MC system is illustrated in figure 10.13.4-1.

Pre-conditions:

1.	MC service client has already been provisioned (statically or dynamically) with the functional alias(es) information that the MC service client is allowed to activate.

2.	MC service server may have retrieved the user subscription and functional alias policy e.g. which user(s) are authorized to activate to what functional alias, priority, and other configuration data.

3.	MC service client may have indicated to the functional alias management server that it wishes to receive updates of functional alias data for the functional aliases for which it is authorized.

4.	The MC service client triggers the functional alias activation procedure. This is an explicit activation caused either by the MC service user or determined by a trigger event such as the MC service UE coming within a permitted geographic operational area of a functional alias.

Figure 10.13.4-1: Functional alias activation procedure within an MC system

1.	MC service client of the MC service user requests the MC service server to activate a functional alias or a set of functional aliases. The equipment type information may be provided for equipment type check.

NOTE 1:	If the MC service server is not the one that performs the functional alias controlling role, the MC service server will forward the request to the MC service server that performs the functional alias controlling role.

2.	The MC service server checks if there are any conflicts with active functional alias(es).If equipment type information is provided, the MC service server checks if the functional aliases for activation are allowed by this equipment type.

3.	If the user of the MC service client is authorised to activate the requested functional alias(es) then the MC service server stores the functional alias(es) status of the requested functional alias(es).

If a certain functional alias(es) can be simultaneously active for multiple MC service users and the upper limit of number of simultaneous MC service users is not reached, the MC service shall activate the functional alias(es) for the MC service user and inform all other MC service user(s) with sharing the same functional alias(es) (step 5). If the limit of number of simultaneous MC service users is reached or the functional alias is not allowed to be shared, the request is rejected and the MC service user is notified (step 4).

If the functional alias(es) is (are) already used by another MC service user(s), an authorized MC service user gets an offer to take over the functional alias from the MC service user currently using the functional alias(es).

NOTE 2:	The functional alias status is notified to other servers (e.g. MC service servers) within the MC system that have subscribed to functional alias status.

4.	MC service server sends a functional alias(es) activation response to the MC service client.

5.	The MC service server informs all other MC service user(s) sharing the same functional alias(es).

# * * * Next Change * * * *