3GPP TSG-SA WG6 Meeting #37-e	S6-200720

# e-meeting, 14th – 26th May 2020											(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 221 | rev | <Rev#> | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the group FD upgrade and FD in-progress priority state cancel request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData3 |  |  |  |  |  | Date: |  |  | 2020-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Existing information elements table is generic and not clear |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added different information table so that it will be clear |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

##### 7.5.2.1.24	MCData group FD upgrade request

Table 7.5.2.1.24-1 describes the information flow for the MCData group FD upgrade request sent from the MCData client to the MCData server and from the MCData server to another MCData client.

Table 7.5.2.1.24-1: MCData group FD upgrade request (MCData client to MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending the upgrade request . |
| Functional alias | O | The associated functional alias of the MCData user sending data or receiving data. |
| MCData group ID | M | The MCData group ID on which the emergency upgrade request is made |
| Conversation Identifier | M | Identifies the conversation |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. |  |  |

Table 7.5.2.1.24-2: MCData group FD upgrade request (MCData server to MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending the upgrade request |
| Functional alias | O | The associated functional alias of the MCData user sending data or receiving data. |
| MCData group ID | M | The MCData group ID on which the emergency upgrade request is made |
| MCData ID | M | The identity of the MCData user receiving the upgrade request |
| Conversation Identifier | M | Identifies the conversation |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. |  |  |

* * * * * * * SECOND CHANGE * * * * * * *

##### 7.5.2.1.26	MCData group FD in-progress priority state cancel request

Table 7.5.2.1.26-1 describes the information for the MCData group FD in-progress priority state cancel request sent from the MCData client to the MCData server and from the MCData server to another MCData client.

Table 7.5.2.1.26-1: MCData group FD in-progress priority state cancel request (MCData client to MCData server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the cancelling MCData User |
| MCData group ID | M | The MCData group ID on which the MCData in-progress emergency state is to be cancelled. |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| Conversation Identifier | M | Identifies the conversation |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. |  |  |

Table 7.5.2.1.26-2: MCData group FD in-progress priority state cancel request (MCData server to MCData client)

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the cancelling MCData User |
| MCData group ID | M | The MCData group ID on which the MCData in-progress emergency state is to be cancelled. |
| MCData ID | M | The identity of the MCData user receiving the cancel request |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| Conversation Identifier | M | Identifies the conversation |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. |  |  |

* * * * * * * END CHANGES * * * * * * *