3GPP TSG-SA WG6 Meeting #38-e	S6-201135

e-meeting, 20th – 31st July  2020	(revision of S6-20xxxx)

Source:	Huawei, Hisilicon

Title:	Update to service provisioning

Spec:	3GPP TS 23.558 v0.3.0

Agenda item:	7.5

Document for:	Approval

Contact:	Niranth (namogh@huawei.com)

1. Introduction

The contribution provides a proposal for update to service provisioning.

2. Reason for Change

The service provisioning procdure is to be updated based on the following principled:

- The ECS provisions the EES information to the UE based on the relationship of the UE and applications.

- Based on operator policy, the ECS may provide more than one EES for a particular application related to the UE. This enables the UE to make a choice of which EES it should connect to avail the services of the application.

Also some errors need to be fixed.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.3.0.

* * * First Change * * * *

## 8.3	Service provisioning

### 8.3.1	General

Service Provisioning allows the Edge Enabler Clients to be configured with information about available edge computing services, based on the hosting UEs location, service requirements, service preferences and connectivity. This procedure provides Edge Enabler Clients with the necessary address information for establishing connections to Edge Enabler Servers.

If the ECS is deployed by MNO contracted with one or more ECSP(s) with its(their) own EES(s), the ECS provides one or more ECSP EES configuration information via MNO ECS as describes in clause 8.3.3.2.

If the ECS is deployed by non-MNO ECSP, the ECS endpoint address is pre-configured with the EEC. The EEC that is configured with multiple ECSP's ECS endpoint address(es) may perform the service provisioning procedure per the ECS of each ECSP multiple times.

The ECS can be configured with the information (e.g. UE identifier) of the UEs which it will serve.

Figure 8.3.1-1 illustrates an overview of service provisioning.

Figure 8.3.1-1: Overview of Service provisioning

The UE is initially provisioned with the configurations required to connect to the Edge Data Network. Upon initial provisioning, the Edge Enabler Client of the UE registers with the selected Edge Enabler Server(s) from the list of provisioned Edge Enabler Server(s). UE further consumes the edge computing services and performs various operations like Edge Application Server discovery, Edge application communications, Application context relocation, etc. While UE is consuming edge computing services, there may be several triggers which may cause service provisioning to be initiated by the UE or by the Edge Configuration Server.

The triggers for service provisioning are classified as:

a.	Triggers at UE - Some examples are:

-	Application Client related updates available at the Edge Enabler Client due to AC installation/re-installation, Application Client requesting application server access (e.g. via internet browser)

-	Edge Enabler Client supporting one or more Application Clients may be updated due to Edge Enabler Client re-installation.

b.	Triggers at Edge Configuration Server – An example is:

-	Edge Enabler Server updates due to Edge Application Server installation/re-installation/re-location, EDN/DNAI changes of the UE.

NOTE:	The DNAI changes can only be detected at the Edge Configuration Server if the Core Network type is 5GC.

Service provisioning is supported for a wide range of UEs. Such UEs may be highly capable of supporting any application or dedicated to a limited set of applications.

The determination of the Edge Enabler Servers for provisioning to the UE can based on:

-	matching Application Client Profiles when provided in the service provisioning request; or

-	the relationship of the requesting UE identifier and one or more supported Application Client profiles.

### 8.3.2	Procedure

Pre-conditions:

1.	The Edge Enabler Client has been pre-configured or discovered the address (e.g. URI) of the Edge Configuration Server.

2.	The Edge Enabler Client has been authorized to enable the communication with the Edge Configuration Server. The UE Identifier is either preconfigured or resulted from a successful authorization.

Editor's Note:	The authorization procedure required to enable communication with the Edge Configuration Server is FFS.

Figure 8.3.2-1: Service provisioning

1.	The Edge Enabler Client sends a provisioning request to the Edge Configuration Server. The request message may include the UE identifier such as GPSI and credentials received during EEC authorization procedure and Application Client Profile information.

2.	Upon receiving the request from the Edge Enabler Client, the Edge Configuration Server determines the UE identifier and performs an authorization check to verify whether the Edge Enabler Client has privileges to perform the operation. The ECS may utilize the capabilities (e.g. location) of the 3GPP core network as specified in clause 8.9.2. The Edge Configuration Server determines the Edge Enabler Server(s) matching Application Client Profiles if provided in step 1. If Application Client Profile(s) are not included in step 1, then the ECS determines a set of registered Edge Enabler Server(s) based on the relationship of the UE identifier and corresponding supported Application Client Profile(s). The Edge Configuration Server also determines other information that needs to be provisioned, e.g. identification of the Edge Data Network, service area information (for LADN), Edge Enabler Server endpoints.

3.	The Edge Configuration Server responds to the Edge Enabler Client by sending Edge services configuration information, e.g. identification of the Edge Data Network, service area information (for LADN), and the information for establishing a connection to the Edge Enabler Server (e.g. URI, IP address).

If the Edge Data Network configuration information includes an LADN DNN as an identifier for the Edge Data Network, the Edge Enabler Client considers the LADN as the Edge Data Network. Therefore, the service area of Edge Data Network is the LADN Service Area which can be discovered using the UE Registration Procedure.

### 8.3.3	Information flows

#### 8.3.3.1	Provisioning request

Table 8.3.3.1-1 describes the information elements for a Provisioning Request from the Edge Enabler Client to the Edge Configuration Server.

Table 8.3.3.1-1: Provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| EEC ID | M | Unique identifier of the EEC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Application Client Profile(s) | O | Information about services the EEC wants to connect to, as described in Table 8.2.2-1. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| UE location | O | The geographical location of the UE |
| Connectivity Filter | O | List of connectivity information for the UE, e.g. PLMN ID, SSID. |

Editor's Note: It is FFS whether the EEC ID is mandatory.

#### 8.3.3.2	Provisioning response

Table 8.3.3.2-1 describes the information elements for a Provisioning Response from the Edge Configuration Server to the Edge Enabler Client.

Table 8.3.3.2.-1: Provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of provisioning. |
| Reason (NOTE 2) | O | Reason for failure of provisioning (E.g. not authorized, application access not allowed at the UE location) |
| List of EES configuration info (NOTE 1) | O | List of EES(s) configuration information which can provide EAS(s) related to the UE. |
| > EDN connection info | M |  |
| >> DNN/APN | M | Data Network Name/Access Point Name |
| > EES Endpoint | M | The endpoint address (e.g. URI, IP address) of the EES |
| > List of EAS IDs | M | The list of EAS ID(s) of the EAS(s) related to the UE. |
| > EDN Service Area (NOTE 3) | O | Cell list, List of TA, PLMN IDs |
| > ECSP info | O | Information for Edge Computing Service Provider |
| NOTE 1: The List of EES configuration info IE is included when Result IE indicates success. NOTE 2: The Reason IE is included when Result IE indicates failure. NOTE 3: The EDN service area IE is included only if the EDN is a LADN. |  |  |