3GPP TSG-SA WG6 Meeting #38-e	S6-201136

e-meeting, 20th – 31st July  2020	(revision of S6-20xxxx)

Source:	Huawei, Hisilicon

Title:	Subscribe-Notify Service provisioning procedures

Spec:	3GPP TS 23.558 v0.3.0

Agenda item:	7.5

Document for:	Approval

Contact:	Niranth (namogh@huawei.com)

1. Introduction

The contribution provides a proposal for subscribe-notify service provisioning procedures.

2. Reason for Change

Two models for service provisioning procedures are supported:

-	Request/Response model; and

-	Subscribe/Notify model.

Subscribe-Notify procedures are introduced based on the principles discussed in S6-201135 – Update to service provisioning.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.3.0.

* * * First Change * * * *

## 8.3	Service provisioning

### 8.3.1	General

Service Provisioning allows the Edge Enabler Clients to be configured with information about available edge computing services, based on the hosting UEs location, service requirements, service preferences and connectivity. This procedure provides Edge Enabler Clients with the necessary address information for establishing connections to Edge Enabler Servers.

If the ECS is deployed by MNO contracted with one or more ECSP(s) with its(their) own EES(s), the ECS provides one or more ECSP EES configuration information via MNO ECS as describes in clause 8.3.3.2.

If the ECS is deployed by non-MNO ECSP, the ECS endpoint address is pre-configured with the EEC. The EEC that is configured with multiple ECSP's ECS endpoint address(es) may perform the service provisioning procedure per the ECS of each ECSP multiple times.

Two models for service provisioning procedures are supported:

-	Request/Response model; and

-	Subscribe/Notify model.

Figure 8.3.1-1 illustrates an overview of service provisioning.

Figure 8.3.1-1: Overview of Service provisioning

The UE is initially provisioned with the configurations required to connect to the Edge Data Network. Upon initial provisioning, the Edge Enabler Client of the UE registers with the selected Edge Enabler Server(s) from the list of provisioned Edge Enabler Server(s). UE further consumes the edge computing services and performs various operations like Edge Application Server discovery, Edge application communications, Application context relocation, etc. While UE is consuming edge computing services, there may be several triggers which may cause service provisioning to be initiated by the UE or by the Edge Configuration Server.

The triggers for service provisioning are classified as:

a.	Triggers at UE - Some examples are:

-	Application Client related updates available at the Edge Enabler Client due to AC installation/re-installation, Application Client requesting application server access (e.g. via internet browser)

-	Edge Enabler Client supporting one or more Application Clients may be updated due to Edge Enabler Client re-installation.

b.	Triggers at Edge Configuration Server – An example is:

-	Edge Enabler Server updates due to Edge Application Server installation/re-installation/re-location, EDN/DNAI changes of the UE.

NOTE:	The DNAI changes can only be detected at the Edge Configuration Server if the Core Network type is 5GC.

### 8.3.2	Procedures

#### 8.3.2.1	Service provisioning – Request/Response model

Pre-conditions:

1.	The Edge Enabler Client has been pre-configured or discovered the address (e.g. URI) of the Edge Configuration Server.

2.	The Edge Enabler Client has been authorized to enable the communication with the Edge Configuration Server. The UE Identifier is either preconfigured or resulted from a successful authorization.

Editor's Note:	The authorization procedure required to enable communication with the Edge Configuration Server is FFS.

Figure 8.3.2.1-1: Service provisioning – Request/Response model

1.	The Edge Enabler Client sends a provisioning request to the Edge Configuration Server. The request message may include the UE identifier such as GPSI and credentials received during EEC authorization procedure and Application Client Profile information.

2.	Upon receiving the request from the Edge Enabler Client, the Edge Data Network Configuration Server performs an authorization check to verify whether the Edge Enabler Client has privileges to perform the operation. The Edge Configuration Server determines the Edge Enabler Servers matching Application Client Profiles (or Application Client Profile IDs) if provided in step 1. The Edge Enabler Server also determines other information that needs to be provisioned, e.g. identification of the Edge Data Network, service area information, Edge Enabler Server endpoints.

3.	The Edge Configuration Server responds to the Edge Enabler Client by sending Edge services configuration information, e.g. identification of the Edge Data Network, service area information (for LADN), and the information for establishing a connection to the Edge Enabler Server (e.g. URI, IP address).

If the Edge Data Network configuration information includes an LADN DNN as an identifier for the Edge Data Network, the Edge Enabler Client considers the LADN as the Edge Data Network. Therefore, the service area of Edge Data Network is the LADN Service Area which can be discovered using the UE Registration Procedure.

#### 8.3.2.2	Service provisioning - Subscribe/Notify model

##### 8.3.2.2.1	Subscription

Figure 8.3.2.2.1-1 illustrates the service provisioning subscription procedure between the Edge Enabler Client and the Edge Configuration Server.

Pre-conditions:

1.	The Edge Enabler Client has been pre-configured or has discovered the address (e.g. URI) of the Edge Configuration Server.

2.	The Edge Enabler Client has been authorized to enable the communication with the Edge Configuration Server. The UE Identifier is either preconfigured or resulted from a successful authorization.

Figure 8.3.2.2.1-1: Service provisioning - Subscription

1.	The Edge Enabler Client sends a provisioning subscription request to the Edge Configuration Server. The request message may include the UE identifier such as GPSI and credentials received during EEC authorization procedure and Application Client Profile information.

2.	Upon receiving the request from the Edge Enabler Client, the Edge Configuration Server determines the UE identifier and performs an authorization check to verify whether the Edge Enabler Client has privileges to perform the operation. The ECS may utilize the capabilities (e.g. location, user plane path management) of the 3GPP core network as specified in clause 8.N.2. The Edge Configuration Server creates and stores a service provisioning subscription information (including information received in step 1) corresponding to the EEC and UE information.

3.	The Edge Configuration Server responds to the Edge Enabler Client by sending the subscription identification information. If immediate flag is included in step 1, then the Edge Configuration Server determines the Edge configuration information available at the ECS and sends to the Edge Enabler Client. The Edge Configuration Server determines the Edge Enabler Server(s) matching Application Client Profile(s), if provided in step 1. The Edge Configuration Server also determines other information that needs to be provisioned, e.g. identification of the Edge Data Network, service area information (for LADN), Edge Enabler Server endpoints (e.g. URI, IP address). If Application Client Profile(s) are not included in the request in step 1, then the ECS determines a set of registered Edge Enabler Server(s) based on the relationship of the UE identifier and corresponding supported application client profile(s).

Editor's note:	How the UE identifier and corresponding supported application client profile(s) is made available at the ECS is FFS?

Editor's note:	The procedures for subscription update and unsubscription are FFS.

##### 8.3.2.2.2	Notification

Figure 8.3.2.2.2-1 illustrates the service provisioning notification procedure between the Edge Enabler Client and the Edge Configuration Server.

Pre-conditions:

1.	The Edge Enabler Client has subscribed with the Edge Configuration Server for the provisioning information as specified in clause 8.3.2.2.1.

2.	The DNAI information related to the Edge Data Network is available at the Edge Configuration Server.

Figure 8.3.2.2.2-1: Service Provisioning - Notification

1.	When an event (e.g. DNAI change via user plane path management event notifications) that satisfies the triggering conditions for service provisioning occurs, the Edge Configuration Server may determine the Edge configuration information that needs to be provisioned to the Edge Enabler Client based on UE's location related information (e.g. DNAI) and the application related information in the service provisioning subscription information available at the ECS corresponding to the EEC and UE information.

2.	The Edge Configuration Server sends a provisioning notification to the Edge Enabler Client with the Edge configuration information available at the ECS. The Edge Configuration Server determines the Edge Enabler Server(s) matching Application Client Profile(s), if provided in the service provisioning subscription information. The Edge Configuration Server also determines other information that needs to be provisioned, e.g. identification of the Edge Data Network, service area information (for LADN), Edge Enabler Server endpoints (e.g. URI, IP address). If Application Client Profile(s) are not provided in the service provisioning subscription information, then the ECS determines a set of registered Edge Enabler Server(s) based on the relationship of the UE identifier and corresponding supported application client profile(s).

If the Edge Data Network configuration information includes an LADN DNN as an identifier for the Edge Data Network, the Edge Enabler Client considers the LADN as the Edge Data Network. Therefore, the service area of Edge Data Network is the LADN Service Area which can be discovered using the UE Registration Procedure.

### 8.3.3	Information flows

#### 8.3.3.a	Provisioning subscription request

Table 8.3.3.a-1 describes the information elements for a Provisioning Subscription Request from the Edge Enabler Client to the Edge Configuration Server.

Table 8.3.3.a-1: Provisioning subscription request

| Information element | Status | Description |
| --- | --- | --- |
| EEC ID | M | Unique identifier of the EEC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Application Client Profile(s) | O | Information about services the EEC wants to connect to, as described in Table 8.2.2-1. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Connectivity Filter | O | List of connectivity information for the UE, e.g. PLMN ID, SSID. |
| Event reporting mode | M | Mode of reporting - e.g reporting up to a maximum number of reports, periodic reporting along with periodicity, reporting up to a maximum duration |
| Maximum number of reports (NOTE) | O | Maximum number of reports after which the event subscription ceases to exist |
| Maximum duration of reporting (NOTE) | O | Maximum duration after which the event subscription ceases to exist |
| Immediate reporting flag | O | The ECS notifies the current status of the subscribed event, if available, immediately to EEC. |
| NOTE: Depending on Event reporting mode IE, atleast one of Maximum number of reports IE or Maximum duration of reporting IE is included, or both are included,. |  |  |

#### 8.3.3.b	Provisioning subscription response

Table 8.3.3.b-1 describes the information elements for a Provisioning Subscription Response from the Edge Configuration Server to the Edge Enabler Client.

Table 8.3.3.b.-1: Provisioning subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription Identifier | M | Subscription identifier for the corresponding Provisioning subscription request. |
| List of EES configuration info | M | List of EES configuration information which can provide EAS(s) related to the UE. |
| > EDN connection info | M |  |
| >> DNN/APN | M | Data Network Name/Access Point Name |
| > EES Endpoint | M | The endpoint address (e.g. URI, IP address) of the EES |
| > List of EAS IDs | M | The list of EAS ID(s) of the EAS(s) related to the UE. |
| > EDN Service Area (NOTE) | O | Cell list, List of TA, PLMN IDs |
| > ECSP info | O | Information for Edge Computing Service Provider |
| NOTE: The EDN service area IE is included only if the EDN is a LADN. |  |  |

Editor's note:	Whether and how EES profile information is used in provisioning subscription response is FFS.

#### 8.3.3.c	Provisioning notification

Table 8.3.3.c-1 describes the information elements for a Provisioning Notification from the Edge Configuration Server to the Edge Enabler Client.

Table 8.3.3.c.-1: Provisioning notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription Identifier | M | Subscription identifier for which the notification is generated. |
| List of EES configuration info | M | List of EES configuration information which can provide EAS(s) related to the UE. |
| > EDN connection info | M |  |
| >> DNN/APN | M | Data Network Name/Access Point Name |
| > EES Endpoint | M | The endpoint address (e.g. URI, IP address) of the EES |
| > List of EAS IDs | M | The list of EAS ID(s) of the EAS(s) related to the UE. |
| > EDN Service Area (NOTE) | O | Cell list, List of TA, PLMN IDs |
| > ECSP info | O | Information for Edge Computing Service Provider |
| NOTE: The EDN service area IE is included only if the EDN is a LADN. |  |  |