3GPP TSG-SA WG6 Meeting #38e	S6-201068 rev1

E-meeting, 20 – 31 July 2020

Source:	Samsung

Title:	ECS discovery

Spec:	3GPP TS 23.558 v0.3.0

Agenda item:	7.5

Document for:	Approval

Contact:	Jicheol Lee <jicheol.lee at samsung.com>

1. Introduction

There are multiple scenarios and multiple ways to configure the ECS address and configuration information. This paper discusses the potential scenarios with considerations and lists the potential ways to configure ECS configuration to the ECS.

ECS address information can be pre-configured with the EEC, configured by an edge-aware Application Client, configured by the user, provisioned by MNO through 5GC procedure or derived from HPLMN identifier for non-roming scenario or from VPLMN identifier for roaming scenario.

2. Reason for Change

There is no mechanism to configure the ECS for out-of-box scenario

3. Conclusions

It is proposed to describe multiple ECS configuration mechanisms

4. Proposal

Provide a new clause to describe “for ECS discovery” as a new clause under section 8.

## * * * First Change * * * *

## 8.X	Discovery of Edge Configuration Server

ECS address information can be pre-configured with the EEC, configured by an edge-aware Application Client, configured by the user, provisioned by MNO through 5GC procedure, or derived from HPLMN identifier for non-roming scenario or from VPLMN identifier for roaming scenario.

Editor’s Note: 5GC provision of ECS configuration information to the UE is in scope of SA2.

NOTE: How the ECS address information is configured to the EEC by the AC, user, or pre-configuration is out of scope of the present specification.

If the ECS configuration information is provided by 5GC, the EEC shall use the information for the initial provisioning request. Otherwise, the EEC shall use pre-configured ECS address for the initial provisioning if ECS configuration information is preconfigured with the EEC.

Editor’s Note: It is FFS whether ECS address can be derived from HPLMN or VPLMN identifier when no ECS address information is avaiable.

* * * End of Change * * * *