3GPP TSG-SA WG6 Meeting #38-e	S6-201116

e-meeting, 20th – 31st July  2020	(revision of S6-20xxxx)

Source:	Samsung Electronics, Intel, AT&T, Convida, Qualcomm, Sony

Title:	Pseudo-CR on EAS discovery

Spec:	3GPP TS 23.558 v0.3.0

Agenda item:	7.5

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

This contribution provides a clean-up of the EAS discovery procedures including updates to the related information flows.

2. Reason for Change

The current procedures are limited and do not fulfil the requirements.

Changes in the procedure include:

- reference to clause 8.9.3 for usage of 3GPP core network capabilities such as UE's location by the EES.

- alignment of the examples of information that can be included in the EAS discovery request and EAS discovery response with the information flows specified in clause 8.5.3.

Changes in the information flows include:

- Edge Application Server discovery request

> 'Query filters' IE is renamed to 'EAS discovery filters' to reflect the intent of the filters.

> 'Application Client IDs' IE is moved to the 'EAS discovery filters' IE to reflect the restructuring.

> 'EAS discovery filters' IE is restructured to reflect 2 categories of query filters the EEC can provide:

>> 'List of Application Client characteristics' IE which includes the 'Application Client profiles' IE; allowing an EEC to include the characteristics of the available Application Clients.

>> 'List of 'Edge Application Server characteristics' IE allowing an EEC to include several characteristics of the required EAS(s), assisting Edge Enabler Server to filter the information included in the EAS discovery response.

>> Only one of the above elements should be present in a EAS discovery request.

Changes to the Application Client profile and Edge Application Server's profile:

- Application Client profile:

> 'Application Client ID' is included as a mandatory IE allowing to discover EASs based on Application Client IDs, when Application Client profile is included in the EAS discovery request.

> 'Default EAS ID' is updated to 'List of EAS IDs' as an Application Client may require to connect to multiple EASs (for e.g. game server, matching server, advertisement server etc.)

> 'List of EAS IDs' IE is made optional to accommodate Application Clients that are edge-unaware.

> 'Application Client Type' IE is made optional to accommodate Application Clients that may not have a defined type.

- Edge Application Server profile:

> included 2 missing information elements being used for EAS discovery:

>> EAS service permission level

>> EAS Feature(s)

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.3.0.

* * * First Change * * * *

### 8.5.2	Edge Application Server Discovery

#### 8.5.2.1	General

Edge Application Server discovery enables the Edge Enabler Client to obtain information about available Edge Application Servers of interest. The Edge Enabler Client can provide EAS discovery filters to the Edge Enabler Server in order to assist the Edge Enabler Server in determining the Edge Application Servers of interest to the Edge Enabler Client..

Editor's Note: It is FFS how to select an EAS when the EEC discovered multiple EAS(s) from multiple ECSPs.

#### 8.5.2.2	Procedure

Pre-conditions:

1.	The Edge Enabler Client has received information (e.g. URI, IP address) related to the Edge Enabler Server.

Figure 8.5.2.2-1: Edge Application Server Discovery procedure

1.	The Edge Enabler Client sends an Edge Application Server discovery request to the Edge Enabler Server. The Edge Application Server discovery request includes the EEC ID along with the security credentials and may include EAS discovery filters to retrieve information of select Edge Application Servers.

2.	Upon receiving the request from the Edge Enabler Client, the Edge Enabler Server checks if the Edge Enabler Client is authorized to discover the requested Edge Application Server(s). The authorization check may apply to an individual Edge Application Server, a category of Edge Application Servers or to the Edge Data Network, i.e. to all the Edge Application Servers. If required, the EES may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.9.3. The Edge Enabler Server retrieves the Edge Application Server information based on UE's location. If the discovery request contained EAS discovery filters, then the Edge Enabler Server uses the EAS discovery filters to select the Edge Application Server information that matches the EAS discovery filters.

3.	The Edge Enabler Server sends an Edge Application Server discovery response to the Edge Enabler Client, which includes information about the discovered Edge Application Servers. For discovered Edge Application Servers, this includes endpoint information. Depending on the EAS discovery filters received in the Edge Application Server discovery request, the response may include additional information regarding matched capabilities, e.g. service permissions levels, Application Client locations(s) that the Edge Application Servers can support, KPIs, etc.

Upon receiving the Edge Application Server discovery response, the Edge Enabler Client uses the endpoint information for routing of the outgoing application data traffic to Edge Application Server(s), as needed, and may provide necessary notifications to the Application Client(s). The EEC may cache the EAS information (e.g. EAS endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the response, the Edge Enabler Client may cache the Edge Application Server information  only for the duration specified by the Lifetime IE.

NOTE:	Within the duration specified by the Lifetime IE, the cached EAS Profile can be updated (e.g. according to notifications from the Edge Enabler Server for changes of Edge Application Server information due to EAS status change) or the cached Edge Application Server Profile can be invalidated due to new Edge Application Server information discovery (e.g. due to UE mobility).

#### 8.5.2.3	Information flows

##### 8.5.2.3.1	Edge Application Server discovery request

Table 8.5.2.3.1-1 describes information elements for the Edge Application Server Discovery Request from the Edge Enabler Client to the Edge Enabler Server. Table 8.5.2.3.1-2 provides further detail about the Query Filter information element.

Table 8.5.2.3.1-1: Edge Application Server discovery request

| Information element | Status | Description |
| --- | --- | --- |
| EEC ID | M | Unique identifier of the Edge Enabler Client. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EAS discovery filters | O | Set of characteristics to determine required Edge Application Servers, as detailed in Table 8.5.2.3.1-2. |

Table 8.5.2.3.1-2: EAS discovery filters

| Information element | Status | Description |
| --- | --- | --- |
| List of Application Client characteristics (NOTE 1) | O | Describes the Application Clients for which a matching Edge Application Server is needed. |
| > Application Client profile (NOTE 2) | M | Application Client profile containing parameters used to determine matching EAS. Application Client profiles are further described in Table 8.2.2-1. |
| List of Edge Application Server characteristics (NOTE 1, NOTE 3) | O | Describes the characteristic of required Edge Application servers. |
| > EAS ID | O | Identifier of the required EAS. |
| > EAS provider identifier | O | Identifier of the required EAS provider |
| > EAS type | O | The category or type of required EAS (e.g. V2X) |
| > EAS schedule | O | Required availability schedule of the EAS (e.g. time windows) |
| > EAS service area | O | Location(s) (e.g. geographical area, route) where the EAS service should be available. |
| > Service continuity support | O | Indicates if the service continuity support is required or not. |
| > EAS status | O | Required status of the EAS (e.g. enabled, disabled, etc.) |
| > Service permission level | O | Required level of service permissions e.g. trial, gold-class |
| > Service feature(s) | O | Required service features e.g. single vs. multi-player gaming service |
| NOTE 1: Only one of the information elements shall be present. NOTE 2: 'Preferred ECSP list' IE shall not be present. NOTE 3: The ‘List of Edge Application Server characteristics’ IE must include at least one optional IE, if used as an EAS discovery filter. |  |  |

8.5.2.3.2	Edge Application Server discovery response

Table 8.5.2.3.2-1 describes information elements for the Edge Application Server Discovery Response from the Edge Enabler Server to the Edge Enabler Client.

Table 8.5.2.3.2-1: Edge Application Server discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Discovered EAS list | O | List of discovered EAS(s). Each element includes the information described below. |
| >EAS profile | M | Profile of the EAS. Each element is described in clause 8.2.4 |
| >Lifetime | O | Time interval or duration during which the information elements in the EAS profile is valid and supposed to be cached in the EEC (e.g. time-to-live value for an EAS Endpoint) |

Editor's Note:	The use of EAS Instance ID (including optionality) is dependent on how the ID is defined and assigned, which is FFS.

* * * Next Change * * * *

### 8.2.2	Application Client Profile

An Application Client Profile includes information about Application Client used to determine services and service characteristics required.

Table 8.2.2-1: Application Client Profile

| Information element | Status | Description |
| --- | --- | --- |
| Application Client ID | M | Identity of the Application Client. |
| List of EAS IDs | O | List of EAS Ids that serve the Application Client |
| Application Client Type | O | The category or type of Application Client (e.g. V2X). This is an implementation specific value. |
| Preferred ECSP list | O | The identity of Edge Computing Service Provider(s) whose associated EES(s), the Application Client or the EEC prefers to connect to. |
| Application Client Schedule | O | The expected operation schedule of the Application Client (e.g. time windows) |
| Application Client Service Area | O | The expected location(s) (e.g. route) of the hosting UE during the Application Client's operation schedule. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| Application Client Service KPIs | O | KPIs required in order for Application Clients to receive services form the EAS, as described in Table 8.2.3-1 |
| Service Continuity Support | O | Indicates if service continuity support is required or not for the application. |

* * * Next Change * * * *

### 8.2.4	Edge Application Server Profile

Table 8.2.4-1: Edge Application Server Profile

| Information element | Status | Description |
| --- | --- | --- |
| EAS ID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to Application Clients so that application clients can establish contact with the EAS. |
| Application Client ID(s) | O | Identifies the Application Client(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the EAS Provider |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Service Area | O | The geographical service area that the EAS serves |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS |
| Service continuity support | O | Indicates if the EAS supports service continuity or not. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heart beat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Required Service APIs | O | A list of the Service APIs that are required by the EAS |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |

Editor's Note: The inclusion of EAS Instance ID (including optionality) is dependent on how the ID is defined and assigned, which is FFS.

Editor's Note: The current text in the TS has the definition of a 'service area' IE associated with certain entities (such as the EAS Service Area in the table above). The following are FFS: (1) Which entities should have a 'service area' and (2) What is the relationship between the service areas of the different entities if any.