3GPP TSG-SA WG6 Meeting #39-e	S6-201472

e-meeting, 31st August – 8th September 2020	(revision of S6-20xxxx)

Source:	Samsung Electronics, Convida

Title:	Pseudo-CR on EEC registration – General updates

Spec:	3GPP TS 23.558 v0.4.0

Agenda item:	7.5

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

This contribution proposes several updates to EEC registration procedure.

2. Reason for Change

The EEC registration procedure lacked the following along with multiple editorial errors which are not fixed –

- A clear description of expiration time;

- Use of 3GPP core network capabilities y the Edge Enabler Server; and

- Registration ID in the response.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.4.0.

* * * First Change * * * *

### 8.4.2	Edge Enabler Client Registration

#### 8.4.2.1	General

An Edge Enabler Client performs registration with an Edge Enabler Server in order to use Edge Computing services offered by the Edge Enabler Server, e.g. discovering Edge Application Servers in an area of interest. The procedure also enables initialization or update of the Edge Enabler Client context information at the Edge Enabler Server. The context can be further used for edge-specific operations or processing, e.g. analytics.

Editor's Note:	It is FFS whether the Edge Enabler Client Registration may be considered optional. It is FFS to determine which EES functionality (e.g. discovery) needs to rely upon maintaining EEC context in the system.

Editor's Note:	It is FFS whether and which information is provided by the Edge Enabler Client to enable the Edge Enabler Server to select the way of interaction with 3GPP network (e.g. selecting NEF, SCEF, or NEF+SCEF) for a UE.

#### 8.4.2.2	Procedure

Pre-conditions:

1.	The Edge Enabler Client is authorized to access the Edge Enabler Server for the purpose of performing registration and has received relevant security credentials; and

Editor's Note: The required authentication and authorization procedures required to receive the relevant security credentials is FFS and in scope of SA3.

2.	The Edge Enabler Client has received service provisioning information from the Edge Configuration Server, including information for accessing the Edge Enabler Server.

Figure 8.4.2.2-1: Edge Enabler Client registration procedure

1.	The Edge Enabler Client sends Edge Enabler Client registration request to the Edge Enabler Server. The request from the client includes the security credentials received after successful authorization for edge computing services and may include a proposed expiration time. The request also optionally includes information indicating to the Edge Enabler Server how the Edge Enabler Client expects to use the services of the Edge Enabler Server.

The Edge Enabler Client registration request indicates if the request is a new registration or a registration update. A registration update may be used to send new or updated Application Client Profile(s) to the Edge Enabler Server.

If the Edge Enabler Client is moving from the purview of a source Edge Enabler Server to this Edge Enabler Server, the request from the Edge Enabler Client may include the identity of the source Edge Enabler Server and an Edge Enabler Client Registration Context ID that was provided by the source Edge Enabler Server.

2.	Upon receiving the request from the Edge Enabler Client, the Edge Enabler Server validates the registration request and verifies the security credentials. The Edge Enabler Server further determines whether the requirements that were indicated in the Application Client Profile(s) can be fulfilled.

3.	Upon successful validation of the request, if the received Registration request contains a Context ID and a source Edge Enabler Server Identifier, the Edge Enabler Server retrieves the Edge Enabler's Registration Context from the source Edge Enabler Server. Otherwise, this step is skipped.

4.	The Edge Enabler Server sends a successful registration response, which includes the registration ID and may include a newly assigned Context ID. The Edge Enabler Client stores the new Context ID and uses it if it needs to register with a new Edge Enabler Server later on. The Edge Enabler Server may also provide an expiration time to indicate to the Edge Enabler Client when the registration will automatically expire. To maintain the registration, the Edge Enabler Client shall send a registration update request prior to the expiration. If a registration update request is not received prior to the expiration time, the Edge Enabler Server shall treat the Edge Enabler Client as implicitly de-registered.

#### 8.4.2.3	Information flows

##### 8.4.2.3.1	Edge Enabler Client registration request

Table 8.4.2.3.1-1 describes information elements in the Edge Enabler Client registration request from the Edge Enabler Client to the Edge Enabler Server.

Table 8.4.2.3.1-1: Edge Enabler Client registration request

| Information element | Status | Description |
| --- | --- | --- |
| EEC ID | M | Unique identifier of the EEC. |
| Registration type indication | M | Indication for new or updated registration |
| UE Identifier | O | The identifier of the hosting UE (i.e. GPSI or identity token) |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Application Client Profile(s) | O | Profiles of Application Clients for which the EEC provides edge enabling services. Application Client Profiles are further described in Table 8.2.2-1. |
| Proposed expiration time | O | Proposed expiration time for the registration. |
| Context ID | O | Identifier of the EEC context obtained from a previous registration. |
| Context ID provider EES ID | O | If a Context ID is included, identifier of the EES that provided it. |
| EAS Identifiers | O | Identifiers of already discovered EAS which can support the services needed by the EEC. |

##### 8.4.2.3.2	Edge Enabler Client registration response

Table 8.4.2.3.2-1 describes information elements in the Edge Enabler Client registration response from the Edge Enabler Server to the Edge Enabler Client.

Table 8.4.2.3.2-1: Edge Enabler Client registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the registration request was successful. |
| > Registration ID | M | Identifier of the EEC registration. |
| > Expiration time | M | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. |
| > Authorization info | O | Information resulting from any authorization procedure performed by the EES based on the registration request, which can be used for future requests. |
| > Context ID | O | Identifier of the EEC Context information available at the EES that performed the registration. |
| EAS Information List | O | Optional information for the EASs specified in the request or those to which the client information is being forwarded. Each element may include the information detailed below |
| >>EAS ID | M | The identifier of the EAS |
| >>EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) that Application Clients use to send requests to the EAS |
| >>EAS Provider Identifier | O | The identifier of the EAS Provider |
| >>EAS Available Compute | O | The maximum compute resource available for the Application Client |
| >>EAS Available Graphical Compute | O | The maximum graphical compute resource available for the Application Client |
| >>EAS Available Memory | O | The maximum memory resource available for the Application Client |
| >>EAS Available Storage | O | The maximum storage resource available for the Application Client |
| Failure response | O | Indicates that the registration request failed. |
| > Cause | M | Provides the cause for registration request failure. |

Editor's Note:	It is FFS whether EAS Profiles should replace several of the IEs in the Discovered EAS List.