3GPP TSG-SA WG6 Meeting #39-e	S6-201475

e-meeting, 31st August – 8th September 2020	(revision of S6-20xxxx)

Source:	Samsung Electronics

Title:	Pseudo-CR on EEC registration – EEC context

Spec:	3GPP TS 23.558 v0.4.0

Agenda item:	7.5

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

This contribution provides several updates related to EEC context and its handling.

2. Reason for Change

Apart from multiple editorials fixes that are required, current specification does not clearly define the how the failure of EEC context retrieval is handled by the new Edge Enabler Server, and what are its impact on the EEC registration.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.4.0.

* * * First Change * * * *

#### 8.4.2.2	Procedure

Pre-conditions:

1.	The Edge Enabler Client is authorized to access the Edge Enabler Server for the purpose of performing registration and has received relevant security credentials

Editor's Note: The required authentication and authorization procedures required to receive the relevant security credentials is FFS and in scope of SA3.

2.	The Edge Enabler Client has received configuration information from the Edge Configuration Server, including information for accessing the Edge Enabler Server.

Figure 8.4.2.2-1: Edge Enabler Client registration procedure

1.	The Edge Enabler Client sends Edge Enabler Client registration request to the Edge Enabler Server. The request from the client includes the security credentials received after successful authorization for edge computing services. The request also optionally includes information indicating to the Edge Enabler Server how the Edge Enabler Client expects to use the services of the Edge Enabler Server.

The Edge Enabler Client registration request indicates if the request is a new registration or a registration update. A registration update may be used to send new or updated Application Client Profile(s) to the Edge Enabler Server.

If the Edge Enabler Client is moving to this Edge Enabler Server from the purview of another Edge Enabler Server, called source Edge Enabler Server, the request from the Edge Enabler Client may include the identity of the source Edge Enabler Server and an Edge Enabler Client context ID that was provided by the source Edge Enabler Server to maintain continuity of the EEC context.

2.	Upon receiving the request from the Edge Enabler Client, the Edge Enabler Server validates the registration request and verifies the security credentials. The Edge Enabler Server further determines whether the requirements that were indicated in the Application Client Profile(s) can be fulfilled.

3.	Upon successful validation of the request, if the received Edge Enabler Client registration request contains an EEC context ID and a source Edge Enabler Server ID, the Edge Enabler Server retrieves the Edge Enabler Client's context from the source Edge Enabler Server as specified in clause 8.4.x.y.z. Otherwise, this step is skipped.

4.	The Edge Enabler Server sends a successful registration response, which may include a newly assigned Edge Enabler Client context ID. The Edge Enabler Client stores the new Edge Enabler Client context ID and uses it if and when it registers with another Edge Enabler Server. If the Edge Enabler Client registration request contained Edge Enabler Client context ID, then the Edge Enabler Client registration response includes context retrieval result, indicating if the Edge Enabler Server was successful in retrieving the Edge Enabler Client context or not. The Edge Enabler Server also provides a registration expiration value to indicate when the registration will automatically expire. The Edge Enabler Client must send a registration request with an Update Indication prior to the expiration to maintain the current registration. The Edge Enabler Server will treat the registration expiration as an implicit de-registration. If the Edge Enabler Server is not able to fulfil, or provide, the services that are indicated by Application Client Profile(s), then the Edge Enabler Server will indicate this in the Edge Enabler Client registration response.

If the Edge Enabler Client registration response indicates that the Edge Enabler Client context retrieval failed, the Edge Enabler Client may reconstruct the context related associations with the Edge Enabler Server, such as subscriptions for information.

* * * Next Change * * * *

#### 8.4.x.y.z	EEC context retrieval

Figure 8.4.x.y.z-1 illustrates the procedure to retrieve EEC context from the source Edge Enabler Server.

Pre-conditions:

1.	The Edge Enabler Server has received the EEC context ID and the source EES ID from the EEC in the registration request.

Figure 8.4.x.y.z-1: EEC context retrieval procedure

1.	The Edge Enabler Server sends retrieve EEC context request to the source Edge Enabler Server. The request includes the EEC context ID.

2.	Upon receiving the request from the Edge Enabler Server, the source Edge Enabler Server validates the request. The Edge Enabler Server further determines the EEC context details that needs to be provided to the Edge Enabler Server.

3.	The source Edge Enabler Server sends a retrieve EEC context response including the EEC context available at the source Edge Enabler Server. The Edge Enabler Server stores the received EEC context and responds to the registration request from the Edge Enabler Client accordingly.

* * * Next Change * * * *

#### 8.4.2.3	Information flows

##### 8.4.2.3.1	Edge Enabler Client Registration request

Table 8.4.2.3.1-1 describes information elements in the Edge Enabler Client Registration Request from the Edge Enabler Client to the Edge Enabler Server.

Table 8.4.2.3.1-1: Edge Enabler Client Registration request

| Information element | Status | Description |
| --- | --- | --- |
| EEC ID | M | Unique identifier of the EEC. |
| Registration type indication | M | Indication for new or updated registration |
| UE Identifier | O | The identifier of the hosting UE (i.e. GPSI or identity token) |
| Security credentials | O | Security credentials resulting from a successful authorization for the edge computing service. |
| Application Client Profile(s) | O | Profiles of Application Clients for which the EEC provides edge enabling services. Application Client Profiles are further described in Table 8.2.2-1. |
| Proposed expiration time | O | Proposed expiration time for the registration. |
| Edge Enabler Client context ID | O | Identifier of the EEC context, obtained from a previous registration. |
| Source EES ID | O | Identifier of the EES that provided Edge Enabler Client context ID. |
| EAS Identifiers | O | Identifiers of already discovered EAS which can support the services needed by the EEC. |

##### 8.4.2.3.2	Edge Enabler Client Registration response

Table 8.4.2.3.2-1 describes information elements in the Edge Enabler Client Registration Response from the Edge Enabler Server to the Edge Enabler Client.

Table 8.4.2.2.2-1: Edge Enabler Client Registration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the registration operation |
| Expiration time | M | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. |
| Authorization info | O | Information resulting from any authorization procedure performed by the EES based on the registration request, which can be used for future requests. |
| Edge Enabler Client context ID | O | Identifier of the EEC Context information available at the EES that performed the registration. |
| Context retrieval result | O | Indicates if the Edge Enabler Client context was successfully retrieved from the source Edge Enabler Server or not. |
| EAS Information List | O | Optional information for the EASs specified in the request or those to which the client information is being forwarded. Each element may include the information detailed below |
| >EAS ID | M | The identifier of the EAS |
| >EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) that Application Clients use to send requests to the EAS |
| >EAS Provider Identifier | O | The identifier of the EAS Provider |
| >EAS Available Compute | O | The maximum compute resource available for the Application Client |
| >EAS Available Graphical Compute | O | The maximum graphical compute resource available for the Application Client |
| >EAS Available Memory | O | The maximum memory resource available for the Application Client |
| >EAS Available Storage | O | The maximum storage resource available for the Application Client |

Editor's Note:	It is FFS whether EAS Profiles should replace several of the IEs in the Discovered EAS List.

##### 8.4.2.3.a	Retrieve EEC context request

Table 8.4.2.3.a-1 describes information elements in the retrieve EEC context request from an Edge Enabler Server to the source Edge Enabler Server.

Table 8.4.2.3.a-1: Retrieve EEC context request

| Information element | Status | Description |
| --- | --- | --- |
| EES ID | M | Unique identifier of the EES. |
| EEC context ID | M | Identifier of the EEC context, received in the EEC registration request |

##### 8.4.2.3.b	Retrieve EEC context response

Table 8.4.2.3.b-1 describes information elements in the retrieve EEC context response from the source Edge Enabler Server to an Edge Enabler Server.

Table 8.4.2.2.b-1: Retrieve EEC context response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the retrieval request was successful. |
| > EEC context | M | EEC context details. |
| Failure response | O | Indicates that the retrieval request failed. |
| > Cause | O | Indicates the cause of retrieval request failure |