3GPP TSG-SA WG6 Meeting #39-e	S6-201490_rev1

e-meeting, 31st August – 8th September 2020	(revision of S6-201490)

Source:	Qualcomm Incorporated

Title:	Pseudo-CR on source EES-executed application context relocation

Spec:	3GPP TS 23.558 v0.4.0

Agenda item:	7.5

Document for:	Approval

Contact:	Eddy Hall

edhall@qti.qualcomm.com

1. Introduction

This pCR captures the Edge Enabler Layer execution procedures for source EES-executed application context relocation.

2. Reason for Change

Currently, there is no procedure specified for source EES executed application context relocation.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.4.0

* * * First Change * * * *

#### 8.8.2.x	Source EES executed application context relocation

Figure 8.8.2.x-1 illustrates the procedure for the S-EES to execute the application context relocation from the S-EAS to the T-EAS

Pre-condition:

1.	The AC at the UE already has a connection to the S-EAS.

2.	The EEC is able to communicate with the S-EES.

Figure 8.8.2.x-1: S-EES executed application context relocation procedure

1. Detection entities (EEC, S-EAS, S-EES) detects that application context relocation may be required.

Editor's Note: It is FFS whether the EEC or S-EAS performs T-EAS discovery and provides the S-EES with the information of the T-EAS and T-EES before informing the S-EES of the detected entities.

2. The detection entity informs the S-EES that application context relocation is required.

3. The S-EES decides to proceed required procedures for application context relocation.

Editor’s Note: Gathering consent from other EDGEAPP entities is FFS.

4. The S-EES determines T-EES and T-EAS either from provided information in step 2, or via the Discover target Edge Application Server procedure in clause 8.8.2.1 of the present document.

5. The S-EES sends the Application Context Relocation Request message to the S-EAS to initiate application context transfer between the S-EAS and the T-EAS.

6. The S-EAS transfers the application context to the T-EAS. This process is out of scope of the present specification.

7. The T-EAS sends the Application Context Relocation Complete message to the T-EES to confirm that the application context relocation has completed.

8. The S-EAS sends the Application Context Relocation Complete message to the S-EES to confirm that the application context relocation has completed.

9. The S-EES sends the Application Context Relocation Complete message to the EEC to confirm that the application context relocation has completed.

10. The EEC updates binding between AC and its corresponding EAS information.

Editor’s Note: Definition of the Information Flows in these procedures is FFS.

Editor’s Note: How the EEC modifies user plane connectivity for the AC is FFS.

* * * End of Changes * * * *