3GPP TSG-SA WG6 Meeting #39-e	S6-201654r1

e-meeting, 31st August – 8th September 2020	(revision of S6-201490)

Source:	Qualcomm Incorporated, Samsung, InterDigital

Title:	Pseudo-CR on source EES-executed application context relocation

Spec:	3GPP TS 23.558 v0.4.0

Agenda item:	7.5

Document for:	Approval

Contact:	Eddy Hall

edhall@qti.qualcomm.com

1. Introduction

This pCR captures the Edge Enabler Layer execution procedures for source EES-executed application context relocation.

2. Reason for Change

Currently, there is no procedure specified for source EES executed application context relocation.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.4.0

* * * First Change * * * *

#### 8.8.2.x	Source EES executed application context relocation

Figure 8.8.2.x-1 illustrates the procedure for the S-EES to decide and execute the application context relocation from the S-EAS to the T-EAS

Pre-condition:

1.	The AC at the UE already has a connection to the S-EAS.

2.	The EEC is able to communicate with the S-EES.

Figure 8.8.2.x-1: S-EES executed application context relocation procedure

1. Detection entities (EEC, S-EAS, S-EES) detects that application context relocation may be required.

Editor's Note: It is FFS whether the EEC or S-EAS performs T-EAS discovery and provides the S-EES with the information of the T-EAS and T-EES before informing the S-EES of the detected entities.

Editor's Note: It is FFS how detection entities (e.g., EEC, S-EAS, S-EES) detects the event which requires application context relocation.

2. The detection entity informs the S-EES that application context relocation is required.

3. The S-EES aurhorises the received message from step 2. The S-EES decides to execute application context relocation based on the information received in step 2 and the information of EEC context or EAS profile, and then proceed the below steps.

Editor's Note: Detailed decision criteria is FFS.

Editor’s Note: Gathering consent from other EDGEAPP entities is FFS.

4. The S-EES determines T-EES and T-EAS either from provided information in step 2, or via the Discover target Edge Application Server procedure in clause 8.8.2.1 of the present document. The S-EES may decide not to perform application context relocation if T-EAS is not available.

5. The S-EES sends the Application Context Relocation Notify message to the S-EAS to initiate application context transfer between the S-EAS and the T-EAS.

6. The S-EAS transfers the application context to the T-EAS at implementation specific time. This process is out of scope of the present specification.

7. The T-EAS sends the Application Context Relocation Complete message to the T-EES to confirm that the application context relocation has completed.

8. The S-EAS sends the Application Context Relocation Complete message to the S-EES to confirm that the application context relocation has completed.

9. The S-EES sends the Application Context Relocation Complete message to the EEC to confirm that the application context relocation has completed.

10. The EEC updates binding between AC and its corresponding EAS information.

Editor’s Note: How the EEC modifies user plane connectivity for the AC is FFS.

Editor’s Note: Definition of the Information Flows in these procedures is FFS.

Editor's Note: How the EES influences 3GPP Core Network in this procedure to modify user plane connectivity for the UE is FFS.

* * * End of Changes * * * *