3GPP TSG-SA WG6 Meeting #40-e	S6-202239

e-meeting, 16th – 24th November 2020	(revision of S6-202140)

Source:	Nokia, Nokia Shanghai Bell, BDBOS

Title:	Procedure refinements when the MC server performs connection authorization

Spec:	3GPP TR 23.700-79 version 0.2.0

Agenda item:	8.6

Document for:	Approval

Contact:	Martin Öttl, martin.oettl@nokia.com

1. Introduction

This paper is further elaborating the connection establishment procedure when the MC server performs connection authorization. Following changes were introduced:

-	Clarification that the GW MC service ID is pre-configured by the Mission Critical Organisation and used for connection authorization.

-	Clarification that the MC gateway UE does not authorize the connection request, it is the MC server.

-	Clarification that the GW MC service ID indicates the MC service and the MC gateway UE checks whether the MC service is supported.

-	Clarification that the MC gateway UE has performed service authorization for one or more MC services before receiving a request for connection.

2. Reason for Change

Adds more details to an existing procedure.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-79 version 0.2.0.

* * * Start of Change * * * *

## 7.3	Connection authorisation with an MC server via an MC gateway UE

### 7.3.1	General

This solution addresses the key issue 2 described in clause 5.2 on authorisation for connection of non-3GPP devices with an MC gateway UE. The solution only applies to non-3GPP devices which can host an MC client.

### 7.3.2	Information flows

#### 7.3.2.1	Connection authorization request

Table 7.3.2.1-1 describes the information flow connection authorization request sent from the MC client, which resides on a non-3GPP device, to the MC gateway UE, and from the MC gateway UE to the MC server.

Table 7.3.2.1-1: Connection authorization request

| Information element | Status | Description |  |
| --- | --- | --- | --- |
| GW MC service ID (see NOTE) | M | The GW MC service ID of the requesting MC service user. |  |
| NOTE: The GW MC service ID indicates for which MC service the connection is to be authorised. |  |  |  |

NOTE:	The MC service ID used for MC service authorisation and the GW MC service ID used for connection authorization may have different values. Both identities are configured by the Mission Critical Organisation.

#### 7.3.2.2	Connection authorization response

Table 7.3.2.2-1 describes the information flow connection authorization response sent from the MC server to the MC gateway UE, and from the MC gateway UE to the MC client residing on a non-3GPP device.

Table 7.3.2.2-1: Connection authorization response

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user. |
| Result | M | Success or failure of the connection authorization request (authorization successful/failed; service not supported). |

### 7.3.3	Procedure

The procedure for connection authorisation via an MC gateway UE towards an MC server is shown in figure 7.3.3-1.

Pre-conditions

-	The MC service user wishes to have access to MC services using a non-3GPP device.

-	The MC client has been configured with the necessary parameters needed for connectivity with the MC gateway UE.

-	The MC client has been provided with an appropriate GW MC service ID.

-	The MC gateway UE has performed service authorization for one or more MC services with the MC system.

Editor's Note:	The proper MC gateway UE configuration requires further study.

Editor's Note:	How the MC client discovers the MC gateway UE is FFS.

Figure 7.3.3-1: Connection authorisation with an MC server via an MC gateway UE

1.	The MC client requests connection authorization via the MC gateway UE with an MC server. The client of the MC service user provides the GW MC service ID.

2.	The MC gateway UE checks whether the requested MC service, as indicated by the GW MC service ID, is supported by the MC service gateway. If the MC service is supported, the procedure continues with step 3, otherwise the procedure proceeds with step 7.

3.	The MC gateway UE sends the connection authorization request to the MC server.

4.	The MC server performs an authorization check, to verify that access via the MC gateway UE is permitted. The MC server marks the MC service user as authorized to have access via the MC gateway UE.

Editor's Note:	The configuration of the MC server to perform the authorisation check needs further study.

Editor's Note:	Whether there is a need for control when the MC service user/client uses multiple MC gateway UEs simultaneously is FFS.

Editor's Note: It is FFS whether and how the MC server can mark the MC service user as allowed to have access before user authentication and service authorisation has been performed.

5.	The MC server sends the connection authorization response to the MC gateway UE.

6.	The MC gateway UE marks the MC client as authorized to have MC service access via the MC gateway UE.

7.	The MC gateway UE sends the connection authorization response to the MC client, confirming that the connection to the MC server via the MC gateway UE is authorized.

The MC client has now access to the MC server and may continue with user authentication and service authorization.

### 7.3.4	Solution evaluation

The MC gateway UE is not required to check whether the MC service user can have MC service access via the MC gateway UE.

Editor's Note:	This clause will provide the evaluation of this solution.

* * * End of Change * * * *