3GPP TSG-SA WG6 Meeting #41-e	S6-210068

e-meeting, 18th – 26th January 2021	(revision of S6-21xxxx)

Source:	Nokia, Nokia Shanghai Bell, UIC, BDBOS

Title:	Migration to another MC system during an ongoing group communication

Spec:	3GPP TR 23.700-90

Agenda item:	8.5

Document for:	Approval

Contact:	Martin Öttl (martin.oettl@nokia.com)

1. Introduction

This papers suggests a solution to address key issue 3 described in clause 5.3 on group communication between MC systems.

The solution provides the capability for an MC service user for migration to another MC system during an ongoing group call with minimal call break and without additional interaction to stay in the group.

2. Reason for Change

Support for smoothless migration within an ongoing group communication.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-90 version 0.1.0.

* * * First Change * * * *

## 7.x	Migration during an ongoing group communication

### 7.x.1	General

This solution addresses the key issue 3 described in clause 5.3 on group communication between MC systems.

The solution provides the capability for an MC service user for migration to another MC system during an ongoing group communication and to remain in the group communication.

### 7.x.2	Solution description

#### 7.x.2.1	Procedure

The procedure is based on the following existing procedures:

-	MC service user receiving MC service from a partner MC system described in TS 23.280 [5] clause 10.1.4.3.2.

-	Exiting group call due to de-affiliation described in TS 23.379 [2] clause 10.6.2.3.2.

-	Late entry pre-arranged group call described in TS 23.379 [2] clause 10.6.2.3.1.1.4.

NOTE 1:	The solution is about ongoing group calls but may be applicable for ongoing private calls too.

NOTE 2: The solution is about MCPTT group calls but is applicable for other services too.

Pre-conditions:

- The MCPTT client has one or more ongoing group calls in the primary MC system.

- The MCPTT client has migrated into the partner MC system.

Figure 7.x.2.1-1:	Migration to partner MC system during ongoing group calls

Modifications compared to MC user profile migration in TS 23.280 [5] clause 10.1.4.3.2:

1.	After the Primary MC system has retrieved MC user profile for migration, the Primary MC system determines the ongoing group calls of the migrating MCPTT user.

2.	If there are any ongoing group calls for the migrating MCPTT user, the Primary MC system sends a group call leave request to the MCPTT client to indicate that the MCPTT user is de-affiliated from ongoing group call(s) provided in the Primary MC system. The group call leave request indicates that de-affiliation happens due to an MCPTT user migration to avoid sending an indication of de-affiliation to the MCPTT user.

3.	The Primary MC system sends all ongoing group call IDs in which the migrated MCPTT user is participating to the Partner MC system as part of the get migrated MC user profile response message.

4.	The Partner MC system shall interrogate group calls in its database and check whether those are ongoing in the Partner MC system as well.

5.	If any of the received group calls are ongoing in the Partner MC system, the Partner MC system shall initiate a late-entry procedure towards the MCPTT client.

6.	The MCPTT client indicates the successful migration to the MCPTT user.

NOTE 3: If there is no ongoing group calls in the Primary MC system, the Primary MC system will not provide any group ID to the Partner MC system and current procedure as described in TS 23.280 [5] clause 10.1.4.3.2 is applied.

#### 7.x.2.2	Messages

Editor's Note:	This clause will provide the detailed information flows.

### 7.1.3	Solution evaluation

Editor's Note:	This clause will provide the evaluation of this solution.

* * * End of Change * * * *