3GPP TSG-SA WG6 Meeting #41-e	S6-210097

e-meeting, 18th – 26th January 2021	(revision of S6-21xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 257 | rev | <Rev#> | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Deposit an Object procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | at&t |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData3 |  |  |  |  |  | Date: |  |  | 2021-01-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The procedure in 7.13.3.8 is not correct; the steps are not in the right order. The response message back to the MCData server should only occur after the FD file is retrieved and stored locally in the MCData message store. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Move step 2 in the procedure to the last step as the response message should only go back to the MCData server once all the operations are done. - Update the figure and affected text accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The guidance to stage 3 development will be wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.13.3.8.1, 7.13.3.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.13.3.8	Deposit an object

##### 7.13.3.8.1	General

MCData server needs to store the communication information (i.e. an object) for a MCData user during an active MCData communication. If there is a file URL in the object for file distribution in the communication, the MCData server may instruct the MCData message store to retrieve a copy of the file to store locally in the MCData user’s account.

##### 7.13.3.8.2	Procedure

The procedure in figure 7.13.3.8.2-1 describes how the MCData server deposit an object into the MCData message store during an active MCData communication.

Pre-conditions:

1.	A successful authentication and authorization has been performed between the MCData server and the MCData message store.

2.	The MCData user has been allocated a secured storage area in the MCData message store.

3.	MCData user has requested to store his MCData communication and also store the distributed file content into his MCData message store account if the MCData communication is for file distribution through URL.

Figure 7.13.3.8.2-1: Deposit an object

1.	The MCData server would like to deposit a MCData communication information (i.e. object) to the MCData user's storage area in the MCData message store and initiates a MCData deposit an object request toward the MCData message store. The object is constructed by the MCData server and is included in the request. If the object is a message that carries a URL for file distribution, the MCData server may instruct the MCData message store to retrieve a copy of the file and store locally in the MCData user's account by setting the retrieve file indication information element to true.

2.	The MCData message store deposits the object into the MCData user's storage area. If the retrieve file indication is set in the MCData deposit an object request the MCData message store retrieves the file URL from the stored object and fetches the file content from the MCData content server.

3.	The MCData message store stores the file content into the MCData user's storage area and update the object with the URL referencing the file content stored in the MCData user's storage area.

4.	The MCData message store communicates the result back to the MCData server in the MCData deposit an object response. The object identifier of the stored object is returned