3GPP TSG-SA WG6 Meeting #41-e	S6-210107

e-meeting, 18th – 26th January 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0038 | rev | <Rev#> | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service identification in location management procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2021-01-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As addressed in subclause 9.1 of TS 23.434: "The location management is a SEAL service that offers the location management related capabilities to one or more vertical applications", when the consumer of the location management service (e.g. a VAL UE or a VAL server) is using multiple VAL services (identified by the VAL service ID of each), it is necessary to identify what specific service the location management procedure is for. The reasoning for distinguishing the services for location management: - The use of VAL user ID or VAL UE ID in the location reqeusts is not nessisarily considered that a specific VAL service will authorize the VAL user or UE for using the requested location functions; - Different VAL services have different location privacy policies towards the same VAL UE/user or the same VAL server; potential security and legal risks exist when the location of the same UE is shared among different VAL services; - The legitimacy of location reporting or fetching of VAL UE, VAL user, and the location reporting/fetching of VAL user for a VAL service are often achieved by separate authorizations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add VAL service ID in the location management information flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risks exist for location information to be achieved by unauthorized services or parties. The location management services cannot distinguish VAL services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.0, 9.3.2.1, 9.3.2.2, 9.3.2.3, 9.3.2.4, 9.3.2.5, 9.3.2.6, 9.3.2.7, 9.3.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.2.0	Location reporting configuration request

Table 9.3.2.0-1 describes the information flow from the location management client to the location management server for requesting the location reporting configuration.

Table 9.3.2.0-1: Location reporting configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or identity of the VAL UE. |
| VAL service ID | O | Identity of the VAL service for which the location reporting configuration is requested. |

* * * Next Change * * * *

9.3.2.1	Location reporting configuration response

Table 9.3.2.1-1 describes the information flow from the location management server to the location management client for the location reporting configuration. This information flow may be sent individually addressed or group addressed on unicast or multicast.

Table 9.3.2.1-1: Location reporting configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or VAL group to which the location reporting configuration is targeted or identity of the VAL UE. |
| VAL service ID | O | Identity of the VAL service for which the location reporting configuration is requested. |
| Requested location information | O (NOTE) | Identifies what location information is requested |
| Triggering criteria | O (NOTE) | Identifies when the location management client will send the location report |
| Minimum time between consecutive reports | O (NOTE) | Defaults to 0 if absent otherwise indicates the time interval between consecutive reports |
| NOTE: If none of the information element is present, this represents a cancellation for location reporting. |  |  |

* * * Next Change * * * *

9.3.2.2	Location information report

Table 9.3.2.2-1 describes the information flow from the location management client to the location management server for the location information reporting or from the location management server to the requesting location management client or VAL server to report location information.

Table 9.3.2.2-1: Location information report

| Information element | Status | Description |
| --- | --- | --- |
| Set of identities | M | Set of identities of the reporting VAL users or VAL UEs |
| VAL service ID | O | Identity of the VAL service for which the location information is reported. |
| Triggering event | M | Identity of the event that triggered the sending of the report |
| Location Information | M | Location information |

* * * Next Change * * * *

9.3.2.3	Location information request

Table 9.3.2.3-1 describes the information flow from the VAL server to the location management server and from the location management server to the location management client for requesting an immediate location information report.

Table 9.3.2.3-1: Location information request

| Information element | Status | Description |
| --- | --- | --- |
| Identity list | M | List of VAL users or VAL UEs whose location information is requested |
| VAL service ID | O | Identity of the VAL service for which the location information is requested. |

* * * Next Change * * * *

9.3.2.4	Location reporting trigger

Table 9.3.2.4-1 describes the information flow from the location management client or VAL server to the location management server for triggering a location reporting procedure.

Table 9.3.2.4-1: Location reporting trigger

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M (NOTE 1) | Identity of the requesting authorized VAL user or VAL UE |
| Identity | M (NOTE 1) | Identity of the requested VAL user or VAL UE |
| VAL service ID | O | Identity of the VAL service for which the location reporting trigger is set. |
| Immediate Report Indicator | O (NOTE 2) | Indicates whether an immediate location report is required |
| Requested location information | O (NOTE 2) | Identifies what location information is requested |
| Triggering criteria | O (NOTE 2) | Identifies when the client will send the location report |
| Minimum time between consecutive reports | O (NOTE 2) | Defaults to 0 if absent otherwise indicates the interval time between consecutive reports |
| Endpoint information | O | Information of the endpoint of the requesting VAL server to which the location report notification has to be sent. It is provided if Immediate Report Indicator is set to required. |
| NOTE 1: The identity of the requesting VAL user/UE and the requested VAL user/UE should belong to the same VAL service. NOTE 2: At least one of these rows shall be present. |  |  |

* * * Next Change * * * *

9.3.2.5	Location information subscription request

Table 9.3.2.5-1 describes the information flow from the VAL server to the location management server for location information subscription request.

Table 9.3.2.5-1: Location information subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE |
| Identities list | M | List of VAL users or VAL UEs whose location information is requested. |
| VAL service ID | O | Identity of the VAL service for which the location information is subscribed. |
| Time between consecutive reports | M | It indicates the interval time between consecutive reports |

* * * Next Change * * * *

9.3.2.6	Location information subscription response

Table 9.3.2.6-1 describes the information flow from the location management server to the VAL server for location information subscription response.

Table 9.3.2.6-1: Location information subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE |
| VAL service ID | O | Identity of the VAL service for which the location information is subscribed. |
| Subscription status | M | It indicates the subscription result |

* * * Next Change * * * *

9.3.2.7	Location information notification

Table 9.3.2.7-1 describes the information flow from the location management server to the VAL server or the location management client.

Table 9.3.2.7-1: Location information notification

| Information element | Status | Description |
| --- | --- | --- |
| Identities list | M | List of the VAL users or VAL UEs whose location information needs to be notified |
| Identity | M | Identity of the VAL user or VAL UE subscribed to location of another VAL user or VAL UE (NOTE) |
| VAL service ID | O | Identity of the VAL service for which the location information is notified. |
| Triggering event | M | Identity of the event that triggered the sending of the notification |
| Location Information | M | Location information |
| NOTE: This is only used for location management server sends location information notification to the VAL user or VAL UE who has subscribed the location. |  |  |

* * * Next Change * * * *

9.3.2.8	Location reporting configuration cancel request

Table 9.3.2.8-1 describes the location reporting configuration cancel request information flow from the location management client or VAL server to the location management server.

Table 9.3.2.8-1: Location reporting configuration cancel request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting authorized VAL user or VAL UE |
| Identity | M | Identity of the requested VAL user or VAL UE |
| VAL service ID | O | Identity of the VAL service for which the location reporting configuration is requested to be canceled. |