3GPP TSG-SA WG6 Meeting #41-e	S6-210123

e-meeting, 18th – 26th January 2021	(revision of S6-21xxxx)

Source:	Ericsson

Title:	Pseudo-CR on MCVideo functional model using the 5GS

Spec:	3GPP TS 23.289 version 0.2.0

Agenda item:	7.5

Document for:	Approval

Contact:	Camilo Solano (camilo.solano@ericsson.com)

1. Introduction

The MCVideo functional model using the 5G system is introduced with a focus on supporting unicast transmissions.

2. Reason for Change

The MCVideo functional model using the 5G system is introduced.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to approve the following changes to 3GPP TS 23.289 version 0.2.0.

* * * First Change * * * *

## 5.x	MCVideo functional model description

### 5.x.1	On-network functional model

Figure 5.x.1-1 shows the functional model for the application plane for an MCVideo system using the 5GS.

Figure 5.x.1-1: MCVideo functional model for application plane

In the functional model shown in figure 5.x.1-1, the following is considered:

-	The description of the corresponding functional entities and reference points in 3GPP TS 23.281 [4] applies.

-	The description of the MCVideo-4 and MCVideo-7 reference points in 3GPP TS 23.281 [4] applies considering that it utilizes the N6 reference point defined in 3GPP TS 23.501 [7].

-	The description of the MCVideo-5 reference point in 3GPP TS 23.281 [4] applies considering that it exists between the MCVideo server and the 5GS. It is used for MCVideo resource management, e.g. QoS control, and utilizes the N5 and Rx reference points as defined in 3GPP TS 23.501 [7].