3GPP TSG-SA WG6 Meeting #41-e	S6-210117

e-meeting, 18th – 26th January 2021	(revision of S6-21xxxx)

Source:	Samsung

Title:	Pseudo-CR on High level feature description

Spec:	3GPP TS 23.558 v1.2.0

Agenda item:	7.6

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

This contribution provides text for the high level features clause.

2. Reason for Change

The high level features clause is currently empty.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v1.2.0.

* * * First Change * * * *

# 4	Overview

4.1	General

For edge computing, it is essential that the Application Clients locate and connect with the most suitable application server on the edge, depending on the needs of the application. The edge enabler layer exposes APIs to support such capabilities

The edge computing capabilities supported by 3GPP are illustrated in the figure 4.1-1.

Figure 4.1-1: Overview of 3GPP edge computing

The application layer is a consumer of 3GPP specified edge computing capabilities. The 3GPP edge computing capabilities are typically organized as follows:

-	Edge enabler layer, specified in this specification;

-	Edge hosting environment, details of which are outside the scope of 3GPP;

-	3GPP transport layer, specified in 3GPP TS 23.682 [17] and 3GPP TS 23.501 [2]; and

-	Edge management layer.

Editor's note:	The reference to the specification for edge management layer is FFS.

Following clauses provide an overview of the features of edge enabler layer.

## 4.2	Service provisioning

Service provisioning procedures supply the information required by the UE to access the edge services. The procedure takes UEs location, service requirements, service preferences and connectivity information into account to provide the required configuration. Service provisioning procedures support the following models, specified in clause 8.3:

-	Request-response; and

-	Subscribe-notify.

## 4.3	Registration

Registration procedures specified in clause 8.4, allow entities in the edge enabler layer to provide information about itself to other entities of the edge enabler layer.

## 4.4	EAS discovery

EAS discovery procedures enable the UE to obtain information about available Edge Application Servers of interest. The procedure takes into account the discovery filters provided in the request. EAS discovery procedures support the following models, specified in clause 8.5:

-	Request-response;

-	Subscribe-notify; and

-	Dynamic information subscription.

## 4.5	Capability exposure to EAS

The edge enabler layer exposes services towards the Edge Application Servers. The exposed capabilities include the services of the Edge Enabler Layer and the services of the 3GPP core network accessible directly or via the edge enabler layer. The capabilities exposed by the edge enabler layer are specified in clause 8.6 and the 3GPP network capability exposure is specified in clause 8.7.

4.6	Support for service continuity

When a UE moves to a new location, different Edge Application Servers can be more suitable for serving the UE. Such transitions can result from a non-mobility event also, requiring support from the edge enabler layer to maintain the continuity of the service. Support for service continuity includes features for Application Clients in the UE to minimize service interruption while replacing the serving Edge Application Server, called source Edge Application Server, with a target Edge Application Server. Support for service continuity is further specified in clause 8.8.

## 4.7	Utilizing 3GPP core network capabilities

The functional entities of the edge enabler layer can utilize the 3GPP core network capabilities (i.e. 5GC, EPC) to fulfill the needs of the edge service operations. Clause 8.9 specifies the details of the 3GPP core network capabilities consumed by each functional entity.

## 4.8	Security

The edge enabler layer supports secure communication amongst the enabler layer entities. Clause 8.10 provides details on EEC authentication and authorization.

## 4.9	Dynamic EAS instantiation

The Edge Enabler Layer can interact with the EAS management system to trigger instantiation of the Edge Application Server. Details of the EAS instantiation triggering are specified in clause 8.11.

## 4.x	<High level feature>

Add a copy of this clause for each high level feature, adding a title and description.