3GPP TSG-SA WG6 Meeting #41-e	S6-210122

e-meeting, 18th – 26th January 2021	(revision of S6-21xxxx)

Source:	Samsung

Title:	Pseudo-CR on New annex for mapping with GSMA OP

Spec:	3GPP TS 23.558 v1.2.0

Agenda item:	7.6

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

The contribution provides the relationship between the EDGEAPP architecture and the reference architecture of GSMA OP.

2. Reason for Change

SA6 received LS from GSMA OPG in S6-210013, informing about the publication of the GSMA OP whitepaper - Operator Platform Telco Edge proposal. The LS described the purpose of the whitepaper and sought information on existing or future work being done in 3GPP that can support the development of the Operator Platform.

This contribution documents the parity between the two architectures in an informative annex.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v1.2.0.

* * * First Change * * * *

Annex x (Informative):
Relationship with GSMA OPG

## x.1	General

GSMA Operator Platform Group has published a GSMA Whitepaper OPG.01 "Operator Platform: Telco Edge Proposal”. The Operator Platform aims to expose capabilities in operator networks to 3rd party developers and applications. This annex provides a high level mapping between the architecture for enabling edge applications defined in clause 6 with the Operator Platform's reference architecture defined in GSMA Whitepaper OPG.01 [xx].

## x.2	Relationship between EDGEAPP architecture and GSMA OPG reference architecture

Editor's Note:	More details are FFS.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedure for the 5G System; Stage 2".

[4]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[5]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[6]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[7]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[8]	3GPP TS 36.305: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[9]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[10]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[11]	3GPP TS 23.003: "Numbering, addressing and identification".

[12]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[13]	3GPP TS 23.434: "Service enabler architecture layer for verticals; Functional architecture and information flows; Stage 2".

[14]	3GPP TS 23.286: "Application layer support for Vehicle-to-Everything (V2X) services; Functional architecture and information flows ".

[15]	ETSI ISG MEC ETSI GS MEC 003 V2.1.1 (2019-01), "Multi-access Edge Computing (MEC); Framework and Reference Architecture"

[16]	OMA-TS-REST_NetAPI_ACR: "RESTful Network API for Anonymous Customer Reference Management"

[17]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[18]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[xx]	GSMA Whitepaper OPG.01: "Operator Platform: Telco Edge Proposal”, https://www.gsma.com/futurenetworks/wp-content/uploads/2020/10/GSMA-Operator-Platform-Proposal-Oct-2020.pdf

[yy]	3GPP TR 28.814: "Study on enhancements of edge computing management".

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Application Client

ACID	Application Client Identification

AF	Application Function

ASP	Application Service Provider

DN	Data Network

DNAI	Data Network Access Identifier

DNN	Data Network Name

EAS	Edge Application Server

EASID	Edge Application Server Identification

ECS	Edge Configuration Server

ECSP	Edge Computing Service Provider

EDN	Edge Data Network

EEC	Edge Enabler Client

EECID	Edge Enabler Client Identification

EES	Edge Enabler Server

EESID	Edge Enabler Server Identification

EHE	Edge Hosting Environment

FQDN	Fully Qualified Domain Name

GPSI	Generic Public Subscription Identifier

GSMA	Global System for Mobile Communications

LADN	Local Area Data Network

NEF	Network Exposure Function

OP	Operator Platform

SCEF	Service Capability Exposure Function

SSID	Service Set Identifier

TAI	Tracking Area Identity