3GPP TSG-SA WG6 Meeting #41-e	S6-210xxx

e-meeting, 18th – 26th January 2021	(revision of S6-210094)

Source:	Ericsson

Title:	Edge deployment for FFAPP

Spec:	3GPP TR 23.745 v1.2.0

Agenda item:	8.2

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution provides solution for FFAPP in edge deployment.

2. Reason for Change

The existing solution #5 and #6 put FF enabler (client & server) as EAS in the context of EDGEAPP architecture. It is still unclear how the FF specific application (client & server) can utilize the EDGEAPP service.

Rev1:

Updated to remove description related to sol#4.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.745.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; Stage 1".

[4]	IEEE 802.1Qbv-2015: "Standard for Local and Metropolitan Area Networks-Media Access Control (MAC) Bridges and Virtual Bridged Local Area Networks Amendment: Enhancements for Scheduled Traffic".

[5]	IEEE 802.1AS-2011: "IEEE Standard for Local and Metropolitan Area Networks - Timing and Synchronization for Time-Sensitive Applications in Bridged Local Area Networks".

[6]	IEEE 802.1Q-2018: "IEEE Standard for Local and Metropolitan Area Networks—Bridges and Bridged Networks".

[7]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[8]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows;"

[9]	oneM2M TS-0001: "Functional Architecture".

[10]	3GPP TS 23.558: "Architecture for enabling Edge Applications (EA)".

[11]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[12]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[13]	3GPP TR 23.700-20: "Study on enhanced support of Industrial Internet of Things (IIoT) in the 5G System (5GS)".

[14]	3GPP TS 29.549: "Service Enabler Architecture Layer (SEAL); Application Programming Interface (API) specification; Stage 3".

[15]	3GPP TS 23.503 : "Policy and charging control framework for the 5G System (5GS); Stage 2".

[16]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs".

[17]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[18]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[19]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[20]	OPC 10000-1: "OPC Unified Architecture Specification Part 1: Overview and Concepts".

[21]	OPC 10000-6: "OPC Unified Architecture - Part 6: Mappings".

[22]	OPC 10000-14: "OPC Unified Architecture - Part 14: PubSub ".

[23]	DIN SPEC 91345:2016-04: "Referenzarchitekturmodell Industrie 4.0 (RAMI4.0)".

[24]	3GPP TS 22.263: "Service requirements for video, imaging and audio for professional applications (VIAPA); Stage 1".

[25]	IEEE Std 802.1Qcc-2018: "Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[26]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[27]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

[28]	IETF RFC 8323: "CoAP (Constrained Application Protocol) over TCP, TLS, and WebSockets".

[29]	IETF RFC 7959: "Block-Wise Transfers in the Constrained Application Protocol (CoAP)".

[30]	IETF RFC 7641: "Observing Resources in the Constrained Application Protocol (CoAP)".

[31]	IETF RFC 6690: "Constrained RESTful Environments (CoRE) Link Format".

[32]	IETF RFC 8613: "Object Security for Constrained RESTful Environments (OSCORE)".

[33]	IETF draft-ietf-ace-oauth-authz-35: "Authentication and Authorization for Constrained Environments (ACE) using the OAuth 2.0 Framework (ACE-OAuth)".

[34]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[35]	3GPP TS 23.003: "Numbering, addressing and identification".

[36]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[rfc6749]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[TS33434]	3GPP TS 33.434: "Service Enabler Architecture Layer (SEAL); Security aspects for Verticals".

* * * Next Change * * * *

## 7.X	Solution #XX (merging Sol#5, #6): Edge computing for FFAPP

### 7.X.1	Solution description

This solution corresponds to the key issue #9 - communication service on the Edge deployments and merges sol#5 and sol#6 with a complete analysis.

The EDGEAPP architecture is specified in 3GPP TS 23.558 [10]. The EDGE-5 reference point details are not specified in Rel-17.

Figure 7.x.1-1 illustrates the edge deployment example for the FFAPP. For simplicity, the reference points between enabler server and 5GS are omitted, and the reference points for inter-enabler server communication in the same enabler layer are also omitted. At UE side, FF Application Specific client(s) and FAE client interact with the Edge Enabler Client (EEC) via EDGE-5 reference point. In an Edge Data Network (EDN), the Edge Application Server (EAS), e.g. FF Application Specific Server and FF Application Enabler server, interacts with the Edge Enabler Server (EES) via EDGE-3 reference point, for instance, to register its profile into the EES. The EEC interacts with the Edge Configuration Server (ECS) via EDGE-4 reference point, for instance, to discover candidate EES(s). The EEC interacts with the EES via EDGE-1 reference point, for instance, to discover candidate EAS(s) (e.g. FF Application Specific Server and FF Application Enabler Server) and provide the discovered EAS(s) to the Application Client (e.g. FF Application Specific client and FF Application Enabler client).

Figure 7.x.1-1: FFAPP in EDGE deployment

In an EDN, there could be several EES(s) provided by the same or different ECSP. The FFAS sever(s) and FAE server shall be able to discover and register into an appropriate EES. If CAPIF is used, this can be done by utilizing the AEF serving area and/or the AEF location as described in 3GPP TS 23.222 [16]; otherwise, local configuration of the EES endpoint may be used.

For SEAL, most management entities have no association between the VAL server and the SEAL servers due to loosely administrative procedure specified in 3GPP. But for SEAL Identity Management (IM) function, before the SEAL IM client is granted with any token from the SEAL IM server, the SEAL IM client needs to register in the SEAL IM server as described in 3GPP TS 33.434 [TS33434]. If there is any interaction between the VAL server and SEAL IM server for token validation purpose (see IETF RFC 6749 [rfc6749]), the EAS discovery needs to take into consideration for the association between the VAL server (e.g. FAE server) and the SEAL IM server.

In order to enable the selection of a VAL server (e.g. FFAS server or FAE server) and its associated SEAL IM server, in an edge deployment, the VAL server and the SEAL IM server need to exchange its application server information. After both sides stored the associated application server information, the VAL server and SEAL IM server register in the EES via EDGE-3 reference point. And consequently, the EEC, based on the interaction with SEAL IM client and/or VAL client (e.g. FFAS client or FAE client) over EDGE-5 reference point, can discover a list of candidate SEAL IM server(s) including its associated VAL server(s) and a list of candidate VAL server(s) including its associated SEAL IM server(s).

In the UE, there are several ways to achieve the selection of associated EAS servers, below list describes two alternatives for information purpose since both SEAL-C and EDGE-5 are not detailed within Rel-17 timeline:

a.	The VAL client discovers the VAL server(s) and the SEAL IM client discovers the SEAL IM server(s) separately via EDGE-5 reference point. The final selection of the VAL server and the SEAL IM server relies on the coordination between the VAL client and SEAL IM client via SEAL-C reference point.

b.	The VAL client can also delegate the discovery of VAL server to the SEAL IM client via SEAL-C reference point, and then the SEAL IM client triggers the discovery (possibly with the request to discover SEAL IM server together) and further selects the VAL server matching the selected SEAL IM server from the discovered result.

According to 3GPP TS 23.558 [10], the EEC may narrow down the number of discovered application server(s) when providing the discovery result to the application client. If such function in the EEC is used (e.g. being delegated by the application client), the EEC shall not narrow down the number of discovered application servers solely based on a single application which has associated EAS information.

Table 7.x.1-1 shows the impact in the EAS profile which includes a list of associated EAS information (in bold and italic). For instance, in the profile of VAL server, the SEAL IM server is provided as parent EAS. In the profile of SEAL IM server, one or more VAL servers are provided as child EAS.

Table 7.x.1-1: Edge Application Server Profile

| Information element | Status | Description |
| --- | --- | --- |
| EAS ID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to Application Clients so that application clients can establish contact with the EAS. |
| … | … | … |
| A list of associated EAS information | O | A list of associated EAS(s) including EAS application ID, its endpoint information and the relationship (parent, child or sibling). |

Note that the other EDGE-3 exposure services are not re-exposed by the FAE server or SEAL servers to the FFAS server, for instance, the FFAS server directly consumes location or QoS API provided by the EES.

### 7.X.2	Solution evaluation

This solution addresses KI#9 for "How to support FFAPP communications over Edge deployments" and provides a deployment option for the FF application in the EDGE application architecture.

In such deployment, if the SEAL IM function is used and there is a relationship between the VAL server and SEAL IM server with respect to the token validation process, the associated EAS information needs to be provided during EAS registration procedure and the UE needs to select the suitable application servers with joint consideration in the associated application server.

In general, this solution can be applied for applications other than FFAPP. Within edge deployment as described in 3GPP TS 23.558 [10], if the application enabler client in the UE and application specific server need to communicate with the same application enabler server, the selection of the application specific server and enabler server needs to have a consolidated view for the associated application servers