3GPP TSG-SA WG6 Meeting #43	S6-211246

e-meeting, 24th May – 2nd June 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0066 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SEAL Location Deviation Service Information flows and APIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP |  |  |  |  |  | Date: |  |  | 2021-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per conclusions in TR 23.755, for the KI#12 (Track UAV location deviation), it is concluded that the solution #18 (Monitor UAV location deviation) as the way forward for the normative phase and SEAL should be enhanced with this new solution. This CR proposes the information flows and the APIs of Location Deviation Monitoring as new SEAL LMS service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Monitoring Location Deviation information flow and APIs are defined as new SEAL LMS service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Monitoring UAV location as concluded in UASAPP work, will be unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.X (new), 9.3.2.Y (new), 9.3.2.X (new), 9.4.1, 9.4.X (new), 9.4.X.1 (new), 9.4.X.2 (new), 9.4.X.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 9.3.2.X	Monitor Location Subscription Request

Table 9.3.2.X-1 describes the information flow from the VAL server to the LM server for monitoring the location deviation of the VAL UE(s).

Table 9.3.2.X-1: Monitor Location Subscription Request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identifier of the VAL users or VAL UE whose location monitoring is requested to be monitored in a given location. |
| Area of Interest | M | Geographic area location information where the VAL server wishes to monitor the VAL UE’s location adherence. |
| Notify Interval | M | Periodic time interval in which the LM server needs to notify the VAL UE’s location information to the VAL server. |
| Notification Target URI | M | Target URI where the VAL server wishes to receive the notifications about VAL UE’s location. |
| Timeout | O | A timeout period when subscription response is not received. |

#### 9.3.2.Y	Monitor Location Subscription Response

Table 9.3.2.Y-1 describes the information flow from LM server to the VAL server for Monitor Location Subscription Response.

Table 9.3.2.Y-1: Monitor Location Subscription Response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result |

#### 9.3.2.Z	Notify Location Monitoring Event

Table 9.3.2.Z1-1 describes the information flow from LM server to the VAL server for notification of location monitoring events.

Table 9.3.2.Z-1: Notify Mismatch Location message

| Information element | Status | Description |
| --- | --- | --- |
| Event | M | Information of the event to be reported. The event shall be one of the following: "Notify_Mismatch_Location" – When the location information of the VAL UE, from the location management client and the core network do not match. "Notify_Location_Absence" – When the VAL UE’s current location is deviating from the VAL server’s area of interest information. "Notify_Presence" – When the VAL UE’s current location is within the VAL server’s area of interest information. |
| Identity | M | Identifier of the VAL UE whose location information is reported. |
| Location | M | Current location of the VAL UE. |

* * * Next Change * * *

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting | VAL server | Request /Response |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Monitoring_Events | VAL server |  |

* * * Next Change * * *

### 9.4.X	SS_LocationMonitoring API

#### 9.4.x.1	General

API description: This API enables the VAL server to monitor the VAL UE’s location in a given area of interest, from the location management server over LM-S.

#### 9.4.x.2	Subscribe_Location_Monitoring

API operation name: Subscribe_Location_Monitoring

Description: Subscription to location monitoring.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.X

Outputs: 9.3.2.Y.

See subclause 9.3.X.2 for the details of usage of this API operation.

#### 9.4.x.3	Notify_Location_Monitoring_Events

API operation name: Notify_Location_Monitoring_Events

Description: Notifying the VAL server with the location monitoring events related to VAL UE.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.Z

Outputs: None.

See subclause 9.3.X.2 for the details of usage of this API operation.

* * * End of	 Changes * *