3GPP TSG-SA WG6 Meeting #43-e	S6-211330

e-meeting, 24th May – 2nd June 2021	(revision of S6-211146)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0294 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of temporary regroup procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions, FirstNet, Home Office, Sepura Ltd, Softil, L3 Harris Technologies, Airbus, Nokia, Nokia Shanghai Bell, UIC, Ericsson, Kontron Transportation France, AT&T, BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT |  |  |  |  |  | Date: |  |  | 2021-05-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Rel-14, Rel-15 and Rel-16 the procedure ‘Temporary group call - user regroup’ was removed as this procedure cannot create secure communications. As described previously, in LS C1-184907/S6-181030, CT1 have stated that because of SA3's concerns, neither this procedure or the 'Temporary group – broadcast group call' procedure were planned to be developed in stage 3 for Rel-15. Similarly, no stage 3 procedures have been developed for Rel-16 and Rel-17. Since then, SA6 has developed new procedures using an alternative approach based on preconfigured groups to fulfil SA1 requirements for temporary broadcast group calls and for user regrouping. However, the procedure ‘Temporary group call - user regroup’ is closely related to “ad hoc group call” a feature that is implemented in some commercial PTT applications. There is interest in this feature from the public safety community and some companies have expressed interest in drafting Stage 1 requirements for it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the procedure for 'Temporary group – broadcast group call', and remove relevant parameters in information flows. Rewrite the Note in section 10.6.2.8.1 indicating that it is reserved pending Stage 1 requirements, and remove the current procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedures will continue to have no stage 3 solution, and security concerns will exist on the procedures in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.2.2.7, 10.6.2.2.9, 10.6.2.2.14, 10.6.2.2.14a, 10.6.2.5.3, 10.6.2.8, 10.6.2.8.1, 10.6.2.8.2, 10.6.2.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Corresponding Rel-16 CR is 23.379 CR0244 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 10.6.2.2.7	Group call request (MCPTT client – MCPTT server)

Table 10.6.2.2.7-1 describes the information flow group call request from the MCPTT client to the MCPTT server.

Table 10.6.2.2.7-1 Group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | O | The MCPTT group ID of the group on which the call is requested |
| SDP offer | M | Media parameters of MCPTT clients |
| Implicit floor request | O | When originating client requests the floor, this element shall be included |
| Broadcast indicator | O | Indicates that the group call request is for a broadcast group call |
| Location information | O | Location of the calling party. |
| Requested priority | O | Application priority level requested for this call |

* * * Next Change * * * *

##### 10.6.2.2.9	Group call request (MCPTT server – MCPTT client)

Table 10.6.2.2.9-1 describes the information flow group call request from the MCPTT server to the MCPTT client.

Table 10.6.2.2.9-1 Group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is initiated |
| SDP offer | M | Media parameters of MCPTT server |
| Broadcast indicator | O | Indicates that the group call request is for a broadcast group call |

* * * Next Change * * * *

##### 10.6.2.2.14	Group call release request (MCPTT server – MCPTT client)

Table 10.6.2.2.14-1 describes the information flow group call release request from the MCPTT server to the MCPTT client.

Table 10.6.2.2.14-1 Group call release request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the MCPTT group member |
| Functional alias | O | The functional alias of the MCPTT group member |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is released |

* * * Next Change * * * *

##### 10.6.2.2.14a	Group call release request (MCPTT client – MCPTT server)

Table 10.6.2.2.14a-1 describes the information flow group call release request from the MCPTT client to the MCPTT server.

Table 10.6.2.2.14a-1 Group call release request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the MCPTT group member |
| Functional alias | O | The functional alias of the MCPTT group member |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is released |

* * * Next Change * * * *

##### 10.6.2.5.3	Void

* * * Next Change * * * *

#### 10.6.2.8	Temporary group call – user regroup

##### 10.6.2.8.1	General

NOTE:	This subclause is reserved dependent on SA1 requirements.

.

* * * End of Changes * * * *