3GPP TSG-SA6 Meeting #43-e	S6-211389

# Online, , 24th May 2021 - 2nd Jun 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0061 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve EN on PC5 provisioning for multi-operator V2X services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  |  | 2021-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR resolves the ENs regarding the possible overlapping with the procedure in clause 9.6.4 related to PC5 provisioning. Clause 9.15 discusses the provisioning in multi-operator V2X service, whereas clause 9.6.4 focuses on the single operator use case and refers to LTE-PC5. To better capture potential overlap between the two procedures, the procedure in 9.6.4 was enhanced to include provisioning parameters based on TS 23.287(referring also to the multi-operator case). Following, the steps 5-6 of procedure in 9.15.3.2 were removed and replaced by procedure in 9.6.4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The main changes are: At the clause 9.6 and in particular the information flow in Table 9.6.2.2-1 and the procedure in 9.6.4 to include reference to TS 23.287 and reference to 9.15 (for multi-operator case). Also, sub clauses 9.15.2.4 and 9.15.2.5 were removed (the removed steps 5-6), and in clause 9.15.3.2 steps 5-6 were replaced by a new step 5 which refers to the procedure in 9.6.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the ENs will remain and potential overlap between the two procedures may be possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.6.2.2, 9.6.4.1, 9.6.4.2, 9.15.2.4, 9.15.2.5, 9.15.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

#### 9.6.2.2	Set PC5 parameters request

Table 9.6.2.2-1 describes the information flow set PC5 parameters request from the VAE server to the VAE client.

Table 9.6.2.2-1: Set PC5 parameters request

| Information element | Status | Description |
| --- | --- | --- |
| V2X UE ID | O | Identity of the V2X UE to which the configuration is required |
| PC5 parameters configuration data | M | The PC5 parameters configuration data as specified in 3GPP TS 23.285 [5] and 3GPP TS 23.287 [8]. For PC5 provisioning in multi-operator V2X services (as specified in clause 9.15), these parameters may be the joint provisioning policies/parameters to be used by the V2X-UEs within the V2X service. |

* * * * * * * SECOND CHANGE * * * * * * *

### 9.6.4	PC5 parameters provisioning

#### 9.6.4.1	General

This subclause describes the procedure for the VAE server provisioning the PC5 parameter provisioning information to the V2X UE via V1-AE reference point for V2X communication on PC5. The configuration information includes the configuration parameters specified in 3GPP TS 23.285 [5], 3GPP TS 23.287 [8] and additional application related configuration parameters.

#### 9.6.4.2	Procedure

Figure 9.6.4.2-1 illustrates the procedure for provisioning PC5 parameters to the V2X UE via V1 reference point.

Pre-conditions:

1.	The V2X UE has discovered and connected to the VAE server.

2.	If multicast delivery mode is used, the MBMS bearer being used is activated by the VAE server.

Figure 9.6.4.2-1: PC5 parameters provisioning

1.	The VAE server sends the set PC5 parameters request to the VAE client with the information of the PC5 parameters. The PC5 parameters information are specified in subclause 4.4.1.1.2 in 3GPP TS 23.285 [5] and in subclause 5.1.2.1 in 3GPP TS 23.287 [8]. This message can be sent via unicast or multicast.

2.	Upon receiving the set PC5 parameters request, the VAE client stores the received PC5 parameters in the V2X UE.

3.	The VAE client provides an acknowledgement by sending the set PC5 parameters response to the VAE server.

* * * * * * * THIRD CHANGE * * * * * * *

#### 9.15.2.4	Void

#### 9.15.2.5	Void

* * * * * * * FOURTH CHANGE * * * * * * *

#### 9.15.3.2	Procedure

Figure 9.11.3.2-1 illustrates the procedure where the VAE server supports the PC5 provisioning for multi-operator V2X services.

Pre-conditions:

1.	The V2X UEs have connected to the V2X application server.

2.	The V2X UEs are authorized by their corresponding PLMNs to use PC5 communication.

Figure 9.15.3.2-1: PC5 provisioning for multi-operator V2X services

1.	The V2X application specific server provides a V2X PC5 provisioning requirement to the VAE server.

2.	Optionally, the VAE Server sends a PC5 provisioning status request to the VAE Client (within the multi-operator V2X service) to receive up-to-date information on the per PLMN provisioning policies/ parameters.

3.	Optionally, upon receiving the request as in Step 2, the VAE Client sends a PC5 provisioning status response to VAE server, which includes a report on the up-to-date policies /parameters per PLMN (e.g. application QoS - to - PQI mapping, range, radio parameters) and optionally PC5 related events (PC5 expected link downgrade).

4.	The VAE Server determines the updated PC5 provisioning policies/parameters to be jointly used across the V2X-UEs within the multi-operator V2X service.

NOTE: How the VAE server derives a merged set of PC5 policies is implementation dependent.

5.	The VAE Server provides the PC5 parameter provisioning information to the V2X UE via V1-AE reference point for V2X communication on PC5 as specified in clause 9.6.4.

NOTE: How the V2X UE selects which policy to apply is implementation dependent.

6.	VAE Server sends a V2X PC5 provisioning requirement notification to the V2X application specific server to inform on the result of the multi-operator PC5 provisioning.