3GPP TSG-SA WG6 Meeting #43	S6-211257

e-meeting, 24th May – 2nd June 2021	(revision of S6-21xxxx)

Source:	Samsung

Title:	Pseudo-CR on EAS DNAIs

Spec:	3GPP TS 23.558 v2.1.0

Agenda item:	7.6

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

This contribution closes the following ENs:

Editor's Note: Should the EAS DNAIs be limited to a subset of DNAIs of the EES it is registered at is FFS.

Editor's Note:	Whether or not to split EAS/EES DNAI (s) is FFS.

2. Reason for Change

The EAS service area is equal to or a subset of the service area of the EES, which serves the EAS. Therefore, same relation must apply to the DNAIs of the EAS and the EES; i.e. the EAS DNAIs must be equal to or a subset of the DNAI of the EES, which serves the EAS. There is no practical application of having additional DNAIs associated with the EAS.

With the relation between the EAS and EES DNAIs clarified, only EES DNAIs are sufficient in the EES profile for the ECS to select appropriate EESs as and when required.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v2.1.0.

* * * First Change * * * *

### 8.2.4	EAS Profile

Table 8.2.4-1: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| ACID(s) | O | Identifies the AC(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the EAS Provider |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Geographical Service Area | O | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Topological Service Area | O | The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS |
| EAS Service continuity support | O | Indicates if the EAS supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EAS. |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |
| List of N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Required Service APIs | O | A list of the Service APIs that are required by the EAS |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |

* * * Next Change * * * *

### 8.2.6	EES Profile

The EES profile includes information about the EES and the services it provides.

Table 8.2.6-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| EASIDs | M | List of EASIDs registered with the EES. |
| EES Provider Identifier | O | The identifier of the EES Provider (such as ECSP) |
| EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| List of EES/EAS DNAI(s) | O | DNAI(s) associated with the EES/EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. |
| EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EES. |

NOTE:	For DNAI information, the ECS utilizes only the EES DNAI(s) for determining EES in service provisioning.