3GPP TSG-SA WG6 Meeting #43	S6-211166_rev1

e-meeting, 24th May – 2nd June 2021	(revision of S6-21xxxx)

Source:	Huawei, Hisilicon

Title:	Service-based Architecture

Spec:	3GPP TS 23.554

Agenda item:	0.3.0

Document for:	Approval

Contact:	Hongxia Hao, haohongxia@huawei.com

1. Introduction

This pCR is to propose service based architecture/interface of MSGin5G service.

2. Reason for Change

In stage 2, it is to be decided that which function in MSGin5G service needs to be service based function.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554 v0.3.0.

* * * First Change * * * *

# X	Service-based interface representation for MSGin5G service

## X.1	General

The Service-based architecture for MSGin5G services is represented using functional entities and reference points between the functional entities as specified in subclause 5.

## X.2	Service-based architecture

Figure X.2-1 is the Service-based Architecture for MSGin5G service.

Figure X.2-1: Service-based MSGin5G Architecture

The M5S function is a service based function exhibited by MSGin5G Server.

The M5C function is the MSGin5G Client.

The AC is the application client.

The L3G function is a service based function exhibited by Legacy 3GPP Message Gateway.

The N3G function is a service-based function exhibited by Non-3GPP Message Gateway.

The M5S manages the distribution of the messages it has received from MSGin5G UE, from Application Server, or from N3G (on behalf of Non-3GPP UE) or from L3G (on behalf of Legacy 3GPP UE).

The M5S invokes services provided by L3G/N3G to send MSGin5G Messages towards Legacy 3GPP UE or Non-3GPP UE.

The AS/L3G/N3G invokes services provided by M5S to send MSGin5G Messages to M5S on behalf of Legacy 3GPP UE or Non-3GPP UE.

The M5S invokes services provided by SEAL Group management function to do MSGin5G Group management.

The M5S/L3G/N3G invokes services provided by SEAL Configuration management function to do service configuration (including UE service ID provisioning).

## X.3	Service-based interfaces

Table X.3-1 specifies the service-based interfaces supported by MSGin5G Service.

Table X.3-1: Service-based interfaces supported by MSGin5G Service

| Service-based interface | Application functionEntity | Mapping server entity | APIs offered |
| --- | --- | --- | --- |
| Mm5s | MSGin5G Server function | MSGin5G Server | Specified in 8.X |
| Ml3g | Legacy 3GPP Message Gateway function | Legacy 3GPP Message Gateway | Specified in 8.Z |
| Mn3g | Non-3GPP Message Gateway function | Non-3GPP Message Gateway | Specified in 8.Z |