3GPP TSG-SA WG6 Meeting #43	S6-211167

e-meeting, 24th May – 2nd June 2021	(revision of S6-21xxxx)

Source:	Huawei, Hisilicon

Title:	APIs provided by MSGin5G server

Spec:	3GPP TS 23.554

Agenda item:	0.3.0

Document for:	Approval

Contact:	Hongxia Hao, haohongxia@huawei.com

1. Introduction

This pCR proposes APIs provided by MSGin5G Server.

2. Reason for Change

In 5GMARCH TR, there is a conclusion that the solution 19 and solution 23 for MSGin5G Server to interact with Application Server will be used as the baseline solution for technical specification with necessary enhancements.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554 v0.3.0.

* * * First Change * * * *

# 8	Procedures, APIs and information flows

Editor's note:	The clause number of the following procedure will be changed in the future version.

* * * Next Change * * * *

## 8.X	APIs provided by MSGin5G Server

### 8.X.1	M5S_AS_Originating_Message_Delivery API

#### 8.X.1.1	General

API description: This API enables the AS to send MSGin5G message or delivery status report to the MSGin5G Server.

#### 8.X.1.2	Send_MSGin5G_Message operation

API operation name: Send_MSGin5G_Message

Description: Send a MSGin5G message to MSGin5G Server.

Known Consumers: Application Server

Inputs: Refer subclause 8.2.2

Outputs: Refer subclause 8.2.2

Editor's Note:	The additional IEs used in this API is FFS.

See subclause 8.2.2 for the details of usage of this API operation.

### 8.X.2	M5S_UE_Originating_Message_Delivery API

#### 8.X.2.1	General

API description: This API enables the Message Gateway or other 5GS Function to delivery MSGin5G message to the MSGin5G Server.

#### 8.X.2.2	Send_ MSGin5G_Message operation

API operation name: Send_MSGin5G_Message

Description: Send a MSGin5G Message to MSGin5G Server.

Known Consumers: L3G, N3G.

Inputs: Refer subclause 8.2.2

Outputs: Refer subclause 8.2.2

See subclause 8.2.2 for the details of usage of this API operation.

### 8.X.3	M5S_AS_Originating_Delivery_Status_Report API

#### 8.X.3.1	General

API description: This API enables the MSGin5G Server to delivery MSGin5G Message to the Legacy 3GPP Message Gateway.

#### 8.X.3.2	Report_Message_Delivery_Status operation

API operation name: Report_Message_Delivery_Status

Description: Send a MSGin5G message delivery status report to MSGin5G Server.

Known Consumers: Application Server

Inputs: Refer subclause 8.2.2

Outputs: Refer subclause 8.2.2

See subclause 8.2.2 for the details of usage of this API operation.

### 8.X.4	M5S_UE_Originating_Delivery_Status_Report API

#### 8.X.4.1	General

API description: This API enables the MSGin5G Server to delivery MSGin5G Message to the Legacy 3GPP Message Gateway.

#### 8.X.4.2	Report_Message_Delivery_Status operation

API operation name: Report_Message_Delivery_Status

Description: Send a MSGin5G message delivery status report to MSGin5G Server.

Known Consumers: L3G, N3G.

Inputs: Refer subclause 8.2.2

Outputs: Refer subclause 8.2.2

See subclause 8.2.2 for the details of usage of this API operation.

### 8.X.5	M5S_AS_Registration API

#### 8.X.5.1	General

API description: This API enables the AS to register to MSGin5G Server.

#### 8.X.5.2	Registration operation

API operation name: Registration

Description: Do registration or update registration to a MSGin5G Server, by using this API, the AS provides/updates its informations, including the URL used for the message delivery from MSGin5G Server to AS.

Known Consumers: Application Server

Inputs: Refer subclause 8.Y.1

Outputs: Refer subclause 8.Y.2

Editor's Note:	The procedure of AS registration is to be added.

#### 8.X.5.2	Deregistration operation

API operation name: Send_MSGin5G_Message

Description: Do deregistration with a MSGin5G Server.

Known Consumers: Application Server

Inputs: Refer subclause 8.Y.3

Outputs: Refer subclause 8.Y.4

Editor's Note:	The procedure of AS deregistration is to be added.

* * * Next Change * * * *

## 8.Y	Information flows

### 8.Y.1	M5S AS registration request

The information flows from the Application Server to the MSGin5G server for registration request includes the information elements in Table 8.Y.3-1.

Table 8.Y.1-1: AS Registration request

| Information element | Status | Description |
| --- | --- | --- |
| AS service ID | M | The MSGin5G identifier of the Application Server. This ID is configured before registration. |
| Application ID | M | The identifier of the application specified by the application provider. |
| Authorization Infomation | O | The authorization information used to determine whether the AS is allowed to send the message |
| Security Credentials | O | Security information required for the Application Server to register to the MSGin5G Server. |
| Recall URLs |  | The URL for receiving message, message delivery status report , etc. The MSGin5G Server uses this URL to interact to AS. |
| Application Profile (NOTE) | O | The elements in Application Profile include the information of the Application Server, e.g. application name, application provider, application scenario description, application category, etc. This IE is used by MSGin5G Server to compare with application client information. |
| NOTE: The detailed definition of Application Profile is out of scope of this document. |  |  |

### 8.Y.2	M5S AS registration response

The information flows from the MSGin5G server to the Application Server for registration response includes the information elements in Table 8.Y.2-1.

Table 8.Y.2-1: AS registration response

| Information element | Status | Description |
| --- | --- | --- |
| AS service ID | M | The MSGin5G identifier of the Application Server. |
| Registration result | M | Indication if the registration is success or failure |
| UE Service ID list | O | list of UEs' identifier which may communicate with the Application Server. |

### 8.Y.3	M5S AS deregistration request

The information flows from the Application Server to the MSGin5G server for registration request includes the information elements in Table 8.Y.3-1.

Table 8.Y.3-1: AS Registration request

| Information element | Status | Description |
| --- | --- | --- |
| AS service ID | M | The MSGin5G identifier of the Application Server. |

### 8.Y.4	M5S AS deregistration response

The information flows from the MSGin5G server to the Application Server for registration response includes the information elements in Table 8.Y.4-1.

Table 8.Y.4-1: AS registration response

| Information element | Status | Description |
| --- | --- | --- |
| AS service ID | M | The MSGin5G identifier of the Application Server. |
| Deregistration result | M | Indication if the registration is success or failure |