3GPP TSG-SA WG6 Meeting #43	S6-211233 Rev1

e-meeting, 24th May – 2nd June 2021	(revision of S6-21xxxx)

Source:	China Mobile

Title:	Pesudo-CR on solve hanging paragraphs

Spec:	3GPP TS 23.554 v0.3.0

Agenda item:	7.9

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

- 1. Introduction

Solve hanging paragraphs in clause 5.3.4 and 6.1.

- 2. Reason for Change

Solve hanging paragraphs in clause 5.3.4 and 6.1.

- 3. Conclusions

<Conclusion part (optional)>

- 4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554 v0.3.0.

* * * First Change * * * *

# 6	Identities

## 6.1	Identities for MSGin5G service endpoints

### 6.1.1	General

MSGin5G service endpoints shall be identified by unique identifiers within the MSGin5G service domain. For each MSGin5G service endpoint the identifier shall be a unique URI that can be used to identify the MSGin5G service endpoint's home service domain.

The following clauses describe different types if MSGin5G service endpoint identifiers

### 6.1.2	MSGin5G UE Service Identity (UE Service ID)

UE Service ID is the MSGin5G Service ID of a UE (i.e. MSGin5G UEs, Legacy 3GPP UEs, Non-3GPP UEs).

For a MSGin5G UE, the assigned UE Service ID is used by the MSGin5G Client to register with the MSGin5G Server and to send and receive MSGin5G messages. It is used by the MSGin5G Server to authenticate and authorize the associated UE to the MSGin5G service at the application layer.

For a Legacy 3GPP UE or a Non-3GPP UE, the assigned MSGin5G Service ID is used by the Message Gateway to register with the MSGin5G Server, to map into the Service ID in their defined message delivery mechanisms for interworking, and to send/receive MSGin5G messages to/from MSGin5G Server onbehalf of Legacy 3GPP UE or Non-3GPP UE.

### 6.1.3	Application Server Service Identity (AS Service ID)

AS Service ID is the MSGin5G Service ID of an Application Server. It is used to perform mutual authentication with the MSGin5G Server for establishing a secured service connection, and it is used to send/receive MSGin5G messages to/from other MSGin5G service endpoints via MSGin5G Server.

### 6.1.4	Message Gateway Service Identity (MGW Service ID)

MGW Service ID is the MSGin5G Service ID of Message Gateway. It is used to perform mutual authentication with the MSGin5G Server for establishing a secured service connection, and it is used to send/receive MSGin5G Messages to/from MSGin5G Server on behalf of Legacy 3GPP UE or Non-3GPP UE.