23GPP TSG-SA WG6 Meeting #43	S6-211322

e-meeting, 24th May – 2nd June 2021	(revision of S6-21xxxx)

Source:	Deutsche Telekom

Title:	Changes in Initiation of C2 mode selection

Spec:	3GPP TS 23.255 v.0.4.0

Agenda item:	7.10

Document for:	Approval

Contact:	Vasil Aleksiev <vasil dot Aleksiev at magenta dot at>

1. Introduction

This contribution provides proposal for change in Initiation of C2 mode selection

2. Reason for Change

In subclause 6.4.2.1 it was agreed to have initiation of C2 mode selection from UAS application specific server. The current step 2/response does not take in mind acknowledgement from C2 communication modes configuration (6.4.2.2).

3. Conclusions

<Conclusion part (optional)>

4. Proposal

To provide clarification that after successful execution of C2 modes configuration (6.4.2.2) UAE server could send to the UAS application specific server positive response. Precondition is added in 6.4.2.1 to show from where the UAS application specific server knows the UAS ID which is sent in C2 operation mode management request.

* * * First Change * * * *

## 6.4	C2 Communication mode selection and switching

### 6.4.1	General

This feature introduces the UAS application enablement services for supporting the selection and re-selection of C2 communication modes. In particular, the UAE layer provides support for the following operations:

-	Support the switch between the Network-Assisted C2 communication and Direct C2 communication (e.g. when the direct link becomes feasible/available, or when a UAV is moving towards BLOS or has poor direct link conditions, etc.) as described in clause 6.4.2.4.

-	Support the switch between the Network-Assisted/Direct C2 communication and UTM-navigated C2 communication (e.g. for air traffic control, the UAV is approaching a No Drone Zone, and detected potential security threats, etc.) as described in clause 6.4.2.5.

-	Support the selection of the communication mode between: utilizing more than one C2 communication links, and among applicable C2 communication links, selecting a mode as the primary one as described in clause 6.4.2.3.

-	Activation for the support of the above operations in the UAE Server in the UAE client is performed using procedure described respectively in clause 6.4.2.1 and clause 6.4.2.2.

Below, the different procedures for C2 communication mode selection and switching are described using UAE Client assisted and UAE Server controlled based mechanisms. Such functionality is supported by means of policies delivered to the UAV/UAV-C via the UAE layer and assisting the dynamic switching of C2 modes.

NOTE:	For direct C2 communication mode, usage of ProSe/PC5 is not considered in the present specification (e.g. direct wireless communication over a technology outside the scope of 3GPP is assumed).

### 6.4.2	Procedures

#### 6.4.2.1	Initiation of C2 mode selection / switching capability

This procedure initiates the capability at the UAE server, based on an application request from UAS application specific server (which can be the UTM/USS) to manage the C2 operation modes (direct, network-assisted) of C2 communication for a UAS.

Figure 6.4.2.1-1 illustrates the procedure where the UAE server receives an application request for managing the operation mode for C2 communications for a UAS.

Pre-conditions:

- The UAV has received its CAA level ID from the UAS application specific server.

Figure 6.4.2.1-1: C2 operation mode management request / response

1.	The UAS application specific server sends to the UAE Server a C2 operation mode management request for managing the operation modes for the C2 communication for a UAS (consisting a UAV and a UAV-C) and to subscribe for UAE notifications.

2.	The UAE Server sends to the UAS application specific server a C2 operation mode management response with a positive or negative acknowledgement of the request.

#### 6.4.2.2	C2 communication modes configuration

This procedure enables the configuration of the UAE Client, based on an application request from UAS application specific server (which can be the USS/UTM) to manage the C2 operation modes (direct, network-assisted) of C2 communication for a UAS.

Figure 6.4.2.2-1 illustrates the C2 communication modes configuration procedure.

Pre-conditions:

1.	The UAV is connected to 5GS and authenticated and authorized by UAS AS.

2.	UAE Server has established a UAE session with the respective UAE Clients.

3.	UAE Server has performed the C2 mode switching/selection capability initiation as in 6.4.2.1.

Figure 6.4.2.2-1: C2 communication modes configuration

1.	The UAE Server sends a C2 communication modes configuration command including the UAV identification (CAA Level UAV ID, IP address), allowed C2 communication modes (e.g., direct, network assisted, USS/UTM navigated), primary C2 communication mode and policy for C2 link selection and switching.

Editor's note:	Details of the C2 link selection and switching policy (e.g., Application QoS thresholds) are FFS.

2.	The UAE Client stores the C2 communication mode configuration parameters.

3.	The UAE Client sends an acknowledgement to the UAE Server.

4. The UAE Server sends notification to the UAS application specific server.

#### 6.4.2.3	C2 communication mode selection by UAE Client

This procedure provides a mechanism for the UAE client to select a primary C2 communication mode and optional secondary C2 communication mode based on C2 communication mode configuration enabled as described in clause 6.4.2.2.

Figure 6.4.2.3-1 illustrates the C2 communication mode selection and redundant C2 link negotiation.

Pre-conditions:

1.	The UAE Clients are configured with a C2 communication modes configuration as described in clause 6.4.2.2.

2.	UAE Server has activated the dynamic C2 mode switching capability, as described in 6.4.2.1.

Figure 6.4.2.3-1: C2 communication mode selection

1.	UAE Clients (UAV and UAV-C) select a primary and secondary C2 communication mode based on C2 communication mode configuration.

Editor's note:	Details of the negotiation procedure for redundant C2 connection handling between UAV and UAV-C are FFS.

2.	The UAE Client sends a notification message to the UAE Server indicating the selected primary and secondary C2 communication modes and associated C2 link information which may include UAE Client and peer address information (e.g., IP/MAC address).

3.	The UAE Server stores the C2 communication modes and links information.

4a-4b.	The UAE Server forwards the C2 communication modes and links information to the UAS AS and receives an acknowledgement from the UAS AS.

5.	The UAE Server may send an acknowledgement to the UAE Client.

6.	The UAV and UAV-C start C2 communication using the selected primary C2 communication mode.

NOTE:	The details of step 6 are outside the scope of the present specification.

#### 6.4.2.4	UAE-layer assisted dynamic C2 mode switching

This procedure provides a mechanism for supporting dynamic switching between direct and network assisted C2 communications, which may be required while the UAV flight is ongoing, due to possible change of network conditions, expected location/mobility of the UAV, unpredictable events etc.

Figure 6.4.2.4-1 illustrates the procedure where the UAE server supports the dynamic C2 mode switching for network-assisted C2 communications.

Pre-conditions:

1.	UAE Server has activated the dynamic C2 mode switching capability, as described in 6.4.2.1

2.	UAE Server has subscribed for using SEAL/LMS services and has configured the location event reporting, based on 3GPP TS 23.434 [5].

3.	UAE Client has selected a primary C2 communication mode as described in clause 6.4.2.3. And UAV and UAV-C are engaged in C2 communication.

Figure 6.4.2.4-1: UAE-assisted dynamic C2 mode switching

1.	When the UAE Client detects a condition for switching C2 communication mode based on local conditions (e.g. using the C2 communication mode switching policy) or based on a UAS AS command (as described in clause 6.4.2.5). A trigger event is sent from the UAE Client of the UAV and/or the UAV-C to the UAE Server, denoting a UAS AS command or an application QoS attribute change (experienced or expected) e.g. based on the experienced packet delay or packet loss for the Uu or direct link (e.g. packet loss greater than a pre-defined threshold).

2.	Additionally, the UAE Server receives a location report for the UAV/UAV-C by the SEAL's LM server. The report can be either periodical or event-based (e.g. UAV moving towards an area covered by a different cell or different operator), as specified in 3GPP TS 23.434 [5] SEAL's LM server procedures (UAE Server acting as a VAL server).

3.	The UAE Server determines the switching of the C2 mode from direct to network assisted or vice versa or to USS/UTM navigated. If the switching is from direct to network assisted or vice versa, this is done by calculating the relative actual or expected UAV-to-UAV-C location, as well as other factors like QoS fulfilment/unfulfilment, augmented location, mobility/speed, direction, topography, weather conditions.

4.	The UAE Server sends a C2 mode switching confirmation request message to the UAS application specific server, which includes the UAS identifier as well as the cause for switching and the switching option (direct to network-assisted or network-assisted to direct or to USS/UTM navigated). The UAE Server sends this request to obtain confirmation from the UAS AS before proceeding with switching to USS/UTM navigated. This step is optional in the case of switching from direct to network assisted or vice versa.

5.	Conditional on Step 3, the UAE Server receives from the UAS application specific server a response indicating a positive or negative result for the requested change.

6.	The UAE Server sends to the involved UAE Clients, a C2 operation mode switching command message which provides an instruction to the UAV and UAV-C to switch to network-assisted mode or to direct mode or to USS/UTM navigated. The UAV and UAV-C start C2 communication using the indicated C2 communication mode (e.g., direct, network assisted, USS/UTM navigated).

7.	If an emergency switch of the C2 communication is deemed necessary by the UAE Client (e.g. sudden loss of the active C2 link), the UAE Client changes the link prior to the steps 1-6, which are skipped. The UAE Clients send a notification message to the UAE Server to confirm the switching of the C2 communication mode.

#### 6.4.2.5	UAS AS triggered C2 communication mode switching

This procedure provides a mechanism for supporting dynamic switching between direct or network assisted C2 communications to USS/UTM navigated, initiated by the UAS AS (which can be the USS/UTM) after detecting a C2 switching condition which may be required while the UAV enters a no-fly zone. For example, the UAS AS needs to take over the control of UAV and fly it to safety (see 3GPP TS 22.125 [2] clause 4.2).

Editor's note:	Consistently usage of terminology must be ensured for the terms USS/UTM. UAS-AS UASS and UAS Application specific server.

Figure 6.4.2.5-1 illustrates a UAS AS triggered C2 communication mode switching.

Pre-conditions:

1.	UAE Server has activated the dynamic C2 mode switching capability, as described in clause 6.4.2.1.

2.	UAE Client has selected a primary C2 communication mode as described in clause 6.4.2.3, and UAV and UAV-C are engaged in (e.g., direct or network assisted) C2 communication.

Figure 6.4.2.5-1: UAS AS triggered C2 communication mode switching

1.	The UAS application specific client is instructed directly by UAS AS to switch to USS/UTM navigated mode.

NOTE:	This procedure between UAS AS and UAS application specific client is out of scope of the present specification.

2.	The UAE Client initiates the procedure described in clause 6.4.2.4.

Editor's note:	release of connection used for C2 communication with UAV-C is FFS.

### 6.4.3	Information flows

Editor's note:	The information flow tables related to procedures in clause 6.4.2.2, clause 6.4.2.3, clause 6.4.2.5 are FFS.

#### 6.4.3.1	C2 operation mode management request

Table 6.4.3.1-1 describes the information flow C2 operation mode management request from the UAS application specific server to the UAE server.

Table 6.4.3.1-1: C2 operation mode management request

| Information element | Status | Description |
| --- | --- | --- |
| UAS AS ID | M | Identity of the UAE client of the UAS application specific server which requests the C2 operation mode management. This ID can be the UTM/USS identifier, when the UAS application specific server is the UTM/USS. |
| UAS ID | M | The identification of the UAS for which the C2 QoS management request applies. This could be in form of identifier for the UAS, e.g group ID; or individual identifiers for the UAV and UAV-C, e.g. CAA ID, GPSI |
| C2 operation mode management requirement | M | Identification of the type of the C2 mode switching to be supported by the UAE server. This can be either from direct to network-assisted C2, or from network-assisted to direct C2 or to UTM navigated. |
| C2 service area | O | The area where the C2 operation mode management request applies. This can be geographical area, or topological area in which the capability is active. |

#### 6.4.3.2	C2 operation mode management response

Table 6.4.3.2-1 describes the information flow C2 operation mode management response from the UAE server to the UAS application specific server.

Table 6.4.3.2-1: C2 operation mode management response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative result of the C2 operation mode management request. |

#### 6.4.3.3	C2-related trigger event report

Table 6.4.3.3-1 describes the information flow C2-related trigger event report from the UAE client (of the UAV or the UAV-C) to the UAE server.

Table 6.4.3.3-1: C2-related trigger event report

| Information element | Status | Description |
| --- | --- | --- |
| UAE client ID | M | The identifier of the UAE client which indicates the QoS downgrade |
| Application QoS-related event | M | The report including the expected or actual application QoS / QoE parameters which were change (i.e. latency, throughput, reliability, jitter). The event configuration (thresholds, policies) is provided to the UAE client as described in clause 6.3.2.2. |

#### 6.4.3.4	C2 mode switching confirmation request

Table 6.4.3.4-1 describes the information flow C2 mode switching confirmation request from the UAE server to the UAS application specific server.

Table 6.4.3.4-1: C2 mode switching confirmation request

| Information element | Status | Description |
| --- | --- | --- |
| UAE server ID | M | The identifier of the UAE server which requests the C2 mode switching confirmation from UTM/USS |
| UAS ID | M | The identification of the UAS. This could be in form of identifier for the UAS, e.g group ID; or individual identifiers for the UAV and UAV-C, e.g. CAA ID, GPSI. |
| C2 operation mode switching type | M | The type of the C2 mode switching to be applied (direct to network-assisted, or network-assisted to direct). |

#### 6.4.3.5	C2 mode switching confirmation response

Table 6.4.3.5-1 describes the information flow C2 mode switching confirmation response from the UAS application specific server (UTM/USS) to the UAE server.

Table 6.4.3.5-1: C2 mode switching confirmation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative result of the C2 mode switching confirmation response. |

#### 6.4.3.6	C2 operation mode switching

Table 6.4.3.6-1 describes the information flow C2 operation mode switching command from the UAE server to the UAE client(s) of the affected UAS.

Table 6.4.3.6-1: C2 operation mode switching

| Information element | Status | Description |
| --- | --- | --- |
| UAE server ID | M | The identifier of the UAE server which instructs the UAS to apply the C2 mode switching. |
| C2 operation mode switching requirement | M | The type of the C2 mode switching to be applied (direct to network-assisted, or network-assisted to direct). |
| Time Validity | O | Time validity for the C2 switching requirement |
| Geographical Area | O | Area for which the C2 switching applies |

* * * Next Change * * * *