3GPP TSG-SA WG6 Meeting #43	S6-211150

e-meeting, 24th May – 2nd June 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0071 | rev | 0 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SIP session information request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP, eSEAL |  |  |  |  |  | Date: |  |  | 2021-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The stage 1 requirement of FS_UASAPP (TS23.755) requests media sessions need to be managed to ensure the service continuity of media services such as video streaming, etc. The current specification does not make the existing SIP sessions visible to the VAL server, which could cause SIP session interruptions when there are more SIP sessions requests than one NRM server can handle. By enabling the VAL server to have the visibility of the existing SIP sessions between connected VAL client and NRM server, 1) have the visibility of existing SIP sessions 2) manage SIP session across multiple NRM server such as a) redirect SIP session traffic b) load balancing SIP connection requests The proposed feature is related to S6-211302, where the SIP sessions were established between NRM-C and NRM-S |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced the IE and procedure to request SIP session information between VAL server and NRM server |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The VAL server does not know how many SIP sessions were established between the connected VAL client and NRM server, which could cause the Media session interrupted if traffic is not balanced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.2.x (new), 14.3.2.y (new), 14.3.x.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 14.3.2.x	Session report request

Table 14.3.2.x-1 describes the information flow of session report request from the VAL server to the NRM server

Table 14.3.2.x-1: Session report request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of VAL server performing the request |
| VAL user ID or VAL UE ID | O | The identity of VAL user or VAL UE |
| VAL Group ID | O | The identity of VAL group ID |
| SIP session information | M | The indentation of requesting a list of existing SIP sessions. |

#### 14.3.2.y	Session report response

Table 14.3.2.y-1 describes the information flow of session report response from the NRM server to the VAL server.

Table 14.3.2.y-1: Session report response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates the list of session ID(s) with the current session status. |

* * * Next Change * * * *

#### 14.3.x.1	Request for unicast resources at session establishment from NRM server

###### 14.3.x.1.1	General

The procedure defined in this clause specifies how existing SIP session information is requested from VAL server to NRM server. The request sends from the VAL server includes the specific VAL server identity with optional VAL user/UE or group ID. The VAL server requests the SIP sessions list established between the connect VAL client and NRM server. If one VAL user, UE, or group ID is sent with this request, the NRM shall return the SIP session list that pertains to that element entity.

This procedure enables the VAL server to know about existing SIP sessions established with the VAL client and NRM servers.

###### 14.3.x.1.2	Procedure

Figure 14.3.3.2.1a.2-1 describes the procedure for the request of SIP session information between the VAL server and the NRM server.

Pre-condition:

1.	The NRM client has initiated and established the SIP session with the NRM server.

Figure 14.3.x.1.2-1: SIP session information request from VAL server to NRM server

- The VAL server sends SIP session information requests to NRM server. The request sends from the VAL server includes the specific VAL server identity with optional VAL user/UE or group ID.

- The NRM server returns a list of existing SIP session information. If one VAL user, UE, or group ID is sent with this request, the NRM shall return the SIP session list that pertains to that element entity.