3GPP TSG-SA WG6 Meeting #44	S6-211636

e-meeting, 12th July – 20th July 2021	(revision of S6-211xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0077 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enable VAL server for Peer-to-Peer media parameter negotiation. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2021-07-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Peer-to-peer (p2p) media streaming technology such as WebRTC has been becoming a promising media streaming method with the benefits of reducing the cost of a centralized media server and communication delays. In SEAL, with media payload in verticals such as UASAPP and V2X, enabling p2p media parameter signaling is an essential step for establishing media sessions such as using SIP. One key important enabler to enable p2p media session communication is to use NAT traversal technics such as ICE/STUN/TURN server to locate the IP addresses of all the peers. SDP is one of the protocols being used for media session parameter negotiation. This proposal adding a new feature that enabling the VAL server for p2p media parameters negotiation using existing SEAL signaling architecture such as NAT traversal offered by the NRM-S. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced the IE and procedure to request SDP parameter negotiation between VAL Client and VAL server |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | P2p media session can not be established without knowing what peers’ SDP offering |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.2.x (new), 14.3.2.y (new), 14.3.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 14.3.2.x	Resource request

Table 14.3.2.x-1 describes the information flow for the media parameter signaling request from VAL Client to VAL server for the purpose of media session negotiation.

Table 14.3.2.x-1: media session negotiation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL user or VAL UE performing the request. |
| VAL user ID or VAL UE ID | M | The identity of the VAL user or VAL UE. |
| SDP offer | M | SDP of the media parameters offered from a VAL client media payload |

* * * Next Change * * * *

#### 14.3.2.y	Resource response

Table 14.3.2.y-1 describes the information flow for the media parameter feedback from the VAL server to VAL Client.

Table 14.3.2.y-1: media session negotiation response

| Information element | Status | Description |
| --- | --- | --- |
| SDP feedback | M | SDP of the media parameters feedback (codec, ICE candidates, media types and etc) |

* * * Next Change * * * *

#### 14.3.3.x	Peer-to-peer media session parameter negotiation

###### 14.3.3.x.1	General

The procedure defined in this clause specifies how media session parameters carried in SDP sent by multiple VAL clients are being negotiated between VAL client and VAL server. The request sends from the VAL clients include the VAL user/UE ID and SDP offers. The VAL server utilizes the NRM server for media session signalling such as using NAT traversal for transport layer data retrieval.  The VAL server returns the agreed SDP parameter for all the VAL clients. The VAL clients can use the agreed SDP answers to establish peer-to-peer media streaming.

###### 14.3.3.x.2	Procedure

Pre-condition:

- Two or more VAL clients have requested VAL service with the VAL server.

Figure 14.3.3.x.2-1 describes the procedure for the Peer-to-peer media secession parameter negotiation among VAL Client, VAL server, and NRM server

Figure 14.3.3.x.2-1: Peer-To-Peer Media signaling among VAL Client, VAL server, and NRM server

- The VAL clients send media session parameter negotiation requests to the VAL server with parameters specified in 14.3.2.x.  Minimum two peers are trying to establish a peer-to-peer media communication channel, 1.a and 1.b illustrate those two VAL clients send media session negotiation to a VAL server.

NOTE: the order of 1.a and 1.b is not relevant to how the VAL server handles the negotiation request.

- The VAL server uses the NRM server to locate the transport layer information such as the IP and port numbers for each connecting peer as specified by the SDP. The NRM server utilizes the NAT traversal capability as specified in 14.3.3.2 and returns the transport layer information of each peer back to the VAL server.

- The VAL server consolidates the transport layer data from the NRM server and sends back the agreed SDP answer specified in 14.3.2.y to each peer as shown in 3.a and 3.b.

NOTE: the order of 3.a and 3.b is not relevant to how peers utilize the returned SDP feedback.

- Each peer can use the transport layer information to establish a peer-to-peer media session.