3GPP TSG-SA WG6 Meeting #44	S6-211658

e-meeting, 12nd July – 20th July 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0013 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ACR information notification method |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2021-07-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In scenario 8.8.2.4, 8.8.2.5, if the EEC has not subscribed to the ACR information notification at the S-EES for the target information notification events and ACR complete events, the whole ACR procedure will not be performed as described in current specification. However, even if the EEC has subscribed to the ACR information notification events, the ACR information can be sent via the application layer itself. So, the EAS can check with the EES whether the ACR information can be sent through the EES to EEC, if not, the EAS can determine to send the information via the application layer, and the ACR procedure will not be affected by the subscription status of the EEC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The EAS subscribes for the ACR information notification method from the EES. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The scenario 8.8.2.4, 8.8.2.5 ACR procedure will never be performed if the EEC has not subscribed at the EES for the ACR information notification method. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.3.2.2, 8.6.3.2.3, 8.6.3.3.2, 8.8.2.4, 8.8.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.6.3.2.2	Subscribe

Figure 8.6.3.2.2-1 illustrates the subscribe operation between the EAS and the EES for ACR management event notifications.

Figure 8.6.3.2.2-1: ACR management event API: Subscribe operation

1.	The EAS sends ACR management event subscribe request (e.g. tracking the UE's user plane path change continuously). The EAS shall include UE Identifier or UE group Identifier.

a.	The EAS may include the "user plane path change" event to indicate the EES to notify the EAS when the EES detects there is a user plane path change for the application traffic and the EAS may include Subscription Type (Early and/or Late notification defined in clause 5.6.7 of 3GPP TS 23.501 [2]) and/or Indication of EAS Acknowledgement in the event subscription.

b.	The EAS may include the "ACR monitoring" event to indicate the EES to notify the EAS when the EES detects there is a need for the ACR (e.g. when T-EAS is available at the target DNAI or ACR is initiated by EEC). The EAS may also include the Event Filters to specify the conditions to match for notifying the event, e.g., inter-EDN mobility, intra-EDN mobility.

c.	The EAS may include the "ACR facilitation" event to indicate the EES to notify the S-EAS of the selected T-EAS after the EES detects and determines the need for ACR.

d.	The EAS may include the "ACR information notification method" event to indicate the EES to notify the EAS that the ACR information notification can be sent via EEL when the EEC successfully subscribes the ACR information.

2.	The EES checks if the EAS is authorized for this operation. If authorized, EES checks if there exists a subscription with the 3GPP core network for the user plane path management event notifications corresponding to the UE information obtained in step 1 as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3], which may be triggered by other EAS for the same UE. The EES checks the availability of the user plane path management event service for the UE.

a.	if a subscription with 3GPP core network does not exist, then the EES subscribes with the 3GPP core network (PCF, NEF or SCEF+NEF) for the user plane path management event notifications of the UE as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3] If the EAS provides Subscription Type and/or Indication of EAS Acknowledgement, the EES include the type of subscription and/or the indication of "AF acknowledgement to be expected" as information on AF subscription to corresponding SMF events within the AF Request;

b.	if a subscription with 3GPP core network exists, then the EES uses the locally cached user plane path management event notification information of the UE to respond to the EAS.

The EES stores the subscription related to the EAS.

3.	If the event is "user plane path change", the EES may subscribe to UE expected behaviour analytics (UE mobility and UE communication) for the group of UEs as described in 3GPP TS 23.288 [18].

4.	If EAS is authorized, the EES responds with ACR management event subscribe response. If EAS is not authorized, the EES provides a rejection response with cause information.

If the target UE and the 3GPP network support mobility between 5GC and EPC, the EES monitors the availability of the user plane path management event notification from the 3GPP network by utilizing Nnef_APISupportCapability or Availability of service APIs event notifications provided by the CAPIF core function.

* * * Next Change * * * *

##### 8.6.3.2.3	Notify

Figure 8.6.3.2.3-1 illustrates the notify operation between the EES and the EAS for continuous ACR management event notifications.

Figure 8.6.3.2.3-1: ACR management event API: Notify operation

1.	The EES detects the ACR management event of the UE (e.g. receiving User plane path management event notification for the UE from the 3GPP core network).

a.	If "user plane path change" Event is subscribed, the EES may cache the detected User plane path management event notification locally with timestamp as the latest information of the UE(s) and start the notification aggregation for a group of UEs. The EES decides whether to aggregate and the aggregation period based on the analytics result received from the 3GPP Core Network, local policy and User Plane path management subscription information received from the EAS. The EES determines to notify the user plane path management event notification information (e.g., DNAI) to the EASs which has subscribed for the "user plane path management" event.

b.	If "ACR monitoring" Event is subscribed, based on the detected user plane path change report sent from the 3GPP core network, the EES checks whether the target DNAI is in the EAS profile of the subscribing EAS, if not it further checks whether a T-EAS is available at the target DNAI as described in steps 2-4 of clause 8.8.3.2. If a T-EAS is available, the EES notifies the EAS with T-EAS endpoint; otherwise this event notification will not be sent. Also, when the EES receives the ACR request from the EEC, the EES decides to send the notification to the EAS.

c.	If "ACR facilitation" Event is subscribed, based on the detected user plane path change report sent from the 3GPP core network, the EES checks whether the target DNAI is in the EAS profile of the subscribing EAS, if not it further checks whether a T-EAS is available at the target DNAI as described in steps 2-4 of clause 8.8.3.2. If a T-EAS is available, the EES selects the T-EAS from the discovered EAS list and applies the AF traffic influence with the N6 routing information of the selected T-EAS in the 3GPP Core Network. The EES also notifies the S-EAS with the selected T-EAS endpoint.

d.	If "ACR information notification method" Event is subscribed, the EES notifies the subscribed EAS the ACR information can be sent via EES to the EEC.

2.	The EES sends ACR management event notification to the EAS. The EES includes the ACR management event notification information of the UE(s) and optionally the timestamp. If the event triggering the notification is DNAI change, the timestamp can be included to indicate the age of the user plane path management event notification information. The EES may only provide part of information included in the user plane path management event notification from 3GPP network (e.g. target DNAI). If the EAS had provided "Indication of EAS acknowledgement", the EES waits for acknowledgement from the EAS before it sends AF acknowledgement to the 3GPP core network.

3.	If the EAS had included Indication of EAS Acknowledgement within ACR path management event subscribe request described in clause 8.6.3.2.1, the EAS sends EAS Acknowledgement as a response to ACR management event notification to the EES either immediately or after the required ACT is completed. The EAS may reply in negative, e.g., the EAS may determine not to perform ACR. Then, the EES sends the AF acknowledgement to the 3GPP core network.

* * * Next Change * * * *

##### 8.6.3.3.2	ACR management event subscribe request

Table 8.6.3.3.2-1 describes the information elements for an ACR management event subscribe request from the EAS to the EES.

Table 8.6.3.3.2-1: ACR management event subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| Security credentials | M | Security credentials of the EAS |
| UE ID (NOTE) | O | The identifier of the UE (i.e. GPSI or identity token) |
| UE Group ID (NOTE) | O | Identifies a group of UEs (i.e. internal group ID or external group ID) |
| Event ID(s) | M | Event ID: - user plane path change - ACR monitoring - ACR facilitation - ACR information notification method |
| Event Report | M | Event Reporting Information as specified in 3GPP TS 23.502 [3] |
| Notification Target Address | M | Notification Target Address of the EAS where the notification is to be sent by the EES |
| Type of subscription | O | Indicates Early and/or Late notification to inform if the notification needs to be received before and/or after UP path configuration. Applicable for the "user plane path change" event. |
| Indication of EAS acknowledgement | O | This IE indicates the EES to include indication of "AF acknowledgement to be expected" within the AF request for subscribing UP path management events to 3GPP network and that the EAS will provide an acknowledgement as a response for the notifications of UP path management events to the EES. Applicable for the "user plane path change" event. |
| Event Filter | O | Event filter as specified in 3GPP TS 23.501 [2] |
| EAS characteristics for ACR | O | Set of characteristics to determine required EAS as detailed in Table 8.5.3.2-2. Applicable for the "ACR monitoring" event and "ACR facilitation" event. |
| NOTE: Either UE ID or UE Group ID shall be provided. |  |  |

* * * Next Change * * * *

#### 8.8.2.4	S-EAS decided ACR scenario

The procedure in this clause illustrates the scenario for S-EAS decided ACR.

In this procedure, the S-EAS may detect the need of ACR locally or is notified by the S-EES. The S-EAS make the decision about whether to perform the ACR, and starts the ACR at a proper time.

Pre-conditions:

1.	The S-EAS may depend on the receipt of certain User plane path management events from the S-EES, e.g. "user plane path change" events or "ACR monitoring" events, to detect the need for an ACR. For the following procedure it is assumed that the S-EAS has subscribed to continuously receive the respective events from the S-EES; and

2.	The EAS has subscribed to receive the ACR information notification method from the EES, as described in clause 8.6.3.2.2.

Figure 8.8.2.4-1: S-EAS decided ACR scenario

The S-EAS decided ACR scenario is outlined with four main phases: detection, decision, execution and clean up.

Phase I: ACR Detection

1.	The S-EAS either receives notifications from source Edge Enabler Sever indicating that ACR may be required ("ACR monitoring" event), or self detects the need for ACR (e.g. upon receipt of a "user plane path change" event). If the notification indicates "ACR monitoring" event, then the notification will also contain the T-EAS information (see clause 8.6.3.2.3). The S-EAS may detect that ACR may be required for an expected or predicted UE location in the future as described in clause 8.8.1.

NOTE 1:	How the S-EAS self detects the local need for ACR is outside the scope of this specification.

Phase II: ACR Decision

2.	The S-EAS makes the decision to perform the ACR

NOTE 2:	How the S-EAS determines when to start the ACR is outside the scope of this specification.

Phase III:	ACR Execution

3.	The S-EAS discovers the T-EAS as described in clause 8.8.3.2. When in step 1 the ACR has been triggered for service continuity planning, then UE Location and Target DNAI values in the Retrieve T-EES procedure contain the expected UE Location and expected Target DNAI. After S-EAS determines the T-EAS to use, the S-EAS may apply the AF traffic influence with the N6 routing information of the T-EAS in the 3GPP Core Network (if applicable).

4.	The S-EAS sends selected T-EAS declaration message to S-EES, to inform S-EES the determined T-EAS to use as described in clause 8.8.3.7.

5.	Based on the T-EAS selection information received from the S-EAS, the S-EES sends the target information notification to the EEC as described in clause 8.8.3.5.3.

6.	The S-EAS transfers the application context to the T-EAS selected in step 3. This process is out of scope of the present specification.

When in step 1 the ACR has been triggered for service continuity planning, if the UE does not move to the predicted location, the EEC does not connect to T-EES, the AC does not connect to the T-EAS. Step 6 is skipped.

NOTE 3:	The S-EAS or T-EAS can further decide to terminate the ACR, and the T-EAS can discard the application context based on information received from EEL and/or other methods (e.g. monitoring the location of the UE). It is up to the implementation of the S-EAS and T-EAS whether and how to make such a decision.

NOTE 4:	When in step 1 the ACR has been triggered for service continuity planning, step 6 would only be performed after the UE moves to the expected location.

Phase IV:	Post-ACR clean up

7.	All required entities perform clean-up. When the S-EAS receives the ACR information notification method from the S-EES, the S-EAS informs the S-EES of the complete of ACT and the S-EES informs the EEC of the completion of ACR as described in clause 8.8.3.5.3.

* * * Next Change * * * *

#### 8.8.2.5	S-EES executed ACR

Figure 8.8.2.5-1 illustrates the procedure for the S-EES to detect, decide and execute the ACR from the S-EAS to the T-EAS. This procedure may support automated ACR by S-EES when initiated by S-EAS as per clause 8.8.3.6.

Editor's note:	Usage of network path information for the scenarios in clause 8.8.2.5 is FFS.

Pre-condition:

1.	The AC at the UE already has a connection to the S-EAS;

2.	The EEC is able to communicate with the S-EES; and

3.	The EAS has subscribed to receive the ACR information notification method from the EES, as described in clause 8.6.3.2.2.

Figure 8.8.2.5-1: S-EES executed ACR procedure

1.	The S-EAS may initiate Automated ACR with S-EES as specified in clause 8.8.3.6. In this step, the S-EAS and S-EES negotiate an address of the Application Context storage to S-EES. The S-EAS puts the Application Context at this address which can be further accessed by the S-EES when the ACT is required.

In this case, the S-EES executes steps 2 (i.e., S-EES detection), 4, 5, 6, 7, 8, 9 and 11. Rest of steps are skipped.

Phase I: ACR Detection

2.	Detection entities (S-EAS, S-EES, EEC) detects that ACR may be required as described in clause 8.8.1. The detection by the S-EES may be triggered by the User Plane path change notification received from the 3GPP Core Network due to S-EAS request for "ACR facilitation" event (see clause 8.6.3) or due to step 1.

The detection entity may detect that ACR may be required for an expected or predicted UE location in the future as described in clause 8.8.1.

Phase II: ACR Decision

3.	The detection entity performs ACR launching procedure (as described in clause 8.8.3.4) with the ACR action indicating ACR determination and the corresponding ACR determination data.

4.	The S-EES authorises the message if received. The S-EES decides to execute ACR based on the information received or local detection, and the information of EEC context or EAS profile, and then proceed the below steps.

Phase III:	ACR Execution

5.	The S-EES determines T-EES and T-EAS via the Discover T-EAS procedure in clause 8.8.3.2 of the present document. When in step 2 the ACR has been triggered for service continuity planning, then UE Location and Target DNAI values provided in the Retrieve T-EES procedure contain the expected UE Location and expected Target DNAI. The S-EES may decide not to perform ACR if T-EAS is not available.

6.	The S-EES sends the target information notification to the EEC as described in clause 8.8.3.5.3.

7.	The S-EES may apply the AF traffic influence with the N6 routing information of the T-EAS in the 3GPP Core Network (if applicable).

8.	The S-EES sends the ACR Notify message (e.g. as notification for "ACR facilitation" event or due to step 1) to the S-EAS to initiate ACT between the S-EAS and the T-EAS.

9.	The Application Context is transferred from S-EAS to the T-EAS at implementation specific time. In the case of automated ACR, the S-EES accesses the Application Context from the address as per step 1 and the S-EES and T-EES engage in the ACT from S-EAS to the T-EAS (obtained as per step 5) in a secure way. Further the T-EAS accesses the Application Context made available by the T-EES. If S-EAS performs the ACT directly with T-EAS, the specification of such process is out of scope of the present document.

NOTE 1:	The Application Context is encrypted and protected by the application layer. The S-EES and the T-EES engage in the packet level transport of the Application Context and they have no visibility to the content of the Application Context.

When in step 2 the ACR has been triggered for service continuity planning, if the UE does not move to the predicted location, the EEC does not connect to T-EES, the AC does not connect to the T-EAS. Steps 10 and 11 are skipped.

NOTE 2:	The S-EAS or T-EAS can further decide to terminate the ACR, and the T-EAS can discard the application context based on information received from EEL and/or other methods (e.g. monitoring the location of the UE). It is up to the implementation of the S-EAS and T-EAS whether and how to make such a decision.

NOTE 3:	When in step 2 the ACR has been triggered for service continuity planning, steps 10 and 11 would only be performed after the UE moves to the expected location.

Phase IV:	Post-ACR Clean up

10.	When the S-EAS receives the ACR information notification method from the EES, the S-EAS sends the ACR Complete message to the S-EES to confirm that the ACR has completed.

11.	The S-EES sends the ACR information notification message to the EEC to confirm that the ACR has completed as specified in clause 8.8.3.5.3.

NOTE 4:	The Application Client mechanism to support switchover of the application traffic to T-EAS is out of scope of the specification.

* * * End of Change * * * *