3GPP TSG-SA WG6 Meeting #45-bis-e	S6-212297

e-meeting, 11th – 19th October 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 23.434 | CR | 0083 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Group Deletion procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH |  |  |  |  |  | Date: |  |  | 2021-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | While Group deletion procedure related information flows (in clause 10.3.2.9, 10.3.2.10 and 10.3.2.11) are specified in TS 23.434, the procedure utilizing those information flows is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure for group deletion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-3 would not be able to implement group deletion procedure and 5GMARCH service would remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 10.3.X	Group deletion

Figure 10.3.X-1 below illustrates the group deletion operation by authorized VAL user/UE/administrator to delete a group. It applies to group deletion by a VAL administrator or by authorized user/UE.

Pre-conditions:

1.	The group management client, group management server, VAL server and the VAL group members belong to the same VAL system.

2.	The authorized VAL user/UE/administrator is aware of the group identity which needs to be deleted.

Figure 10.3.X-1: Group deletion

1.	The group management client of the authorized VAL user/UE/administrator requests group deletion operation to the group management server. The identity of the group to be deleted shall be included in this message.

2.	The group management server authorizes the request and if authorized, deletes the group.

3.	The group management server may notify the VAL server regarding the group deletion.

4.	The group members of the VAL group are notified about the deleted VAL group.

5.	The group management server provides a group deletion response to the group management client of the administrator/authorized VAL user/UE.