3GPP TSG-SA WG6 Meeting #45-BIS-e meeting	S6-212458

11th Oct – 19th Oct 2021, Online	(revision of S6-212294)

Source:	Nokia, Nokia Shanghai Bell

Title:	Addition of key issue pertaining to NAT deployments

Spec:	3GPP TR 23.700-98

Agenda item:	10.8

Document for:	Approval

Contact:	omar.elloumi@nokia.com

# 1	Introduction

In the EDGEAPP architecture a NAT is oftentimes deployed within the edge data network, e.g. typically load balancers implement NAT functions. In that cased the EES and the EAS have no means to know the IP address that is known by the core network. Therefore, access to network services is prevented.

# 2	Discussion

In operational deployments of cloud infrastructures including at the edge, Network Address Translation (NAT) deployed in conjunction with Load Balancing (LB) provide the ability to balance incoming traffic across multiple, similar server resources (instances of EAS or EES in the case of EDGEAPP). This is referred to as NAT LB which is designed to balance incoming traffic.

Such an operational constraint will prevent the EES and EAS to have an IP address of the UE that is known by the underlying 3GPP network.

This proposal suggests the addition of a key issue to study how such this operational constraint can be resolved through the EDGEAPP architecture.

# 3		Proposal

It is proposed to modify the text of TR 23.700-98 as follows.

1st CHANGE

## 4.x	Solution X: support of NAT deployed within the edge data network

In operational deployments of cloud infrastructures including at the edge, Network Address Translation (NAT) are oftentimes deployed.

Such an operational constraint will prevent the EES and EAS to have an IP address of the UE that is known by the underlying 3GPP network.

Open issue:

- How the EAS and/or the EES access 3GPP network services pertaining to a UE, such as IP address to UE ID translation, when the edge data network employs Network Address Translators (NATs).

END OF CHANGES